# 3GPP TSG-RAN WG4 Meeting # 102-e	R4-2205639

# Electronic Meeting, February 21 – 3 March, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR TA validation for Small Data Transmissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of TA validation requirements for SDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is based on the endorsed CR (R4-2202711) at RAN4#101-bis-e meeting with the intention to show our view on how to capture the measurement ranges requirements for TA validation.. The changes are highlighted. Change #1: TA validation for SDT -introduced including 2-window requirement for RSRP measurement. Text based on the LTE PUR with adaptation to NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No TA validation requirements when using CG-SDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

## 5.x	Configured Grant based Small Data Transmissions (CG-SDT)

### 5.x.1 Introduction

FFS

### 5.x.2 UE synchronization requirements

FFS

### 5.x.3 TA validation requirements

When cg-SDT-RSRP-ChangeThreshold [TS 38.331] is configured for TA validation based on the RSRP change criterion according to clause 5.8.2.x in [TS 38.321],the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in subclause 7.1 provided that

-	the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and,

-	timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2.x in [TS 38.321].

Table 5.x.3-1 Valid measurement for FR1

| Measurement | Validity condition |
| --- | --- |
| RSRP1 | (T1 – min( 200, M1* TDRX)) ≤ T1’ ≤ (T1 + min( 200 , M1* TDRX)) |
| RSRP2 | T2 – min( 200, M1* TDRX) ≤ T2’ ≤ T2 |

Table 5.x.3-2 Valid measurement for FR2

| Measurement | Validity condition |
| --- | --- |
| RSRP1 | (T1 – min( 400, M1*N1*TDRX)) ≤ T1’ ≤ (T1 + min( 400 , M1*N1*TDRX)) |
| RSRP2 | T2 – min( 400, M1*N1* TDRX) ≤ T2’ ≤ T2 |

RSRP1 is considered valid provided that the following condition is met:

(T1 – min([X1], [X2])) ≤  T1’ ≤  (T1 + min([X1] , [X2]))

RSRP2 is considered valid provided that the following condition is met:

T2 – min([Y1], [Y2]) ≤ T2’ ≤ T2

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT.

Where:

-	T1 is the time when the latest NTAwas obtained by the UE via Timing Advance Command MAC control element.

-	T1’ is the time when the UE has completed RSRP1.

-	T2 is the time when the UE performs TA validation as defined in clause 5.8.2.x in [TS 38.321] for transmission using CG-SDT.

-	T2 is the next CG-SDT occasion that follows in time based on the configured CG-SDT periodicity after TA validation.

-	T2’ is the time when the UE has completed RSRP2.

-     TDRX is the DRX cycle length in ms.

- M1 and N1 are the scaling factors as defined in clause 4.2.2.2.

### <End of Change 1>