3GPP TSG-RAN WG4 Meeting # 102-e 													R4-2207162

Electronic Meeting, Feb 21 - Mar 03, 2022

Agenda item:			10.12.1, 10.12.2.3

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [102-e][322] NR_perf_enh2_Demod_Part1

Document for:	Information

Introduction

This email thread discusses the general part and CRS interference handling in Rel-17 further demodulation performance enhancement WI in agenda 10.12.1 and 10.12.2.3.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to provide comments in section 1.3, 2.3 and 3.3.

  - For round 1, no further discussion on the issues concluded in Tuesday GTW session, and only provide comments on the following open issues:

    - For Issue 1-1-3 (Per cell or per UE NWA signaling), the text in [] from the outcome of GTW discussion

    - Sub-topic 2-1: Test setup for 15 kHz SCS scenario

    - Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario

    - Sub-topic 3-3: Test setup for 30 kHz SCS scenario

- 2nd round: TBA

Topic #1: Signalling aspects for CRS-IM

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203769 | Apple Inc. | Proposal #1: Define UE capability for CRS-IM as a per Feature Set, per CC capability. |
| R4-2203770 | Apple Inc. | Proposal #1: Introduce v-shift information as part of NWA signalling. Proposal #3: Introduce NWA signalling for the following parameters at least for RAN4 requirements in scenario 2: (1) LTE presence, (2) LTE carrier frequency, (3) LTE channel bandwidth. |
| R4-2204375 | Intel Corporation | Proposal 3: Consider the following design of CRS-IM capability signalling: per-UE granularity, FR1 only, no FDD/TDD deference. Proposal 4: Do not define the separate capability signalling for scenario 2 for UEs capable of performance CRS-IM without the NWA signalling on LTE channel bandwidth. |
| R4-2204382 | Intel Corporation | Proposal 1: Use one of the NWA signalling design options listed in Table 2 or 3. Proposal 2: Maximum number of interference cells, for which NWA signalling is provided, is equal to 8. Proposal 3: Consider per Serving Cell granularity for NWA signalling. Proposal 4: Consider signalling of LTE channel bandwidth as mandatory for Scenario 2 to enable CRS-IM processing. |
| R4-2204527 | CMCC | Proposal 1: The maximum number of cell ID information is 3. Proposal 5: The NWA signalling is per UE level. Proposal 6: Introduce the UE CRS-IM capability with the granularity of per UE. Proposal 7: Introduce CRS-IM UE capability signaling as follows: |
| R4-2204833 | Nokia, Nokia Shanghai Bell | CID and V-Shift information Observation 1: Our expectation for the CID list is to provide a semi-static list of CIDs which a UE might encounter as interferes. Observation 2: Providing dynamically updated list of CID is far more complex than providing a list of v-shifts, as the v-shifts are already available at the user plane. Observation 3: As the network is unaware of the most dominant interfering cell(s) for specific UEs, the UEs would need to determine on their own, which of the possible CIDs from the provided down selected interference CID candidate list are the dominant interferers. Proposal 1: The CID list provided in the NWA signalling shall be a serving cell-based candidate list of possible Providing the list of v-shifts in addition to CID could reduce the complexity on UE side. Observation 4: Providing the list of v-shifts in addition to CID could reduce the complexity on UE side. Proposal 2: Introduce an optional field in the NWA signalling containing a serving cell-based list of used v-shifts in parallel with the optional field containing a candidate list of CIDs. Signalling design when the default NW configuration assumptions are NOT valid Observation 9: If an optional field is not provided in NWA signalling, the default value of said the field shall be assumed. Observation 10: With above observation there is no way for the network to inform the UE if a parameter is invalid, without providing an alternative value of said parameter. Proposal 5: Consider introducing a field in the NWA signalling, which can be used to invalidate all default assumptions. How to inform UE to enable CRS-IM Observation 11: InterRAT MO cannot be used to also indicate enabling CRS-IM as it is not used solely for CRS-IM. Observation 12: NWA Signalling can be used to indicate enabling of CRS-IM as it is dedicated to CRS-IM and only will be transmitted in case CRS-IM is to be used. Observation 13: An empty NWA Signalling MO can be used to indicate enabling of CRS-IM when NW are using InterRAT MO instead of NWA signalling. Proposal 6: UE reception of NWA signalling MO will enable CRS-IM. indicate to UE that CRS-IM is to be enabled. NWA signalling framework proposal Observation 14: The best compromise between NW and UE implementation complexity will be to provide NWA signalling “per serving cell”. Observation 15: UE will be required to determine the dominant interferer as the NW will not have this information, hence UE will anyway have to monitor on the interference cells and will need to determine the correct CID for the dominant interferer. Having a reduced list of CIDs candidates will highly reduce the complexity on the UE side. Proposal 7: NWA signalling is “per serving cell”. Observation 16: The CRS-RM pattern IE contains most of the information potentially required to assist in CRS-IM LLR weighing reception. Observation 17: RRC configuration of RateMatchPatternLTE-CRS IE enables CRS-RM and, thus, prevents using CRS-IM approaches to convey the information. Observation 18: The existing RateMatchPatternLTE-CRS IE is missing some of the parameters agreed in the previous RAN4 meetings as required for LLR Weighting. Observation 19: Information provided in any level of NWA is non-dynamic information taken from the network configuration. The information provided is not expected to change during RRC_connected states. Proposal 8: RAN4 to ask RAN2 define new optional RRC signaling for CRS-IM IE, which contains the NW available information from the IE RateMatchPatternLTE-CRS and adds optional information for CRS-muting, (Max) Number of CRS ports, list of v-shift candidates and list of CID Candidates. The information provided in this new IE should be provided in a non-dynamic manner. Configuration of this message does not trigger CRS-RM functionality. Observation 20: It must be up to the NW to decide if CRS-IM is used or not. Proposal 9: Introduce an optional information parameter “defaultParametersValid-r17” to indicate if the default assumed parameters can/cannot be trusted. “defaultParametersValid-r17” should default to true (i.e. the default assumed parameters can be trusted) Proposal 10: Introduce an optional information parameter “CRS-Muting-r17” to indicate if CRS muting is used in the network. “CRS-Muting-r17” should default to the agreed default assumed setting. Proposal 11: Introduce an optional information parameter “v-ShiftList-r17” to indicate which v-shift is used in the LTE network. “v-ShiftList-r17” should default to the agreed default assumed setting. Proposal 12: Introduce an optional information parameter “maxNrofCRS-Ports-r17” to indicate the maximum number of CRS ports used by the network. RAN4 to discuss, if a default assumption for this parameter can be chosen. Proposal 13: Introduce an optional information parameter “cidCandidateList” to indicate which CIDs a UE might encounter as LTE interference cell. This information parameter will not have a default. UE Capability Signalling Observation 21: If a UE supports CRS-IM, it should be for all supported bands. It is too complex to introduce granularity as proposed in option 1. Proposal 14: Capability of CRS-IM should be per UE and applicable for the bands that are overlapping with LTE spectrum (Option 3) Observation 22: Having different capabilities would fragment the UE implementation space even further. Proposal 15: Only provide one capability (i.e. CRS-IM capable) in the UE capability signalling. |
| R4-2204918 | ZTE Corporation | Proposal 1: The maximum number of cell ID information is around 4 per frequency layer if neccessary. Proposal 3: The maximum number of interfering cell should be around 4 per frequency layer if necessary. Proposal 4: To introduce the per band basis for CRS-IM UE capability which is similar as option 3. |
| R4-2205433 | Ericsson | Observation 1: From RRM inter-RAT measurement capability point of view, it is expected UE can measure up to 7 E-UTRA FDD carrier frequency layers. Observation 2: In LTE, eNB can configure CRS assistance information up to 8 cells. Proposal 1: When NW doesn’t configure inter-RAT meas., gNB can optionally configure up to 8 LTE cell IDs and let UE to decide which ID(s) to use. Proposal 2: Follow the baseline assumption to not introduce vshift information into the NWA signaling. Proposal 3: RAN4 to discuss when UE can start to perform CRS-IM receiver according to different UE capabilities of LTE channel bandwidth detection and NWA signaling on LTE channel bandwidth. If necessary, RAN4 will capture the expected delay in TS38.101-4. Proposal 3a: If RAN4 define the CRS-IM requirements with Scenario 2 for UE capable of LTE channel bandwidth acquisition without NWA signaling on LTE channel bandwidth, TE does not start PDSCH scheduling until UE acquires LTE channel bandwidth, e.g. N x inter-RAT measurement period where N is the number of inter-RAT measurement configuration. Proposal 3b: If RAN4 define the CRS-IM requirements with Scenario 2 with NWA signaling on LTE channel bandwidth, TE starts PDSCH scheduling immediately after the connection setup. Proposal 4: RAN4 to send the LS to RAN2 to specify the procedure about NW scheduling and timer for CRS-IM feature. Proposal 7: Proposed signaling structure for CRS-IM receiver: |
| R4-2205494 | China Telecom | draft LS on UE capability and network assistant signalling for CRS interference mitigation in scenarios with overlapping spectrum for LTE and NR |
| R4-2205495 | China Telecom | Proposal 1: Confirm the baseline assumption that no need to introduce NWA signalling for v-shift information. Proposal 2: Confirm the baseline assumption in the last meeting WF on the LTE channel bandwidth for scenario 2. Proposal 3: Use the following NWA signalling rules when the default NW configuration assumptions are NOT valid: Proposal 4: The NWA signalling is signalled per serving cell, and information for maximum of 8 LTE neighbour cells for each serving cell can be indicated. Proposal 5: Defining CRS-IM receiving as a per UE capability. |
| R4-2205799 | Huawei, HiSilicon | Proposal 3: Define the UE CRS-IM capability with granularity of per UE but only applicable for the bands that are overlapping with LTE spectrum. |
| R4-2206071 | MediaTek Inc. | Proposal 3: Introduce the UE CRS-IM capability with the granularity of of per CC, per band, per band combination. Proposal 4: Introduce separate features for scenario 1 and scenario 2. |

Open issues summary

Sub-topic 1-1: Network Assistance Signalling Aspects

Issue 1-1-1: v-shift information

- Agreements in RAN#101-bis-e in the WF R4-2203131:

  - The baseline assumption: No need to introduce NWA signaling for v-shift information.

- Proposals:

  - Option 1: Confirm not introduce vshift information into the NWA signaling (E///, Intel, CTC)

  - Option 2: Introduce v-shift information as part of the optional NWA signalling (Apple, Nokia)

- Recommended WF

  - Since anyway the v-shift information (if included) is optional, can we go with option 2? Any technical issue if option 2 is agreed?

Issue 1-1-2: LTE channel bandwidth information for scenario 2

- Agreements in RAN#101-bis-e in the WF R4-2203131:

  - Option 1 (baseline assumption):

    - For scenario 2, LTE channel bandwidth information can be awared by following possible ways:

      - With inter-RAT MO configured, 1) UE can use PBCH decoding to obtain channel bandwidth information for CRS-IM if PBCH is within the configured measurement gap, or 2) UE can use power difference detection to obtain channel bandwidth information

      - LTE channel bandwidth information can be informed to UE by NWA signalling (optional)

    - For scenario 2, LTE channel bandwidth information can be awared by following possible ways:

      - With inter-RAT MO configured, 1) UE can use PBCH decoding to obtain channel bandwidth information for CRS-IM if PBCH is within the configured measurement gap, or 2) UE can use power difference detection to obtain channel bandwidth information

- For UE capable of obtaining LTE CBW information by PBCH decoding and/or power detection, inter-RAT MO information is needed to perform CRS-IM otherwise UE not expected to perform CRS-IM.

      - LTE channel bandwidth information can be informed to UE by NWA signalling (optional)

- For UE not capable of obtaining LTE CBW information by PBCH decoding and/or power detection, NWA signalling on LTE CBW is needed to perform CRS-IM otherwise UE not expected to perform CRS-IM.

      - Separate capability will be introduced for UE capable of performing CRS-IM in scenario 2 without the above new NWA signalling on LTE channel bandwidth.

      - Enabling CRS-IM receiver should not impact on existing RRM procedure and RRM requirements

  - Other options are not precluded.

- Proposals:

  - Option 1: Confirm option 1 above as the methodology to obtain the LTE channel bandwidth information for UE with different UE capabilities (CMCC, CTC)

  - Option 2: Consider signaling of LTE channel bandwidth as mandatory for Scenario 2 to enable CRS-IM processing, and do not define the separate capability signalling for scenario 2 for UEs capable of performance CRS-IM without the NWA signalling on LTE channel bandwidth. (Intel)

- Recommended WF

  - Given the difficult situation and long discussion in the last meeting, can we move forward and confirm the baseline assumption?

Issue 1-1-3: Per cell or per UE NWA signalling

- Agreements in RAN#101-bis-e in the WF R4-2203131:

  - Signalling design when the default NW configuration assumptions are NOT valid

    - The signalling (when the default NW configuration assumptions are NOT valid) is [per serving cell], and further discuss the maximum number of interference cells can be signalled.

- Proposals:

  - Option 1: NWA is signalled per serving cell (Nokia, Intel, CTC, ZTE, [E///])

    - Option 1A: NWA contain common configurations for all interference cells (Nokia)

    - Option 1B: NWA contain the configurations of each interference cell (Intel, CTC, ZTE, E///)

    - Option 1C: For each of the parameter, the NWA can be the same or different for different interference cells, e.g., up to 8 cell ID information and common information for other parameters (see details in Table 3 of R4-2204382)  (Intel)

  - Option 2: Per UE level (CMCC)

    - CMCC: Whether network should signal the LTE channel bandwidth is depended on UE capability, and whether network should signal the LTE cell carrier frequency is depended on inter-RAT MO configuration.

- Recommended WF

  - TBA

Issue 1-1-4: Maximum interference cells number for each serving cell for NWA signalling

- Agreements in RAN#101-bis-e in the WF R4-2203131:

  - Signalling design when the default NW configuration assumptions are NOT valid

    - The signalling (when the default NW configuration assumptions are NOT valid) is [per serving cell], and further discuss the maximum number of interference cells can be signalled.

- Proposals:

  - Option 1: Reuse assumptions from LTE and consider 8 (Intel, CTC, E///)

  - Option 2: Around 4 (ZTE)

  - Option 3: maximum 3 cells for cell ID information (CMCC)

- Recommended WF

  - Agree option 1?

Issue 1-1-5: A field in the NWA signalling to invalidate all default assumptions

- Proposals:

  - Proposal 1: Consider introducing a field in the NWA signalling, which can be used to invalidate all default assumptions. (Nokia, E///)

- Recommended WF

  - Encourage feedback on Proposal 1.

Issue 1-1-6: Trigger for enabling CRS-IM by NWA signalling

- Proposals:

  - Option 1: UE applies CRS-IM for a serving cell when explicitly configured by network. Up to RAN2 how to indicate this in signalling. (Nokia)

- Recommended WF

  - In moderator’s understanding, the above option 1 conflicts with the previous agreements below:

    - For scenario 1, all NWA signaling are optional and CRS-IM can be triggered by serving cell CRS-RM configuration

    - For scenario 2, if the baseline assumption in Issue 1-1-2 (LTE channel bandwidth information for scenario 2) is confirmed:

      - For UE capable of obtaining LTE CBW information by PBCH decoding and/or power detection, inter-RAT MO information is needed to perform CRS-IM otherwise UE not expected to perform CRS-IM.

      - For UE not capable of obtaining LTE CBW information by PBCH decoding and/or power detection, NWA signalling on LTE CBW is needed to perform CRS-IM otherwise UE not expected to perform CRS-IM.

  - Is it acceptable to all companies to revisit the previous agreement and consider the option 1 from Nokia?

Sub-topic 1-2: CRS-IM Capability Signalling Aspects

Issue 1-2-1: Granularity of UE CRS-IM Capability

- Agreements in RAN#101-bis-e in the WF R4-2203131:

  - Option 1: Introduce granularity of per CC, per band, per band combination (per Feature Set per CC)

  - Option 2: Introduce granularity of per UE

  - Option 3: Introduce granularity of per UE, but only applicable for the bands that are overlapping with LTE spectrum

  - Option 4: Introduce granularity of per band, per band combination (per Feature Set)

- Proposals

  - Option 1: Introduce granularity of per CC, per band, per band combination (per Feature Set per CC) (Apple, MTK)

  - Option 2: Introduce granularity of per UE (CMCC, CTC, Intel)

    - Intel: In LTE, CRS-IM capability signalling is defined with per-UE granularity. Also, in additional to information about support of CRS-IM processing, UE informs about the maximum number of CCs for which UE can apply CRS-IM for scenarios with TM1 - TM9. Same time, for scenarios with TM10, UE just informs about support of CRS-IM processing.

  - Option 3: Introduce granularity of per UE, but only applicable for the bands that are overlapping with LTE spectrum (Nokia, HW)

  - Option 4: Introduce granularity of per band (ZTE)

- Recommended WF

  - For proponent of option 3, clarification on “the bands that are overlapping with LTE spectrum” is encouraged.

  - Considering CRS-IM is a baseband capability, can we consider the similar UE capability signaling as LTE CRS-IM (as mentioned by Intel)?

    - CRS-IM capability signalling is defined with per-UE granularity. Also, in additional to information about support of CRS-IM processing, UE informs about the maximum number of CCs for which UE can apply CRS-IM.

Issue 1-2-2: Applicability of UE CRS-IM Capability

- Agreements in RAN#101-bis-e in the WF R4-2203131:

  - For 15kHz SCS:

    - Option 1: FR1 only, no FDD/TDD difference

    - Option 2: UE capabilities are applicable whenever they are signaled

  - For scenario 1 and 2:

    - Introduce separate features for scenario 1 and scenario 2

- Proposals

  - Proposal 1: Only provide one per-UE capability (i.e. CRS-IM capable) in the UE capability signalling (Nokia)

  - Proposal 2: All UE CRS-IM capabilities for 15kHz SCS to be FR1 only, without FDD/TDD difference (CMCC, Intel)

- Recommended WF

  - For Proposal 1, could proponent clarify is the one per-UE capability applicable for both scenario 1 and scenario 2?

  - For Proposal 2, is it agreeable?

Companies views’ collection for 1st round

Open issues

|  | Sub-topic 1-1: Network Assistance Signalling Aspects Issue 1-1-1: v-shift information Issue 1-1-2: LTE channel bandwidth information for scenario 2 Issue 1-1-3: Per cell or per UE NWA signalling Issue 1-1-4: Maximum interference cells number for each serving cell for NWA signalling Issue 1-1-5: A field in the NWA signalling to invalidate all default assumptions Issue 1-1-6: Trigger for enabling CRS-IM by NWA signalling Sub-topic 1-2: CRS-IM Capability Signalling Aspects Issue 1-2-1: Granularity of UE CRS-IM Capability Issue 1-2-2: Applicability of UE CRS-IM Capability |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Sub-topic 1-1: Network Assistance Signalling Aspects Issue 1-1-1: v-shift information We agree with proposed WF: Introduce v-shift information as part of the optional NWA signalling (Option 2) As agreed in earlier meetings CID is not needed for LLR Weighting, hence it should be allowed for network vendors to provide a list of v-shifts used as this would simplify the implementation on network side. As previously argued, we also see a combination of v-shift and a candidate CID list providing an improvement compared to only having a candidate CID list. Issue 1-1-2: LTE channel bandwidth information for scenario 2 We agree with Intel, that the LTE channel bandwidth is needed for CRS-IM, however it should still be possible for the UE to determine this without NWA, hence the bandwidth field can be kept optional. In addition, we would like to keep the possibility of sending an empty NWA IE, hence all field should be optional. The separate UE capabilities will let the UE overrule the technical agreement above, however we are open to compromise in this matter to move the WI forward. We can accept to go for the baseline assumption. Issue 1-1-3: Per cell or per UE NWA signalling The NW cannot be required to have different CRS-IM NWA messages for each UE. First this would require information from the UE about which are the dominant interfering cells. Also, the only feasible NWA might often be the same for many/all UEs. To maintain the smallest most flexible NWA solution but still maintain the option to provide all fields per CID, we agree that NWA fields should be possible to be provided either per CID or for all CIDs. We can accept option 1C but RAN4 will have to discuss the details given in table 3 of R4-2204382. Issue 1-1-4: Maximum interference cells number for each serving cell for NWA signalling Reuse assumption from LTE and consider 8 (option 1) Issue 1-1-5: A field in the NWA signalling to invalidate all default assumptions Since a NW might not comply to the default assumption for all fields and still not have an actual value for said field, it should be possible to invalidate all default assumptions. Issue 1-1-6: Trigger for enabling CRS-IM by NWA signalling We see the following issue: In scenario 1, a UE configured with CRS-RM, would immediately turn on CRS-IM by default; using the default assumptions. This might lead to performance loss, if the default assumptions are grossly incorrect and the NW is not be able to obtain the correct configurations. There is currently no fall-back, where the NW can disable default-on CRS-IM in case unresolvable performance loss is observed. We would like to ask the opinion of other companies to go back on the earlier decision for scenario 1 (which was quoted by the moderator) and agree that CRS-IM (in both scenario 1 and 2) is not enabled until the NW triggers activation via RRC. Sub-topic 1-2: CRS-IM Capability Signalling Aspects Issue 1-2-1: Granularity of UE CRS-IM Capability We agree with both option 2 and option 3. The intent behind mentioning “LTE overlapping bands” was to answer to concerns raised in the last meeting, where some companies noted that non LTE applicable bands should not be included in the capability declaration. In our opinion, this is a common understanding, but we would be happy to accommodate concerns via a note on LTE overlapping bands. Though, we don’t need this clarification strictly speaking. Issue 1-2-2: Applicability of UE CRS-IM Capability Proposal 2 is also fine for us. |
| Apple | Sub-topic 1-1: Network Assistance Signalling Aspects Issue 1-1-1: v-shift information We support the recommended WF. Issue 1-1-2: LTE channel bandwidth information for scenario 2 We support option 2. We agree with Intel’s observation that depending on how the NWA is signalled, there might not be any benefit of having 2 UE capabilities and it would be more straight forward to have LTE BW information signalled by NWA always. Issue 1-1-3: Per cell or per UE NWA signalling We support option 1B/1C. Signalling per cell with information for LTE interference cells would be less overhead than signalling per UE in our understanding. With option 1A, we don’t see how all interference cells can have the same NWA information signalled. Issue 1-1-4: Maximum interference cells number for each serving cell for NWA signalling We support option 1. Issue 1-1-5: A field in the NWA signalling to invalidate all default assumptions If there is NWA for any parameters, UE would use that over the default assumptions. But if the default assumptions for parameters are not valid, UE would expect NWA in our understanding. What is the purpose to invalidate default parameter assumptions without providing the NWA for the parameters? Issue 1-1-6: Trigger for enabling CRS-IM by NWA signalling This kind of signal would be very helpful to save power for the UE. The UE would perform CRS-IM only on such network indication. In scenario 2 if UE is not in NR deployed neighbouring LTE area, why should UE be expected to do any CRS-IM? Having some kind of indication is necessary. If in scenario 2 LTE channel BW is mandatory in NWA, this is not needed. Also in Scenario 1, with current assumption, if CRS RM for serving cell is configured, UE is expected to enable CRS-IM. Sub-topic 1-2: CRS-IM Capability Signalling Aspects Issue 1-2-1: Granularity of UE CRS-IM Capability We support option 1. Per feature set, per CC is also used for signalling baseband capabilities like max number of MIMO layers, modulation order. We cannot assume that UE can be capable of performing CRS-IM in all bands, all CCs . Keeping UE complexity in mind, this should be a capability indicated per feature set, per CC to give UE sufficient flexibility. Issue 1-2-2: Applicability of UE CRS-IM Capability Proposal 2 is agreeable for 15KHz SCS. |
| CMCC | Sub-topic 1-1: Network Assistance Signalling Aspects Issue 1-1-1: v-shift information We prefer Option 1. In our point of view, we have already included cell-ID information as optional NWA signalling. V-shift information can be derived from cell-ID, then it is duplicated work that also include v-shift as optional in NWA signalling. Issue 1-1-2: LTE channel bandwidth information for scenario 2 We support the recommended WF. Issue 1-1-3: Per cell or per UE NWA signalling Option 2 is our preference. We think whether NWA signalling is needed or not depends on UE capability. Besides, For UEs in different locations in a cell, they may observe different interference cells. Then, the information in NWA signaling should also be different. For Option 1, we don’t see the advantage if all potential interference information is included, regardless which interference cell(s) is/are the most dominate interference cell(s) for a UE. Issue 1-1-4: Maximum interference cells number for each serving cell for NWA signalling Option 1 is fine for us. Issue 1-1-5: A field in the NWA signalling to invalidate all default assumptions We don’t think this field is needed. If a parameter is included in NWA, then corresponding default assumption will be invalidated (if exist) Issue 1-1-6: Trigger for enabling CRS-IM by NWA signalling We support to insist previous agreement. Sub-topic 1-2: CRS-IM Capability Signalling Aspects Issue 1-2-1: Granularity of UE CRS-IM Capability We support Option 2. As stated in recommended WF, CRS-IM is a baseband capability. Issue 1-2-2: Applicability of UE CRS-IM Capability We support Proposal 2. |
| Huawei | Sub-topic 1-1: Network Assistance Signalling Aspects Issue 1-1-1: v-shift information We support not to include v-shift information. The v-shift information can be derived from Cell-ID. Since Cell-ID has already been introduced as optional NWA, it is redundant to introduce v-shift as well. Moreover, UE may be confused on using which information to decide the location of CRS, if there is a mismatch between the total number of Cell-IDs and the total number of v-shifts simultaneously informed by NW. We wonder the condition under which the network only provides the v-shift information instead of PCell ID, considering the v-shift information only will limit the UE implementation, either power difference method, otherwise UE has to detect the PCell ID. Issue 1-1-2: LTE channel bandwidth information for scenario 2 We support Moderator’s suggestion and confirm the baseline assumption. Issue 1-1-3: Per cell or per UE NWA signalling Similar as per LTE, referring to TS36.331, we support per UE NWA signalling. It is more reasonable to inform dedicated Cell-ID to UE, since the possible interference Cell-IDs are different for UEs with different locations in a cell. If the Cell-ID information is transmitted per cell, the complexity of UE will be increased, because the UE has to check all cells included in the Cell-ID which may be not interference cells for it but for other UEs. Moreover, the NWA signalling depends on UE CRS-IM capability. If the UE supports to obtain necessary parameters for CRS-IM without using NWA signalling, then NW can save budget. Issue 1-1-4: Maximum interference cells number for each serving cell for NWA signalling We suggest to reuse 8 as the maximum interference cells number from LTE, see TS36.331 6.4. Issue 1-1-5: A field in the NWA signalling to invalidate all default assumptions It is not necessary to introduce one specific field to invalidate all default assumptions. The current higher RRC signalling mechanism supports to overwrite the default/old configurations with new ones. Issue 1-1-6: Trigger for enabling CRS-IM by NWA signalling We think the agreements reached in last meeting is very clear: UE can use inter-RAT MO configuration or NWA to trigger the CRS-IM, no any additional NWA signalling to trigger the CRS-IM. Sub-topic 1-2: CRS-IM Capability Signalling Aspects As per LTE CRS-IM, the granularity is per UE but with limitation of the maximum number of CC that UE can support CRS-IM simultaneously. Considering the applicable scenario of CRS-IM, UE just needs to perform CRS-IM on certain bands or certain CCs, with more UE flexibility, Option 1 (per FSCC) is the most flexible capability reporting granularity, so we are also fine with Option 1. Issue 1-2-2: Applicability of UE CRS-IM Capability We are fine with Proposal 2. |
| ZTE | Sub-topic 1-1: Network Assistance Signalling Aspects Issue 1-1-1: v-shift information Option 1 is OK. In our understanding, We think that v-shift information can be introduced by inter-RAT MO.If the network configured inter-RAT MO, there is no necessary to introduce v-shift information by NWA signalling. What’s more, in scenario 2 with 30kHz, all the NR REs in interfered symbols will be interfered by LTE CRS. For this reason, there is no necessary to introduce v-shift information. Based on the above analysis,we think option 1 is ok. Issue 1-1-2: LTE channel bandwidth information for scenario 2 Option 1 is ok. If the network configured inter-RAT MO, UE can use PBCH decoding to obtain LTE channel bandwidth. If the netwok configured is not valid, UE need to obtain LTE channel bandwidth by NWA signalling.For this reason, we think introduce LTE channel bandwidth information for UE with different UE capabilities is ok. Issue 1-1-3: Per cell or per UE NWA signalling Option 1B is OK. Based on our understanding, we should define NWA signalling as per serving cell. Maybe every per serving contains different interference cell. Therefore, NWA signalling should contain the configurations of each interference cell as a signalling for serving cell. Each interference has different v-shift information、CRS port and CBW, etc. Consequently, we should consider NWA signalling for each interference cell. Issue 1-1-4: Maximum interference cells number for each serving cell for NWA signalling We support Option 2. Based on the side conditions -6dB for cell identification in RRM spec, which is equal to 10*log10(4). Therefore maximum number of cell ID information should be around 4. Issue 1-1-5: A field in the NWA signalling to invalidate all default assumptions This field shouldn’t be consider. Issue 1-1-6: Trigger for enabling CRS-IM by NWA signalling We think the previous agreement is ok. Sub-topic 1-2: CRS-IM Capability Signalling Aspects Issue 1-2-1: Granularity of UE CRS-IM Capability Option 3 or option 4 is ok. There is no difference between them. First, CRS-IM applies to dynamic spectrum sharing (DSS) scenario which overlapping with LTE spectrum. Per UE maybe a good choice for receiver. In addition, Option 3 could be understandable from RAN4 perspective from the LTE-NR coexistence scenario.To follow the same logic as Rel-16 eMIMO multiple LTE CRS rate matching cases due to multiple TRP, this should be per band basis. Based on our analysis, option 3 is best. Issue 1-2-2: Applicability of UE CRS-IM Capability Based on our analysis, we should consider separate capability for scenario 1 and scenario 2. For scenario 1, the information can be obtained by CRS-RM. For scenario 2, the information can be obtained by power detection or NWA signalling (if inter-RAT MO is not valid). |
| Intel | Issue 1-1-3: Per cell or per UE NWA signalling Based on our understanding, the wording from GTW is clear enough. Probably, we can consider the following in case it will be clearer For parameters included in the NWA signalling (except Cell ID) the values can be same or different for different interference cells e.g. selected parameters except Cell-ID can be informed by NWA independent of interference cells |
| Qualcomm | Issue 1-1-3: Per cell or per UE NWA signalling Ok with GTW wording. |
| China Telecom | Issue 1-1-3: Per cell or per UE NWA signalling Ok with GTW wording. |
| MediaTek | Issue 1-1-3: Per cell or per UE NWA signalling Ok with GTW wording. |
| Ericson | We could consider the following wording based on Intel’s proposal: For parameters included in the NWA signalling (except Cell ID) the values can be same or different for different interference LTE cells e.g. selected parameters except Cell-ID can be informed by NWA independent of interference LTE cells |

Summary for 1st round

Sub-topic 1-1: Network Assistance Signalling Aspects

Issue 1-1-1: v-shift information

Agreement:

v-shift information can be included in NWA signaling as optional

- If cell-ID information informed, then v-shift information shall not be informed

Issue 1-1-2: LTE channel bandwidth information for scenario 2

- Agreements in RAN#101-bis-e in the WF R4-2203131:

  - Option 1 (baseline assumption):

    - For scenario 2, LTE channel bandwidth information can be awared by following possible ways:

      - With inter-RAT MO configured, 1) UE can use PBCH decoding to obtain channel bandwidth information for CRS-IM if PBCH is within the configured measurement gap, or 2) UE can use power difference detection to obtain channel bandwidth information

      - LTE channel bandwidth information can be informed to UE by NWA signalling (optional)

    - For scenario 2, LTE channel bandwidth information can be awared by following possible ways:

      - With inter-RAT MO configured, 1) UE can use PBCH decoding to obtain channel bandwidth information for CRS-IM if PBCH is within the configured measurement gap, or 2) UE can use power difference detection to obtain channel bandwidth information

- For UE capable of obtaining LTE CBW information by PBCH decoding and/or power detection, inter-RAT MO information is needed to perform CRS-IM otherwise UE not expected to perform CRS-IM.

      - LTE channel bandwidth information can be informed to UE by NWA signalling (optional)

- For UE not capable of obtaining LTE CBW information by PBCH decoding and/or power detection, NWA signalling on LTE CBW is needed to perform CRS-IM otherwise UE not expected to perform CRS-IM.

      - Separate capability will be introduced for UE capable of performing CRS-IM in scenario 2 without the above new NWA signalling on LTE channel bandwidth.

      - Enabling CRS-IM receiver should not impact on existing RRM procedure and RRM requirements

  - Other options are not precluded.

Agreement:

Confirm option 1 above as the methodology to obtain the LTE channel bandwidth information for UE with different UE capabilities

Issue 1-1-3: Per cell or per UE NWA signalling

Agreement:

The NWA is signaled under each serving cell with flexibility to support per UE level configuration with up to 8 interference cell information.

- [For each of the parameter except Cell-ID, the NWA can be the same or different for different interference cells e.g. selected parameters except Cell-ID can be informed by NWA independent of interference cells]

- Note: The detailed signalling design subject to RAN2 decision

Email discussion after Tuesday GTW:

- On the text in [] from the outcome of GTW discussion:

  - Ok with the wording from GTW (Intel, QC, CTC, MTK)

  - Possible wording update if needed

    - Intel:

For parameters included in the NWA signalling (except Cell ID) the values can be same or different for different interference cells e.g. selected parameters except Cell-ID can be informed by NWA independent of interference cells

    - E///: (change compared to Intel’s version is highlighted)

For parameters included in the NWA signalling (except Cell ID) the values can be same or different for different interference LTE cells e.g. selected parameters except Cell-ID can be informed by NWA independent of interference LTE cells

Recommendations for 2nd round:

- Check if the latest wording update from E/// is acceptable. If not, go back the GTW agreements.

Issue 1-1-5: A field in the NWA signalling to invalidate all default assumptions

- Proposals:

  - Proposal 1: Consider introducing a field in the NWA signalling, which can be used to invalidate all default assumptions.

Agreement:

No further discussion on issue 1-1-5 in this meeting.

Issue 1-1-6: Trigger for enabling CRS-IM by NWA signalling

- Proposals:

  - Option 1: UE applies CRS-IM for a serving cell when explicitly configured by network. Up to RAN2 how to indicate this in signalling. (Nokia, Apple)

Agreement

No further discussion on issue 1-1-6.

Note: It’s RAN4 common understanding it’s up to UE implementation to turn on/off CRS-IM with reasonable performance.

Sub-topic 1-2: CRS-IM Capability Signalling Aspects

Issue 1-2-1: Granularity of UE CRS-IM Capability

Agreement:

Option 1: Per Feature Set per CC

Issue 1-2-2: Applicability of UE CRS-IM Capability

- Proposals

  - Proposal 2: All UE CRS-IM capabilities for 15kHz SCS to be FR1 only, without FDD/TDD difference

Agreement:

Proposal 2 agreed

Discussion on 2nd round

Topic #2: Test setup for CRS-IM

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203770 | Apple Inc. | Observation #1: Demodulation requirements typically do not have inter-RAT MO configured. Observation #2: Max TP would be different between test cases with and without inter-RAT MO Observation #3: Impact of errors in detection of parameters with inter-RAT measurement may not be accounted for in simulation results. Observation #4: Test requirements for schemes with NWA signalling and with detection of parameters by inter-RAT measurements cannot be the same. Proposal #2: Define one test setup for requirement in scenario 2 with NWA signalling configured. |
| R4-2204527 | CMCC | Proposal 2: Define one set of test requirements, the requirements will be applied for both two UE capabilities. Proposal 3: Define two sets of test setup according to different UE capabilities. Proposal 4: Define the applicability rule that if UE is capable of both BW acquisition schemes, the test setup “Only Inter-RAT MO is configured” should be used for testing. |
| R4-2204833 | Nokia, Nokia Shanghai Bell | Observation 5: Obtaining LTE channel bandwidth using InterRAT MO or NWA signalling will provide the same results. Proposal 3: Test requirements for scheme#1 and scheme#2 can be the same. Observation 6: UE is not required to support both NWA and InterRAT MO when supporting CRS-IM. UE capability signalling will inform the NW what is supported (either InterRAT MO or NWA; or both). Observation 7: It can be assumed, that CRS-IM parameters will be the same for the UE independent on using InterRAT MO or NWA signalling. Observation 8: As the parameters will be the same (assuming no mis-detection) for InterRAT MO and NWA signalling and there is no requirement for a UE to support both InterRAT MO and NWA at the same time. Proposal 4: RAN4 to not have separate requirements defined for InterRAT MO and NWA signalling (Option 2). |
| R4-2204918 | ZTE Corporation | Observation 1: There is no difference of test requirement between Scheme #1 and Scheme #2. Proposal 2: It’s no necessary to consider NWA signaling if test has been passed for scenario 2 which is configured by inter-RAT MO. |
| R4-2205433 | Ericsson | Proposal 5: Define the same test requirement for scheme 1 and 2. Proposal 6: Define 2 sets of test setup with applicability rule: Test 1: Only Inter-RAT MO is configured, and Test 2: Only the new NWA signaling is configured. • Test 1 is applicable for UE capable of LTE channel bandwidth acquisition with inter-RAT measurement. • Test 2 is applicable for UE not capable of LTE channel bandwidth acquisition with inter-RAT measurement. |
| R4-2205799 | Huawei, HiSilicon | Observation 1: PBCH has higher robust than PDSCH and there is no interference when UE is decoding PBCH. Therefore, PBCH performance can be always guaranteed at SNR range targeted for PDSCH. Proposal 1: The requirements for scheme#1 (CRS-IM with Inter-RAT MO configured and perform PBCH decoding and/or power difference detection) and scheme#2 (CRS-IM with NWA signalling) can be same Proposal 2: Define one set of test setup with both Inter-RAT MO and the new NMA signalling configured by the network:  Inter-RAT MO is used for acquiring LTE frequency information and PBCH decoding.  NWA only include LTE bandwidth information. |
| R4-2203771 | Apple Inc. | Proposal #1: Only consider 20% PDSCH loading with INR1 = 10.45 dB and INR2 = 4.6 dB interference levels. Proposal #2: Only cover 4 CRS ports and 4 TX antenna for all requirements for CRS-IM. |
| R4-2204383 | Intel Corporation | Proposal 1: Use the following INR values and Interference PDSCH loading assumptions for CRS-IM requirements: • Scenario 1: Loading 20 %, INR1 = 10.45 dB and INR2 = 4.6 dB or Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB • Scenario 2: Loading 20 %, INR1 = 10.45 dB and INR2 = 4.6 dB Proposal 2: Define the CRS-IM requirements with 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2. Proposal 3: Do not define the CRS-IM requirements for Scenario 2 with configured Inter-RAT MO and consider only tests with NWA signalling configuration. Observations #1: Testable CRS-IM performance benefits (1.2 – 1.7 dB) for Scenario 1 can be achieved for scenarios with 2 and 4 CRS ports and 20% and 30% interference cell loading. Observations #2: Testable CRS-IM performance benefits (1.1 – 1.6 dB) for Scenario 2 can be achieved for scenarios with 20% and 30% interference cell loading |
| R4-2204528 | CMCC | Proposal 1: Take 20% PDSCH loading level as baseline. Further check 30% loading level for Scenario 1. Proposal 2: • For LTE CRS port in Scenario 1, over 2 and 4 for CRS ports • For LTE CRS port in Scenario 2, only cover 4 CRS ports. |
| R4-2204919 | ZTE Corporation | Proposal 1: Add more loadings such as 30%, 40% and 50% with lower performance gain. Proposal 2: Add more interference power levels with lower performance gain. Proposal 3: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2. |
| R4-2205434 | Ericsson | Proposal 1: Option 2: Only consider INR1 = 10.45dB and INR2 = 4.6dB. Proposal 2: Option 2: Not to consider other interference loading level Proposal 3: Only cover 4 LTE CRS ports for defining requirements |
| R4-2205496 | China Telecom | Proposal 1: The test requirement for the following schemes can be the same: − Scheme #1: CRS-IM with Inter-RAT MO configured and perform PBCH decoding and/or power difference detection − Scheme #2: CRS-IM with NWA signalling Proposal 2: Define 2 sets of test setup: 1) Only Inter-RAT MO is configured, and; 2) Only the new NWA signalling is configured. Proposal 3: Add one set of INRs and loading levels for the CRS-IM testing to ensure that UE will turn on CRS-IM in different INR conditions. Select one of the combinations that achieves about 1dB performance gain over the reference scheme: • Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB • Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB • Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB Proposal 4: Cover 2 and 4 CRS ports for the neighbour cell LTE. Cover different CRS port number for different test cases such as different scenario or different INR levels based on the simulation results. |
| R4-2205800 | Huawei, HiSilicon | Proposal 1: Only consider 20% loading. Proposal 2: Consider INR1=10.45dB and INR2=4.6dB Proposal 3: Only consider 4 CRS ports |
| R4-2206052 | Nokia, Nokia Shanghai Bell | PDSCH loading level Observation 1: Low loading levels represent a worst case for the CRS-IM feature as IM is applied to resources that do not experience interference (assuming the algorithm does not include detection of such cases). High loading levels can mask the downsides of simple CRS-IM implementations, but also highlight the performance increase potential of CRS-IM. Both scenarios should be included in testing. Proposal 1: 20% loading level is sufficient (option 2) to verify minimum performance of the implementation. Interference power level Observation 2: The already agreed set of INR values is sufficient for minimum performance requirement coverage as the minimum performance requirements are assured with a single profile, which was already agreed. Proposal 2: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB in case of 20% interference PDSCH loading Tx antenna and LTE CRS port number Observation 3: 2 CRS ports are the most common deployment (option 3). Observation 4: Including 4 CRS ports in addition to 2 CRS ports (option 2A) is not in our view required but can be an option, if requested by companies. Proposal 3: Both option 2A and option 3 are acceptable. |
| R4-2206107 | Qualcomm Incorporated | Proposal 1: Only consider 20% PDSCH loading level. Proposal 2: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB. Proposal 3: Only cover 4 CRS ports for defining CRS-IM requirements. |
| R4-2206071 | MediaTek Inc. | Observation 1: The test requirements for both schemes can be the same if it is guaranteed that PBCH can be decoded correctly in inter-RAT measurement or there is no error in power difference detection. Proposal 1: If it can be guaranteed that PBCH can be decoded correctly in inter-RAT measurement or there is no error in power difference detection. We can define the same requirement for different UE capabilities. Proposal 2: Define two sets of test setups as the UE capability for Scheme #1 and Scheme #2 is different. |

Open issues summary

Sub-topic 2-1: Test setup for 15 kHz SCS scenario

Issue 2-1-1: Test setup for scenario 2

- Agreements in RAN#101-bis-e in the WF R4-2203131

  - In the next meeting, discuss whether the test requirement for the following schemes can be the same:

    - Scheme #1: CRS-IM with Inter-RAT MO configured and perform PBCH decoding and/or power difference detection

    - Scheme #2: CRS-IM with NWA signaling

  - Further discuss the following test setup for scenario 2 in the next meeting:

    - Option 1: Define one set of test setup with both Inter-RAT MO and the new NMA signaling configured by the network

    - Option 2: Define 2 sets of test setup: 1) Only Inter-RAT MO is configured, and; 2) Only the new NWA signaling is configured.

      - FFS the applicability of the 2 sets of test setup

    - Option 3: Define one set of test setup: Only the new NWA signaling is configured.

- Proposals on whether the test requirement for different schemes can be the same:

  - Option 1: Test requirements for the 2 schemes cannot be the same (Apple)

    - Apple: 1) Max TP would be different between test cases with and without inter-RAT MO. 2) Impact of errors in detection of parameters with inter-RAT measurement may not be accounted for in simulation results

  - Option 2: Test requirements for the 2 schemes should be the same (Nokia, ZTE, E///, HW, CTC, MTK)

    - Nokia: With InterRAT MO, UE should be able to determine the LTE bandwidth without misdetection

    - HW: PBCH performance can be always guaranteed at SNR range targeted for PDSCH.

    - E///: RAN4 to discuss when UE can start to perform CRS-IM receiver according to different UE capabilities of LTE channel bandwidth detection and NWA signaling on LTE channel bandwidth, i.e., only measure the throughput after the LTE channel bandwidth information is obtained.

    - If necessary, RAN4 to send the LS to RAN2 to specify the procedure about NW scheduling and timer for CRS-IM feature.

    - MTK: If it can be guaranteed that PBCH can be decoded correctly in inter-RAT measurement or there is no error in power difference detection. We can define the same requirement for different UE capabilities.

- Proposals on the test setup for scenario 2

  - Option 1: Define one set of test setup with both Inter-RAT MO and the new NMA signaling configured only include the CBW information (HW)

  - Option 2: Define 2 sets of test setup for different UE capabilities: 1) Only Inter-RAT MO is configured, and; 2) Only the new NWA signaling is configured. (CMCC, Nokia, ZTE, Ericsson, CTC, MTK)

    - Applicability rule:

      - Option 2A: If UE is capable of both BW acquisition schemes, the test setup “Only Inter-RAT MO is configured” should be used for testing. (CMCC, ZTE)

    - PDSCH scheduling timing

    - Option 2B-1: If RAN4 define the CRS-IM requirements with Scenario 2 for UE capable of LTE channel bandwidth acquisition without NWA signaling on LTE channel bandwidth, TE does not start PDSCH scheduling until UE acquires LTE channel bandwidth, e.g. N x inter-RAT measurement period where N is the number of inter-RAT measurement configuration. (Ericsson)

    - Option 2B-2: If RAN4 define the CRS-IM requirements with Scenario 2 with NWA signaling on LTE channel bandwidth, TE starts PDSCH scheduling immediately after the connection setup. (Ericsson)

  - Option 3: Define one set of test setup: Only the new NWA signaling is configured. (Intel, Apple)

    - Option 3A (Apple): Including LTE presence, LTE carrier frequency, LTE channel bandwidth in the NWA signaling in the test

- Recommended WF

  - Whether the test requirement for different schemes can be the same

    - Can we agree option 2 (test requirements for the 2 schemes should be the same)?

  - Test setup for scenario 2

    - Can we agree option 2 (define 2 sets of test setup for different UE capabilities) together with the Applicability rule in option 2A?

Issue 2-1-2: Interference power level

- Status in RAN#101-bis-e in the WF R4-2203131:

  - Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme

    - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

    - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

    - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

  - Option 2: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB

  - Option 3: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB in case of 20% interference PDSCH loading

  - Option 4: 30% PDSCH loading and INR 1 = 9.69 dB, INR 2 = 3.7 dB for Scenario 1 and 20% interference PDSCH loading and INR1 = 10.45 dB and INR2 = 4.6 dB for Scenario 2

- Proposals

  - Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme (China Telecom, [ZTE])

    - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

    - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

    - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

  - Option 2: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB (Apple, E///, HW, Nokia, QC)

  - Option 3: (Intel)

    - For scenario 1, Loading 20 %, INR1 = 10.45 dB and INR2 = 4.6 dB or Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

    - For scenario 2, Loading 20 %, INR1 = 10.45 dB and INR2 = 4.6 dB

- Recommended WF

  - To move forward, can we agree option 2?

Issue 2-1-3: PDSCH loading level

- Status in RAN#101-bis-e in the WF R4-2203131:

  - Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme

    - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

    - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

    - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

  - Option 2: Only consider 20% PDSCH loading level

  - Option 3: Consider 30% interference PDSCH loading for Scenario 1 and 20% interference PDSCH loading for Scenario 2

  - Option 4: Also include 80% loading

  - Option 5: Further check 30% loading level

  - Option 6: Either 30% or 20%

- Proposals

  - Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme (China Telecom, [ZTE])

    - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

    - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

    - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

  - Option 2: Only consider 20% PDSCH loading level (Apple, E///, HW, Nokia, QC)

  - Option 3: Take 20% PDSCH loading level as baseline. Further check 30% loading level for Scenario 1. (CMCC)

  - Option 4: Add more loadings such as 30%, 40% and 50% with lower performance gain (ZTE)

  - Option 5: (Intel)

    - For scenario 1, Loading 20 %, INR1 = 10.45 dB and INR2 = 4.6 dB or Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

    - For scenario 2, Loading 20 %, INR1 = 10.45 dB and INR2 = 4.6 dB

- Recommended WF

  - To move forward, can we agree option 2?

Issue 2-1-4: Tx antenna and LTE CRS port number

- Status in RAN#101-e in the WF R4-2120705:

  - Option 1: Only cover 4 CRS ports

  - Option 2: Cover 2 and 4 for CRS ports

    - Option 2A: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2

    - Option 2B: Use different CRS port number in the tests with different INR levels

  - Option 3: Only cover 2 CRS ports (Nokia)

- Proposals

  - Option 1: Only cover 4 CRS ports (Apple, E///, HW, QC)

  - Option 2: Cover 2 and 4 for CRS ports (Intel, CMCC, ZTE, CTC, Nokia)

    - Option 2A: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2 (Intel, ZTE, Nokia)

    - Option 2B: Consider 2 and 4 CRS ports for Scenario 1, only 4 CRS ports for Scenario 2 (CMCC)

  - Option 3: Only cover 2 CRS ports (Nokia)

- Recommended WF

  - To move forward, can we agree the following?

    - For scenario 1, companies to bring simulation results for both 2 CRS and 4 CRS ports, and further decide whether to define requirements for 2 CRS and/or 4 CRS ports in the next meeting.

    - For scenario 2, only cover 4 CRS ports.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
|  | Sub-topic 2-1: Test setup for 15 kHz SCS scenario Issue 2-1-1: Test setup for scenario 2 Issue 2-1-2: Interference power level Issue 2-1-3: PDSCH loading level Issue 2-1-4: Tx antenna and LTE CRS port number |
| Nokia, Nokia Shanghai Bell | Sub-topic 2-1: Test setup for 15 kHz SCS scenario Issue 2-1-1: Test setup for scenario 2 Whether the test requirement for different schemes can be the same We understand that the requirements could possibly be the same, as measurement gaps and inter-RAT measurement issues are not part of the demod test setup. Test setup for scenario 2 If it is agreed to have two UE capabilities; one for scenario 2 inter-RAT and one for scenario 2 NWA, then it would make sense to have test setups/requirements for both and UE would need to test all that are declared in capabilities. Issue 2-1-2: Interference power level Support option 2: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB Issue 2-1-3: PDSCH loading level Support Option 2: Only consider 20% PDSCH loading level Issue 2-1-4: Tx antenna and LTE CRS port number Agree to the proposed WF. |
| Apple | Sub-topic 2-1: Test setup for 15 kHz SCS scenario Issue 2-1-1: Test setup for scenario 2 We cannot agree with the recommended WF. If we introduce requirements with inter-RAT MO, we are mixing RRM and demod requirements which is not desirable and something that is not commonly done. For the operating SNR we agree that the PBCH decoding should not have any errors, but with the other power detection based hypothesis, there might be errors in UE detection and cannot always be guaranteed. The test requirements cannot be guaranteed to be the same for these reasons. Hence we support option 3. Only 1 test set up and one set of requirements with NWA signalling. Issue 2-1-2: Interference power level We support the recommended WF. Issue 2-1-3: PDSCH loading level We support the recommended WF. Issue 2-1-4: Tx antenna and LTE CRS port number Option 1 is our preference. We have not evaluated the performance with 2CRS ports and we are not in favor of evaluating it at this stage. We suggest sticking to the requirements with 4 CRS ports. |
| CMCC | Sub-topic 2-1: Test setup for 15 kHz SCS scenario Issue 2-1-1: Test setup for scenario 2 Option 2 and Option 2A can be agreed. Besides, we think Option 2B-1 and Option 2B-2 make sense either. For Option 2B-1, we want to add a clarification for PDSCH scheduling, which should be focus on serving cell. - Option 2B-1: If RAN4 define the CRS-IM requirements with Scenario 2 for UE capable of LTE channel bandwidth acquisition without NWA signaling on LTE channel bandwidth, TE does not start PDSCH scheduling of serving cell until UE acquires LTE channel bandwidth, e.g. N x inter-RAT measurement period where N is the number of inter-RAT measurement configuration. (Ericsson) Issue 2-1-2: Interference power level To move forward, Option 2 with Loading level 20% is fine for us. Issue 2-1-3: PDSCH loading level To move forward, Option 2 is fine for us. Issue 2-1-4: Tx antenna and LTE CRS port number Ok with the recommended WF. |
| Huawei | Sub-topic 2-1: Test setup for 15 kHz SCS scenario Issue 2-1-1: Test setup for scenario 2 We support Moderator’s suggestion to define one test requirement. According to our simulation results, there is no performance difference between the two schemes. We also agree that if UE is capable of both BW acquisition schemes, the test setup “Only Inter-RAT MO is configured” should be used for testing. We think that PDSCH can be scheduled immediately after the measurement gap, no additional inter-RAT measurement period is needed for PBCH decoding. Issue 2-1-2: Interference power level We support Option 2. Issue 2-1-3: PDSCH loading level We support Option 2. Issue 2-1-4: Tx antenna and LTE CRS port number For both scenarios 1 and 2, we support only to cover 4 CRS ports. As per our evaluations, the performance gain of CRS-IM is higher with 4 CRS ports than 2 ports. Since there is no difference of CRS-IM processing with different CRS ports, we suggest to only consider 4 ports. Also, 4 ports means higher interference. If the UE can handle higher interference, then it must can handle less interference. There is no need to increase test complexity. |
| ZTE | Issue 2-1-1: Test setup for scenario 2 We think recommended WF is ok. Issue 2-1-2: Interference power level Option 1 is OK. Based on our understanding, it’s not only focus on 20% loading but also should consider other loadings with relative lower performance gain. Issue 2-1-3: PDSCH loading level Option 1 is OK. Issue 2-1-4: Tx antenna and LTE CRS port number We think recommended WF is ok. |
| Vodafone | Issue 2-1-4: Tx antenna and LTE CRS port number We agree with the views expressed by Huawei. Only 4 CRS ports need to be considered. Option 1. |
| Intel | Issue 2-1-1: Test setup for scenario 2 Whether the test requirement for different schemes can be the same Taking into account that measurement gaps should be configured for scenario with Inter-RAT MO, UE will not be able to make PDSCH reception in slots configured for measurement gap and it should be taken into account in the statistics collection procedure or in the PDSCH scheduling. Therefore, we support Option 1. Test setup for scenario 2 Based on our comments above, the different requirements should be defined and also configuring of Inter-RAT MO and measurement gaps in the test will lead to mixing of RRM and Demod functionality. As for Option 2B-1, it is not clear how many times is required for UE to obtain the necessary information. Also, based on our understanding we can not make dynamic switch off/on of measurement gaps. Therefore, they potentially will exist during all test. Therefore, to simplify test procedure of CRS-IM processing (which is the main purpose of this test), we suggest to consider Option 3 Issue 2-1-2: Interference power level Option 2 is fine for us. Issue 2-1-3: PDSCH loading level Option 2 is fine for us. Issue 2-1-4: Tx antenna and LTE CRS port number Recommended WF is fine for us. |
| Qualcomm | Sub-topic 2-1: Test setup for 15 kHz SCS scenario Issue 2-1-1: Test setup for scenario 2 Before we decide on this issue, we have some questions for clarification. In our opinion, for LTE PBCH decoding, UE will see PBCH SINR as INR_i/INR_j during IRAT Measurement, i.e., for stronger interferer PBCH SINR will be ~6dB, while for weaker interferer, PBCH SINR will be ~ -6dB. PBCH SINR is quite low for weaker interferer. So, unless we assume that LTE CBW will be same for all interferers and only rely on stronger interferer, PBCH decoding may have issues for weaker interferer. Can proponents of Option 2 please clarify this? Issue 2-1-2: Interference power level Ok with the recommended WF. Issue 2-1-3: PDSCH loading level Ok with the recommended WF. Issue 2-1-4: Tx antenna and LTE CRS port number We prefer Option 1. 2 CRS ports have not been evaluated as part of the study and we would rather not study it so close to finishing the WI since it typically takes at least 2 meetings for simulation alignment. |
| China Telecom | Issue 2-1-1: Test setup for scenario 2 Whether the test requirement for different schemes can be the same Yes, they can be the same. We agree with Nokia that the measurement gaps are not part of the demod test setup, and we can exclude that time period for throughput measurement. Test setup for scenario 2 We support option 2 Issue 2-1-2: Interference power level Option 1 was proposed by us, which we think it is important to make sure CRS-IM can be enabled for UEs in different locations. But for the WI progress, we can compromise. Issue 2-1-3: PDSCH loading level Same comment to Issue 2-1-2. Issue 2-1-4: Tx antenna and LTE CRS port number We support the recommended WF as compromise. We don’t agree with the argument that 2CRS port was not considered in the study phase. When we proposed 2 CRS ports in the study phase, companies commented that not to consider all the scenarios for study purpose. |
| MediaTek | Issue 2-1-1: Test setup for scenario 2 Our concern is that if it can be guaranteed that PBCH can be decoded correctly in inter-RAT measurement or there is no error in power difference detection. Even if it does under the current test step, it cannot be guaranteed in the real deployment. We believe that NWA can provide better performance. Also, we are wondering how to conduct the simulations to define the requirements. Do we need to run simulations for Scheme 1 (with Inter-RAT MO configured and perform PBCH decoding and/or power difference detection)? Or we just assume UE can acquire the correct BW information through PBCH decoding/power detection and then apply the simulation results from Scheme 2 (with NWA signaling). Issue 2-1-2: Interference power level OK with the recommended WF. Issue 2-1-3: PDSCH loading level OK with the recommended WF. Issue 2-1-4: Tx antenna and LTE CRS port number Prefer Option 1. |
| Ericsson | Issue 2-1-1: Test setup for scenario 2 Whether the test requirement for different schemes can be the same We are fine with option 2. Test setup for scenario 2 We support option 2 with applicability rule option 2A and also additional clarification option 2B-1 and 2B-2. The inter-RAT measurement specified in RRM does not require to decode MIB. So, we think additional time slot for acquire LTE channel bandwidth is needed for the UE capable of LTE channel bandwidth acquisition without NWA signaling, otherwise it is the risk TE start to transmit PDSCH to calculate throughput before UE can perform the CRS-IM receiver. Thus, option 2B-1 and 2B-2 need to be captured together with the applicability rule option 2A in the spec (e.g., TS38.101-4 5.1.1) Issue 2-1-2: Interference power level Support the recommended WF. Issue 2-1-3: PDSCH loading level Support the recommended WF. Issue 2-1-4: Tx antenna and LTE CRS port number We prefer option 2. Considering that the 4 CRS ports configuration gives the most interfering situation and the most gain from CRS-IM. |

Summary for 1st round

Sub-topic 2-1: Test setup for 15 kHz SCS scenario

Issue 2-1-1: Test setup for scenario 2

Summary of round 1 discussion:

- Whether the test requirement for the 2 schemes on acquiring LTE CBW can be the same:

  - Yes (Nokia, HW, ZTE, CTC, E///)

    - Nokia: measurement gaps and inter-RAT measurement issues are not part of the demod test setup

  - No (Apple, Intel)

    - Apple: Mixing RRM and demod requirements is not desirable. There might be errors in UE detection with the power detection based hypothesis.

    - Intel: UE will not be able to make PDSCH reception in slots configured for measurement gap and it should be taken into account in the statistics collection procedure or in the PDSCH scheduling. We can not make dynamic switch off/on of measurement gaps.

  - Need further discussion (QC, MTK)

    - QC: for stronger interferer PBCH SINR will be ~6dB, while for weaker interferer, PBCH SINR will be ~ -6dB. PBCH SINR is quite low for weaker interferer. So, unless we assume that LTE CBW will be same for all interferers and only rely on stronger interferer, PBCH decoding may have issues for weaker interferer.

    - MTK: Our concern is that if it can be guaranteed that PBCH can be decoded correctly. Do we need to run simulations for Scheme 1 (with Inter-RAT MO configured and perform PBCH decoding and/or power difference detection)? Or we just assume UE can acquire the correct BW information

- Test setup for scenario 2

  - Option 2: Define 2 sets of test setup for different UE capabilities: 1) Only Inter-RAT MO is configured, and; 2) Only the new NWA signaling is configured, and define the applicability rule below (Nokia, CMCC, ZTE, HW, CTC, E///)

    - Applicability rule: If UE is capable of both BW acquisition schemes, the test setup “Only Inter-RAT MO is configured” should be used for testing.

    - PDSCH scheduling timing

      - Option A: (CMCC, E///)

- If RAN4 define the CRS-IM requirements with Scenario 2 for UE capable of LTE channel bandwidth acquisition without NWA signaling on LTE channel bandwidth, TE does not start PDSCH scheduling of serving cell until UE acquires LTE channel bandwidth, e.g. N x inter-RAT measurement period where N is the number of inter-RAT measurement configuration.

- If RAN4 define the CRS-IM requirements with Scenario 2 with NWA signaling on LTE channel bandwidth, TE starts PDSCH scheduling immediately after the connection setup.

      - Option B (HW)

- We think that PDSCH can be scheduled immediately after the measurement gap, no additional inter-RAT measurement period is needed for PBCH decoding.

  - Option 3: Define one set of test setup: Only the new NWA signaling is configured. (Apple, Intel)

Recommendations for 2nd round:

- Whether the test requirement for the 2 schemes on acquiring LTE CBW can be the same

  - Further discuss the following aspects:

    - Is it feasible to assume no error in LTE CBW detection based on PBCH decoding or power detection, for the two interferers with different power level?

    - Can not make dynamic switch off/on of measurement gaps.

- Test setup for scenario 2

  - It is agreeable to define one set of test setup with the new NWA signaling on LTE CBW configured. Meanwhile, further discuss whether to define the other set of test setup with only inter-RAT MO configured, and discuss the PDSCH scheduling timing for this test setup:

    - Option A: TE does not start PDSCH scheduling of serving cell until UE acquires LTE channel bandwidth, e.g. N x inter-RAT measurement period where N is the number of inter-RAT measurement configuration.

    - Option B: PDSCH can be scheduled immediately after the measurement gap, no additional inter-RAT measurement period is needed for PBCH decoding.

Issue 2-1-2: Interference power level

Summary of round 1 discussion:

  - Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme (ZTE)

    - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

    - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

    - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

  - Option 2: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB (Apple, E///, HW, Nokia, QC, CMCC, Intel, MTK)

Recommendations for 2nd round:

  - Check if option 2 is agreeable.

Issue 2-1-3: PDSCH loading level

Summary of round 1 discussion:

  - Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme (ZTE)

    - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

    - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

    - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

  - Option 2: Only consider 20% PDSCH loading level (Apple, E///, HW, Nokia, QC, CMCC, Intel, MTK)

Recommendations for 2nd round:

  - Check if option 2 is agreeable.

Issue 2-1-4: Tx antenna and LTE CRS port number

Summary of round 1 discussion:

  - Option 1: Only cover 4 CRS ports (Apple, [E///], HW, QC, Vodafone, MTK)

  - Option 4 (Recommended WF before round 1 discussion) (Nokia, CMCC, ZTE, Intel, CTC)

    - For scenario 1, companies to bring simulation results for both 2 CRS and 4 CRS ports, and further decide whether to define requirements for 2 CRS and/or 4 CRS ports in the next meeting.

    - For scenario 2, only cover 4 CRS ports.

Recommendations for 2nd round:

  - Further discuss and down-select

Discussion on 2nd round

Topic #3: CRS-IM for 30 kHz SCS scenario

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203769 | Apple | Observation #1: There is no measurable gain with CRS-IM with 30KHz SCS for NR, with the assumption that all parameters are known to the UE. Proposal #2: Do not introduce requirements for CRS-IM with 30KHz SCS. |
| R4-2204381 | Intel Corporation | Observations #1: Different LLR weighting algorithms have rather same performance for different scenarios. Observations #2: For Scenario 2 with 30 kHz SCS and 1+1 DMRS configuration - 0% loading: CRS-IM provides 2.7-4.0 dB performance improvement - 10% loading: CRS-IM provides 1.4-1.8 dB performance improvement - 20% loading: CRS-IM provides 0.8-1.2 dB performance improvement. Observations #3: For Scenario 2 with 30 kHz SCS and 1 DMRS configuration - 0% loading: CRS-IM provides 2.0-3.6 dB performance improvement - 10% loading: CRS-IM provides 0.7-1.1 dB performance improvement - 20% loading: CRS-IM provides 0.0-0.6 dB performance improvement. Proposal 1: Define CRS-IM requirements for scenario with 30 kHz SCS using 10% interference loading assumptions and 1+1 DMRS configuration. Proposal 2: Assume that the following information about interference structure is required for CRS-IM processing in scenario with 30 kHz SCS: LTE carrier frequency, bandwidth and CRS port number Proposal 3: For scenario with 30 kHz SCS use the same as for 15 kHz SCS procedure for obtaining the information about interference parameters required for CRS-IM processing. |
| R4-2204526 | CMCC | Observation 1: For Option 1 and Option 3, the performance loss was observed. The loss is due to inaccurate power difference detection, which has too small sample numbers. Observation 2: (Revisited)Option 1, Option 2 and (Revisited)Option 3 can achieve similar performance gain. Observation 3: (Revisited)Option 1, Option 2 and (Revisited)Option 3 have similar power detection and processing complexity. Proposal 1: LLR weighting implementation can up to UE implementation. Proposal 2: For CRS port number information, reuse the GTW agreement - From RAN4 minimum performance requirements aspect, UE follow below default assumption without blind detection as baseline assumption - 4 CRS ports for scenario 2 - By default, number of CRS ports no need to be informed via signalling with following default assumption from RAN4 performance requirements aspect - Number of CRS ports information can be included into NWA signalling (optional) Proposal 3: Reuse the baseline assumption from last meeting’s GTW agreement: No need to introduce NWA signaling for v-shift information. Proposal 4: TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells. Proposal 5: Use 7DS2U with S=6D+4G+4U for the target cell with TDD 30kHz SCS. Proposal 6: Tighten the time offset to less than CP. For example, the time offset for interfering cell 1 is 2 us, and the time offset for interference cell 2 is -1 us. Observation 4: No simulation result mis-alignment between using 2 or 1 DMRS for channel estimation Proposal 7: Which DMRS will be used for channel estimation can up to UE implementation. Observation 5: Comparing with 1+1 DMRS configuration scenario, in 1+0 DMRS scenario, both baseline receiver and LLR receiver have performance loss. Observation 6: LLR receiver can still achieve performance gain comparing with baseline receiver. Proposal 8: UE capability; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. CRS-IM-30kHz-NR-Blind detection Indicates the UE supports LLR weighting based CRS-IM for NR serving cell and LTE interference cell spectrum overlapping scenario, and UE capable of obtaining LTE CBW information by PBCH decoding and/or power detection. NR serving cell SCS is 30kHz; UE; No; No; FR1 only. CRS-IM-30kHz-NR-NWA Indicates the UE supports LLR weighting based CRS-IM for NR serving cell and LTE interference cell spectrum overlapping scenario, and UE not capable of obtaining LTE CBW information by PBCH decoding and/or power detection. NR serving cell SCS is 30kHz; UE; No; No; FR1 only. |
| R4-2204917 | ZTE Corporation | Observation 1: LLR weighting can work without CRS sequence. Proposal 1: LLR weighting without CRS sequence is feasible. Proposal 2: Leave it up to UE implementation. Observation 2: For power detection, it’s only applicable which power has a significant difference. Proposal 3: There is no need to identify the CRS frequency location in Scenario 2 with NR 30 kHz. |
| R4-2205435 | Ericsson | Proposal 1: Option 4: Up to UE implementation for the implementation of LLR weighting in 30 kHz SCS scenario. Proposal 2: Option 3: Reuse the GTW agreement of 15kHz SCS scenario Proposal 3: Option 4: Reuse the GTW agreement of 15kHz SCS scenario Proposal 4: Option 1 for CBW and SCS for target and interference cells can be used for evaluating the feasibility first. Proposal 5: Option 1 for TDD configuration for target cell with 30kHz SCS can be used for evaluating the feasibility first. Proposal 6: Further decide exact test setup for defining requirement purpose after it is proved that 30kHz SCS is feasible for defining requirement. Proposal 7: Separate UE capabilities are also needed for two types of UE, one is capable for do the CRS-IM without the NWA signaling, and the other doesn’t. Proposal 8: Option 1: FFS until feasibility to support CRS-IM for 30kHz SCS is confirmed. |
| R4-2205798 | Huawei, HiSilicon | Observation 1: In the NR 30kHz+ LTE15kHz deployment, the main interference contributing to kth of NR subcarrier are 2kth, (2k-1)th and (2k+1)th LTE subcarrier. Observation 2: There are three different types interference model for NR 30 kHz. Observation 3: In the practical scenario.i.e.20% loading, the performance gain is smaller than 1dB in some cases Observation 4: In most cases, both options achieve same performance. In few cases, performance of option 1 is better than option 2. Observation 5: For the case with 4T4R, the performance gain for 2 CRS ports is limited and only case with 0% loading can achieve gain larger than 1dB. |
| R4-2206095 | Qualcomm Incorporated | Observation 1: UE will have different sampling rate for 30kHz SCS compared to LTE sampling rate. Observation 2: Different sampling rates will cause LTE interference to smudge all the NR REs on symbols colliding with CRS instead of just CRS REs. Observation 3: Different SCS will cause uneven interference within subcarrier, resulting into complicated or unreliable mitigation. Observation 4: UE will have to spend double the resources on measuring/detecting the interference compared to 15kHz SCS. Observation 5: Gain of LLR weighting over no mitigation is only 0.2dB in case of 1+1 DMRS and no gain in case of 1+0 DMRS case. Proposal 1: Do not consider CRS-IM for 30kHz SCS. |

Open issues summary

Sub-topic 3-1: Feasibility evaluation for 30 kHz SCS scenario

Issue 3-1-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario

- Agreements in RAN4 #101e-bis in the WF R4-2203029

  - Feasibility of performing CRS-IM in 30 kHz SCS scenario

    - Feasibility of LLR weighting with CRS sequence or CRS-IC

      - Not feasible

    - Feasibility of LLR weighting without CRS sequence

      - The need of NWA signalling is discussed separately, together with Scenario2 15kHz SCS

      - FFS the feasibility in next meeting

  - Performance gain of LLR weighting in 30 kHz SCS scenario

    - For the next meeting, encourage companies to further evaluate whether testable CRS-IM performance improvement can be achieved for practical scenarios with 30 kHz serving signal SCS.

    - Evaluate the scenario of DMRS 1+0 and DMRS 1+1 separately.

- Simulation results from companies

  - Apple

    - For DMRS 1+1

| Ant. Conf. | LTE Cell Loading | MCS | Gain with CRS-IM |
| --- | --- | --- | --- |
| 4x2 | 0% | 4 | 1.0 |
|  |  | 13 | 1.1 |
|  | 20% | 4 | 0.1 |
|  |  | 13 | 0.4 |
| 4x4 | 0% | 4 | 1.1 |
|  |  | 13 | 1.4 |
|  | 20% | 4 | 0.6 |
|  |  | 13 | 0.4 |

  - Intel

    - For Scenario 2 with 30 kHz SCS and 1+1 DMRS configuration

      - 0% loading: CRS-IM provides 2.7-4.0 dB performance improvement

      - 10% loading: CRS-IM provides 1.4-1.8 dB performance improvement

      - 20% loading: CRS-IM provides 0.8-1.2 dB performance improvement.

    - For Scenario 2 with 30 kHz SCS and 1 DMRS configuration

      - 0% loading: CRS-IM provides 2.0-3.6 dB performance improvement

      - 10% loading: CRS-IM provides 0.7-1.1 dB performance improvement

      - 20% loading: CRS-IM provides 0.0-0.6 dB performance improvement.

  - CMCC

    - Simulation results for 1+1 DMRS configuration and channel estimation with 2DMRS

|  | Baseline Receiver（MRC） | Revisited Option 1 with whole bandwidth averaging granularity | Revisited Option 1 with 10RB averaging granularity | Option 2 with whole bandwidth averaging granularity | Revisited Option 3 with whole bandwidth averaging granularity |
| --- | --- | --- | --- | --- | --- |
| SNR at 70% TP (dB) | 16 | 7.1 | 7.1 | 7.2 | 7.3 |

    - Simulation results for DMRS 1+1 scenario

|  | Baseline receiver Use both DMRS for channel estimation | Revisited Option 1 with whole bandwidth averaging granularity Use both DMRS for channel estimation | Revisited Option 1 with whole bandwidth averaging granularity Use additional DMRS for channel estimation |
| --- | --- | --- | --- |
| SNR at 70% TP (dB) | 16 | 7.1 | 7 |

    - Simulation results for 1+0 DMRS scenario

|  | Baseline receiver | Revisited Option 1 with whole bandwidth averaging granularity |
| --- | --- | --- |
| SNR at 70% TP (dB) | 17.2 | 9.9 |

  - Huawei

    - Summary of gain of LLR weighting over baseline for 4 CRS ports

| Loading of LTE cells |  | 0 % | 10% | 20% |
| --- | --- | --- | --- | --- |
| Performance gain over baseline (70% of maximum TP) | 2RX, LLR weighting with Option 1 | 1.6 | 1.3 | 1.3 |
|  | 2RX, LLR weighting with Option 2 | 1.5 | 1.3 | 1.3 |
|  | 4RX, LLR weighting with Option 1 | 1.3 | 1.2 | 0.9 |
|  | 4RX, LLR weighting with Option 2 | 1.3 | 1.1 | 0.7 |

    - Summary of gain of LLR weighting over baseline for 2 CRS ports

| Loading of LTE cells |  | 0 % | 10% | 20% |
| --- | --- | --- | --- | --- |
| Performance gain over baseline (70% of maximum TP) | 2RX, LLR weighting | 1.2 | 0.9 | 0.7 |
|  | 4RX, LLR weighting | 1.3 | 1.1 | 0.9 |

  - Qualcomm

    - SNR in dB at 70% of peak throughput comparing LLR weighting with no mitigation

| Case | No Mitigation | LLR Weighting | SNR Gain (dB) |
| --- | --- | --- | --- |
| 1+1 DMRS, 4x2 | 11.6 | 11.39 | 0.21 |
| 1+1 DMRS, 4x4 | 9.52 | 9.36 | 0.16 |
| 1+0 DMRS, 4x2 | 11.13 | 11.17 | -0.04 |
| 1+0 DMRS, 4x4 | 10.26 | 10.28 | -0.02 |

- Moderator’s summary of the simulation results

  - For DMRS 1+1, 4 CRS ports

    - For 0% loading: more than 1 dB gain is observed by 3 companies (Apple, Intel, HW)

    - For 10% loading: more than 1 dB gain is observed by 2 companies (Intel, HW)

    - For 20% loading:

      - More than 1 dB gain is observed by 1 company (CMCC)

      - Around 1 dB gain is observed by 2 companies (Intel, HW)

      - Less than 1 dB gain is observed by 2 companies (Apple, QC)

  - For DMRS 1+0, 4 CRS ports

    - For 0% loading: more than 1 dB gain is observed by 1 company (Intel)

    - For 10% loading: around 1dB gain is observed by 1 company (Intel)

    - For 20% loading:

      - More than 1 dB gain is observed by 1 company (CMCC)

      - Less than 1 dB gain is observed by 1 company (Intel)

      - No gain is observed by 1 company (QC)

- Proposals on whether to define CRS-IM requirements for 30kHz SCS

  - Yes (Intel, CMCC, ZTE)

    - Intel: Define CRS-IM requirements for scenario with 30 kHz SCS using 10% interference loading assumptions and 1+1 DMRS configuration.

  - No (Apple, QC)

    - Apple:

      - There is no measurable gain with CRS-IM with 30KHz SCS for NR, with the assumption that all parameters are known to the UE.

    - QC:

      - Observation 1: UE will have different sampling rate for 30kHz SCS compared to LTE sampling rate.

      - Observation 2: Different sampling rates will cause LTE interference to smudge all the NR REs on symbols colliding with CRS instead of just CRS REs.

      - Observation 3: Different SCS will cause uneven interference within subcarrier, resulting into complicated or unreliable mitigation.

      - Observation 4: UE will have to spend double the resources on measuring/detecting the interference compared to 15kHz SCS.

      - Observation 5: Gain of LLR weighting over no mitigation is only 0.2dB in case of 1+1 DMRS and no gain in case of 1+0 DMRS case.

- Recommended WF

  - For companies not supporting to define CRS-IM requirements for 30 kHz SCS, it seems the main concern is no testable gain under some scenarios. Meanwhile, more than 1dB gain is observed for DMRS 1+1 and 4 CRS port scenario with 0% and 10% interference PDSCH loading by several companies.

  - Given that this issue needs to be concluded in this meeting, moderator recommends companies to consider the following compromised proposal:

    - Only define CRS-IM requirements for scenario with 30 kHz SCS assuming [10%] interference loading, 4 CRS ports and 1+1 DMRS configuration

NOTE 1: Reference receiver is LLR weighting without CRS sequence.

NOTE 2: NWA signaling and UE capability signaling are to be discussed separately.

Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario

- Options in RAN4 #101e-bis in the WF R4-2203029

  - Option 1: Propose a LLR weighting method as follows

    - Acquire the CRS time domain location by power detection

    - Estimate the interference power distribution per RE level in the interfered symbols

    - Calculate the actual SINR on these interfered REs

    - Scaling the LLRs by post processing SINR value

  - Option 2: UE should do power estimation three times for three types of REs with different interference models with several RBs granularity or the whole bandwidth and perform LLR weighting respectively.

    - In the NR 30kHz+ LTE15kHz deployment, the main interference contributing to kth of NR subcarrier are 2kth, (2k-1)th and (2k+1)th LTE subcarrier.

      - Interference pattern 1: Subcarrier index 0, 3, 6, 9…,

      - Interference pattern 2: Subcarrier index 1, 4, 7, 10,…,

      - Interference pattern 3: Subcarrier index 2, 5, 8, 11…,

  - Option 3: Measure the interference power on all REs within the symbol colliding with CRS REs and make the averaging with 1 PRB granularity.

  - Option 4: Up to UE implementation

- Proposals

  - Option 4: Up to UE implementation (Intel, CMCC, ZTE, E///, [HW])

    - Intel: Different LLR weighting algorithms have rather same performance for different scenarios.

    - CMCC:

      - (Revisited) Option 1, Option 2 and (Revisited) Option 3 can achieve similar performance gain.

      - (Revisited) Option 1, Option 2 and (Revisited) Option 3 have similar power detection and processing complexity.

    - HW: we set the power estimation granularity in option 1 to 3RBs. In most cases, both options achieve same performance. In few cases, performance of option 1 is better than option 2.

- Recommended WF

  - Agree option 4 if no simulation result mis-alignment due to this issue.

Sub-topic 3-2: Interference parameters needed for CRS-IM in 30 kHz SCS scenario

Issue 3-2-1: CRS port number information

- Agreements in RAN4 #101e-bis in the WF R4-2203029

  - Option 1: UE verify the CRS port number and interfered symbols based on power comparison per symbol level.

    - Option 1a: Further analyse the feasibility of blind detection if testable performance will be observed with all parameters known to UE

  - Option 2: Need NWA since power detection is unreliable for higher SIR.

  - Option 3: Reuse the GTW agreement

    - From RAN4 minimum performance requirements aspect, UE follow below default assumption without blind detection as baseline assumption

      - 4 CRS ports for scenario 2

    - By default, number of CRS ports no need to be informed via signalling with following default assumption from RAN4 performance requirements aspect

    - Number of CRS ports information can be included into NWA signalling (optional)

  - Other Options are not precluded

- Proposals

  - Option 3: Reuse the agreement for 15kHz SCS (Intel, CMCC, E///)

    - From RAN4 minimum performance requirements aspect, UE follow below default assumption without blind detection as baseline assumption

      - 4 CRS ports for scenario 2

    - By default, number of CRS ports no need to be informed via signalling with following default assumption from RAN4 performance requirements aspect

    - Number of CRS ports information can be included into NWA signalling (optional)

- Recommended WF

  - Agree option 3.

Issue 3-2-2: CRS frequency location information

- Agreements in RAN4 #101e-bis in the WF R4-2203029

  - Option 1: There is no need to identify the CRS frequency location in Scenario 2 with NR 30 kHz.

  - Option 2: CRS location information should be configured with network assistance information.

  - Option 3: Further analyse the feasibility of blind detection if testable performance will be observed with all parameters known to UE

  - Option 4: Reuse the GTW agreement

    - The baseline assumption: No need to introduce NWA signaling for v-shift information

- Proposals

  - Option 1: There is no need to identify the CRS frequency location in Scenario 2 with NR 30 kHz. (ZTE)

  - Option 4: Reuse the agreement for 15kHz SCS (Intel, CMCC, E///)

    - Note: No need to introduce NWA signaling for v-shift information for 30kHz SCS

- Recommended WF

  - Agree option 4?

Sub-topic 3-3: Test setup for 30 kHz SCS scenario

Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS

- Agreements in RAN4 #101e-bis in the WF R4-2203029

  - Option 1: If 30kHz SCS is feasible, use TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells

  - Other options are not precluded

- Proposals

  - Option 1: TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells (CMCC, E/// - for performance evaluation purpose)

- Recommended WF

  - Agree Option 1 if the requirements for 30 kHz SCS will be defined.

Issue 3-3-2: TDD configuration for target cell with 30kHz SCS

- Related agreements for 15kHz SCS in RAN#101-e in the WF R4-2120705:

  - Use DSUDDDSUDD with S = 10D+2G+2U for the interference LTE with TDD 15kHz SCS.

  - Note: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame, which is aligned with the TDD-TDD EN-DC configuration in demod test.

- Agreements in RAN4 #101e-bis in the WF R4-2203029

  - Option 1: If 30kHz SCS is feasible, Use 7DS2U with S=6D+4G+4U for the target cell with TDD 30kHz SCS

  - Other options are not precluded

- Proposals

  - Option 1: 7DS2U with S=6D+4G+4U for the target cell with TDD 30 kHz SCS. (CMCC, E/// - for performance evaluation purpose)

- Recommended WF

  - Agree Option 1 if the requirements for 30 kHz SCS will be defined.

Issue 3-3-3: Time offset among cells

- Related agreements for 15kHz SCS in RAN#101-e in WF R4-2120705:

  - Time offset: The serving cell is 3 us and -1 us for interfering cell 1 and cell 2 respectively

- Proposals

  - Option 1: Tighten the time offset to less than CP. For example, the time offset for interfering cell 1 is 2 us, and the time offset for interference cell 2 is -1 us. (CMCC)

- Recommended WF

  - Encourage feedback

Sub-topic 3-4: UE capability for CRS-IM in 30 kHz SCS scenario

Issue 3-4-1: UE capability

- Proposals

  - Option 1: Similar to the UE capability for 15kHz SCS in scenario 2, define two separate UE capabilities for 30kHz SCS CRS-IM (CMCC, E/// - if the feasibility is confirmed)

    - CRS-IM in scenario 2 with 30kHz SCS without new network assistant signaling

    - CRS-IM in scenario 2 with 30kHz SCS with new network assistant signaling

- Recommended WF

  - Agree Option 1 if the requirements for 30 kHz SCS will be defined.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Sub-topic 3-1: Feasibility evaluation for 30 kHz SCS scenario Issue 3-1-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario Sub-topic 3-2: Interference parameters needed for CRS-IM in 30 kHz SCS scenario Issue 3-2-1: CRS port number information Issue 3-2-2: CRS frequency location information Sub-topic 3-3: Test setup for 30 kHz SCS scenario Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS Issue 3-3-2: TDD configuration for target cell with 30 kHz SCS Issue 3-3-3: Time offset among cells Sub-topic 3-4: UE capability for CRS-IM in 30 kHz SCS scenario Issue 3-4-1: UE capability |
| Nokia, Nokia Shanghai Bell | Sub-topic 3-1: Feasibility evaluation for 30 kHz SCS scenario Issue 3-1-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario We see this as a limited case and with high workload needed for adding requirements for 30kHz. Hence, we prefer to not create requirements for 30kHz. |
| Apple | Sub-topic 3-1: Feasibility evaluation for 30 kHz SCS scenario Issue 3-1-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario We still prefer not to define requirements for this case. There is no significant performance improvement with additional UE complexity, and it is meaningless to expect UE to support this feature. With lower interference loading, more gain will be observed with LLR weighing, it doesn’t mean we decrease the loading for that. No loading would give the most gain, but it wouldn’t be practical to define requirements with 0% loading in our understanding. Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario Okay with recommended WF. Sub-topic 3-2: Interference parameters needed for CRS-IM in 30 kHz SCS scenario Issue 3-2-1: CRS port number information We are okay with recommended WF. But we don’t support defining requirements for this case. Issue 3-2-2: CRS frequency location information Our understanding is the v-shift information doesn’t help much as there would be interference in multiple REs per symbol. Sub-topic 3-3: Test setup for 30 kHz SCS scenario Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS We are okay with recommended WF. Issue 3-3-2: TDD configuration for target cell with 30 kHz SCS We are okay with recommended WF, with same assumption on delayed start of LTE frame by 2 subframes to align the transmission pattern. Issue 3-3-3: Time offset among cells 1us for cell 1 and -0.25 us for cell 2. But we don’t support defining requirements for this case. Sub-topic 3-4: UE capability for CRS-IM in 30 kHz SCS scenario Issue 3-4-1: UE capability We are okay with recommended WF. |
| CMCC | Sub-topic 3-1: Feasibility evaluation for 30 kHz SCS scenario Issue 3-1-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario We are fine with recommended WF. Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario We support recommended WF. Sub-topic 3-2: Interference parameters needed for CRS-IM in 30 kHz SCS scenario Issue 3-2-1: CRS port number information Support the recommended WF Issue 3-2-2: CRS frequency location information Support the recommended WF Sub-topic 3-3: Test setup for 30 kHz SCS scenario Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS Support the recommended WF Issue 3-3-2: TDD configuration for target cell with 30 kHz SCS Support the recommended WF Issue 3-3-3: Time offset among cells We support Option 1. If other companies have concerns on the proposal, we are fine to wait more simulation results. Sub-topic 3-4: UE capability for CRS-IM in 30 kHz SCS scenario Issue 3-4-1: UE capability We support recommended WF. |
| Huawei | Sub-topic 3-1: Feasibility evaluation for 30 kHz SCS scenario Issue 3-1-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario As per our simulation results, around 1dB performance gain can be seen in some scenarios, such as 0% or 10% loading with 2/4Rx. However, in some other scenarios, the performance gain is less than 1dB, such as 20% loading with 2/4Rx. considering still diverse evaluation results, maybe further alignment is needed. Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario If introducing CRS-IM in 30kHz SCS scenario, we prefer to leave this to up to UE implementation. Sub-topic 3-2: Interference parameters needed for CRS-IM in 30 kHz SCS scenario Issue 3-2-1: CRS port number information Maybe it is feasible to reuse the agreement for 15kHz SCS. But it is necessary to check the feasibility later if RAN4 agrees to define the performance requirements. Issue 3-2-2: CRS frequency location information Maybe it is feasible to reuse the agreement for 15kHz SCS and not to introduce v-shift But it is necessary to check the feasibility later if RAN4 agrees to define the performance requirements. Sub-topic 3-3: Test setup for 30 kHz SCS scenario Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS We are fine with Option 1 if the requirements for 30 kHz SCS will be defined. Issue 3-3-2: TDD configuration for target cell with 30 kHz SCS We are fine with Option 1 if the requirements for 30 kHz SCS will be defined. Issue 3-3-3: Time offset among cells We prefer to tighten the time offset to less than CP. We noticed that 3 us for the serving cell agreed for 15kHz is larger than the CP for 30kHz. Therefore, we suggest to use a smaller time offset which is within the CP for 30kHz, such as 1us. Sub-topic 3-4: UE capability for CRS-IM in 30 kHz SCS scenario Issue 3-4-1: UE capability If introducing CRS-IM in 30kHz SCS scenario, we suggest to introduce separate UE capability for UE with and without NWA signalling. |
| ZTE | Sub-topic 3-1: Feasibility evaluation for 30 kHz SCS scenario Issue 3-1-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario If the performance gain of 30 kHz SCS scenario is more than 1dB, maybe we should define CRS-IM requirements. Based on other companies simulation results, it has significant performance in 30 kHz SCS scenario. Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario We agree with recommended WF. Sub-topic 3-2: Interference parameters needed for CRS-IM in 30 kHz SCS scenario Issue 3-2-1: CRS port number information We agree with recommended WF. Issue 3-2-2: CRS frequency location information We think option 3 is ok.In our understanding, CRS frequency location information is indicated by UE via v-shift which is used to estimate the REs of serving cell in 15kHz. However for scenario 2 with NR SCS 30kHz, based on our analysis, all the NR REs in interfered symbols will be interfered by LTE CRS. In this case, it seems no necessary to introduce CRS frequency location information. Sub-topic 3-3: Test setup for 30 kHz SCS scenario Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS We agree with recommended WF. Issue 3-3-2: TDD configuration for target cell with 30 kHz SCS We are okay with recommended WF. Issue 3-3-3: Time offset among cells We are okay with recommended WF. Sub-topic 3-4: UE capability for CRS-IM in 30 kHz SCS scenario Issue 3-4-1: UE capability We are agree recommend WF. |
| Intel | Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario Recommended WF is fine for us, which is aligned with 15 kHz SCS scenario agreement. Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS Support Option 1. Issue 3-3-2: TDD configuration for target cell with 30kHz SCS Support Option 1. Issue 3-3-3: Time offset among cells We have rather similar discussion for MMSE-IRC requirements. One of the options is to keep the same TO assumptions as for 15 kHz SCS. At current stage, we support this option. Probably, for this meeting we can collect more companies views to find the way forward. |
| Qualcomm | Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario Ok with the recommended WF. Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS Ok with the recommended WF. Issue 3-3-2: TDD configuration for target cell with 30kHz SCS We are ok with Option 1 in principle. However, we prefer to keep this open because S slot may have some impact on performance and it is FFS as per GTW agreement. Issue 3-3-3: Time offset among cells Our preference is to keep all the offsets within CP. So, we prefer 1us for cell 1 and -0.25 us for cell 2. |
| MediaTek | Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario Ok with the recommended WF. Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS Ok with the recommended WF. Issue 3-3-2: TDD configuration for target cell with 30kHz SCS Ok with the recommended WF. |
| Ericsson | Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario OK with the recommended WF. Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS Ok with the recommended WF. We also suggest to assume the center frequency of both target cell and interference cell(s) are same. Issue 3-3-2: TDD configuration for target cell with 30kHz SCS OK with the recommended WF. Issue 3-3-3: Time offset among cells We support option 1. Agreement of time offset from 15kHz SCS scenario is exceed the CP length of 30kHz SCS scenario. |

Summary for 1st round

Sub-topic 3-1: Feasibility evaluation for 30 kHz SCS scenario

Issue 3-1-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario

Agreement:

Only define CRS-IM requirements for scenario with 30 kHz SCS assuming [10%] interference loading, 4 CRS ports and 1+1 DMRS configuration under the condition with enough performance discrimination between CRS-IM on and CRS-IM off i.e. at least 1dB performance difference observed

- Channel BW: 20MHz

- FFS on special slot configuration

- Interested companies can bring results with Rel-15 rate matching (symbol level)

Issue 3-1-2: Implementation of LLR weighting in 30 kHz SCS scenario

Summary of round 1 discussion:

  - Up to UE implementation if no simulation result mis-alignment due to this issue. (Apple, CMCC, HW, ZTE, Intel, QC, MTK, E///)

Tentative agreement:

  - Up to UE implementation if no simulation result mis-alignment due to this issue.

Sub-topic 3-2: Interference parameters needed for CRS-IM in 30 kHz SCS scenario

Issue 3-2-1: CRS port number information

- Proposals

  - Option 3: Reuse the agreement for 15kHz SCS

    - From RAN4 minimum performance requirements aspect, UE follow below default assumption without blind detection as baseline assumption

      - 4 CRS ports for scenario 2

    - By default, number of CRS ports no need to be informed via signalling with following default assumption from RAN4 performance requirements aspect

    - Number of CRS ports information can be included into NWA signalling (optional)

Agreement: Option 3

Issue 3-2-2: CRS frequency location information

Agreement:

  - Reuse the agreement for 15kHz SCS under scenario 2 on the information related to the CRS frequency location including centre frequency, channel bandwidth

    - Note: No need to introduce NWA signaling for v-shift information for 30kHz SCS

Sub-topic 3-3: Test setup for 30 kHz SCS scenario

Issue 3-3-1: CBW and SCS for target and interference cells for 30 kHz SCS

Summary of round 1 discussion:

  - Option 1: TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells (CMCC, Apple, HW, ZTE, Intel, QC, MTK, E///)

    - E///: also suggest to assume the center frequency of both target cell and interference cell(s) are same.

Tentative agreement:

  - Agree Option 1 (TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells)

Recommendations for 2nd round:

  - Further discuss the following issue:

    - Whether or not to assume the center frequency of both target cell and interference cell(s) are same

Issue 3-3-2: TDD configuration for target cell with 30kHz SCS

Summary of round 1 discussion:

  - Option 1: 7DS2U with S=6D+4G+4U for the target cell with TDD 30 kHz SCS. (CMCC, Apple, HW, ZTE, Intel, QC - agree in principle, MTK, E///)

    - QC: prefer to keep this open because S slot may have some impact on performance and it is FFS as per GTW agreement.

Tentative agreement:

  - Agree to use 7DS2U for the target cell with TDD 30 kHz SCS

Recommendations for 2nd round:

  - Further discuss the configuration in the special slot.

Issue 3-3-3: Time offset among cells

Summary of round 1 discussion:

- Option 1: Tighten the time offset to less than CP

  - Option 1A: 2 us for cell 1, -1 us for cell 2. (CMCC, E///)

  - Option 1B: 1 us for cell 1, -0.25 us for cell 2 (Apple, QC)

  - Option 1C: 1us (HW)

- Option 2: keep the same TO assumptions as for 15 kHz SCS (Intel)

Recommendations for 2nd round:

  - Further discuss and check if it is possible to reuse the agreement for inter-cell IRC in 30 kHz SCS scenario.

Sub-topic 3-4: UE capability for CRS-IM in 30 kHz SCS scenario

Issue 3-4-1: UE capability

- Proposals

  - Option 1: Similar to the UE capability for 15kHz SCS in scenario 2, define two separate UE capabilities for 30kHz SCS CRS-IM (CMCC, E/// - if the feasibility is confirmed)

    - CRS-IM in scenario 2 with 30kHz SCS without new network assistant signaling

    - CRS-IM in scenario 2 with 30kHz SCS with new network assistant signaling

Agreement:

Agree Option 1 if the requirements for 30 kHz SCS will be defined.

- With LTE-MO configured, the detailed methods to get CRS CHBW information subject to UE

Discussion on 2nd round

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on general part and 15kHz NR SCS scenario for CRS-IM receiver | China Telecom | - Cover Topic #2 - Discuss the UE feature for both 15kHz and 30kHz SCS (to be added in RAN4 UE feature list) - Discuss the CR work split for 15kHz SCS scenario |
| WF on 30 kHz NR SCS scenario for CRS-IM receiver | CMCC | - Cover Sub-topic 3-1 and 3-3 - Discuss the CR work split for 30kHz SCS scenario |
| Draft TR 38.833 v1.2.0: Further enhancement on NR demodulation performance | China Telecom | - To implement the TPs to be approved for MU-MIMO MMSE-IRC - For email approval after the meeting |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2204375 | Discussion on UE feature list and capability signalling | Intel Corporation | Noted |  |
| R4-2205788 | Discussions on UE feature list for MMSE-IRC receiver | Huawei,HiSilicon | Discussed in thread #323 |  |
| R4-2203769 | Discussion on CRS-IM requirements | Apple | Noted |  |
| R4-2204381 | CRS-IM requirements for 30 kHz SCS scenarios with overlapping spectrum for LTE and NR | Intel Corporation | Noted |  |
| R4-2204526 | Discussion on the CRS-IM for NR 30kHz SCS | CMCC | Noted |  |
| R4-2204917 | Discussion CRS-IM Feasibility for 30kHz SCS Scenario | ZTE Corporation | Noted |  |
| R4-2205435 | Discussion on the general for CRS-IM | Ericsson | Noted |  |
| R4-2205494 | draft LS on UE capability and network assistant signalling for CRS interference mitigation in scenarios with overlapping spectrum for LTE and NR | China Telecom | Revised | LS to RAN2 To capture the agreements in Topic #1, Sub-topic 3-2 and Sub-topic 3-4 |
| R4-2205798 | Discussions on 30kHz CRS-IM receiver | Huawei,HiSilicon | Noted |  |
| R4-2206071 | Views on the requirements for CRS-IM receiver | MediaTek inc. | Noted |  |
| R4-2206095 | Views on CRS Interference Mitigation for 30kHz SCS in NR | Qualcomm Incorporated | Noted |  |
| R4-2203770 | Discussion on Network Assistance Signalling for CRS-IM | Apple | Noted |  |
| R4-2204382 | Network assistance signaling for CRS-IM receiver for scenarios with overlapping spectrum for LTE and NR | Intel Corporation | Noted |  |
| R4-2204527 | Discussion on the network assistant signaling necessity for CRS-IM | CMCC | Noted |  |
| R4-2204833 | On Necessity of Network assistant signalling for CRS-IM | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2204918 | Network assistant signaling | ZTE Corporation | Noted |  |
| R4-2205433 | Discussion on the network assistant signaling for CRS-IM | Ericsson | Noted |  |
| R4-2205495 | Discussion on the network assistance signalling and UE capability for CRS-IM | China Telecom | Noted |  |
| R4-2205799 | Discussions on NWA and capability signaliing for 15kHz CRS-IM receiver | Huawei,HiSilicon | Noted |  |
| R4-2203771 | Discussion on Test setup for CRS-IM requirements | Apple | Noted |  |
| R4-2204383 | Test setup for CRS-IM receiver for scenarios with overlapping spectrum for LTE and NR | Intel Corporation | Noted |  |
| R4-2204528 | Discussion on the test setup for CRS-IM | CMCC | Noted |  |
| R4-2204919 | Test setup for 15 kHz SCS scenario | ZTE Corporation | Noted |  |
| R4-2205434 | Discussion on the test set-up for CRS-IM | Ericsson | Noted |  |
| R4-2205496 | Discussion on the test setup for CRS-IM requirement definition | China Telecom | Noted |  |
| R4-2205800 | Discussion on test setup for 15kHz CRS-IM receiver | Huawei,HiSilicon | Noted |  |
| R4-2206052 | On test setup for CRS-IM | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2206107 | Views on Test Setup for CRS Interference Mitigation in NR | Qualcomm Incorporated | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia / Nokia Shanghai Bell | Karsten Petersen | Karsten.petersen@nokia-bell-labs.com |
| Apple | Manasa Raghavan | Manasa.raghavan@apple.com |
| MediaTek | Licheng Lin | licheng.lin@mediatek.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)