3GPP TSG-RAN WG4 Meeting #102-e	R4-227391

Electronic Meeting, February 21 – March 3, 2022

Title: 	TP for 38.101-5: clause 6.3 output power dynamics

Source: 	CATT

Agenda item:	10.13.4.1

Document for:	Approval

- Introduction

This document presents test proposal for introducing the requirements for output power dynamics for NTN satellite UE in TS38.101-5 v0.0.1.

- Text Proposal

================================ Text Proposal =====================================

6.3	Output power dynamics

6.3.1	Minimum output power

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value.

The minimum output power is defined as the mean power in at least one sub-frame 1 ms. The minimum output power shall not exceed the values specified in Table 6.3.1-1.

Table 6.3.1-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -40 | 4.515 |
| 10 | -40 | 9.375 |
| 15 | -40 | 14.235 |
| 20 | -40 | 19.095 |

6.3.2	Transmit OFF power

Transmit OFF power is defined as the mean power in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports.

The transmit OFF power is defined as the mean power in a duration of at least one sub-frame (1 ms) excluding any transient periods. The transmit OFF power shall not exceed the values specified in Table 6.3.2-1.

Table 6.3.2-1: Transmit OFF power

| Channel bandwidth | (MHz) | 5, 10, 15, 20 |
| --- | --- | --- |
| REF_SCS | (kHz) | 15 |
| Transmit OFF power | (dBm) | -50 |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |

6.3.3	Transmit ON/OFF time mask

The requirements for transmit ON/OFF time mask defined in section 6.3.3 in TS38.101-1 [x] are applied for NTN satellite UE.

6.3.4	Power control

The requirements for Power control defined in section 6.3.4 in TS38.101-1 [x] are applied for NTN satellite UE.