3GPP TSG-RAN WG4 Meeting # 102-e	R4-220xxxx

Electronic Meeting, Feb 26 – Mar 3 2022

Agenda Item:	x.x.3

Source: 	ZTE

Title: 	WF on NTN UE RF requirements

Document for:	Approval

- Way forward

NTN UE Tx requirement:

Issue 1-1-1:   where to capture isolation region (1.5km) information

Agreement:

To capture the descriptions on the assumption of 1.5km isolation region just in TR 38.863. A clarification that TN UEs not deploying in isolation region is for simplifying the simulation

Issue 1-2-1:   minimum output power

Agreement:

Option 2:  reuse the TN minimum output power requirements for NTN UE

Issue 1-3-1:   UE coexistence requirement

Agreement: Option 1 (-50dBm/MHz), exception for specific bands need to be further discussed and decided.

Issue 1-3-2:   protected TN bands for UE co-existence n255/n256

  - Option 1: all TN bands with some exceptional bands

Agreement: Option 1

Issue 1-3-4:   exceptional TN bands for UE co-existence n255

Agreement: No need exception on band n24.

Issue 1-4-1:   MPR

- Proposals

  - Option 1:  to reuse TN MPR requirement

Agreement: Option 1

Note that MPR may be revisited in the future based on NTN UE ACLR/SEM requirement.

Issue 1-4-2:   A-MPR for n256

Agreement:

NS_01, NS_24 and NS_100 for n256

FFS for how to protect n2, n25, n70 and whether A-MPR required

NTN UE Rx requirement:

Issue 2-1-1: Duplexer for n256

  - Option 1: dedicated 30MHz [QC, CATT, Ericsson, ZTE, Huawei]

  - Option 2:  reusing n65 [ Xiaomi,Skyworks, Hughes/EchoStar, Omnispace, MTK,Intelsat, THALES]

Agreement:

Keep UE implementation freedom for both options and define RAN4 RF requirements compatible with above options.

For REFSENS, choosing the minimum requirements among option 1 and option 2.

Issue 2-1-2: maximum input power

- Proposals

  - Option 1:  relaxed by 15dB from TN UE

Agreement: Option 1

Issue 2-1-3: ACS requirement in case 2

Agreement: ACS requirements in case 2 can be specified based on the agreed maximum input level if needed.

- Open issues

NTN UE Tx requirement:

Issue 1-3-3:   exceptional TN bands for UE co-existence n256

Agreement:

Tentative agreement: no need for exception on band n65  (pending on further check by EchoStar)

FFS: For the exceptions on TN band protection for n256, a critical one is band 34 as it is adjacent.

Issue 1-4-3:   A-MPR for n255

Agreement:

Further check the regulatory requirements on band n255 and confirm whether A-MPR required and whether new NS needed

Recommendations for 2nd round:

Issue 2-1-1: Duplexer for n256

NTN UE Rx requirement:

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |

If both option 1 and option 2 is supported,  how to define in-band blocking and out of band blocking requirement for option 1 with 30MHz dedicated filter needs further discussions.