3GPP TSG-RAN WG4 Meeting # 103-e                                                            R4-2208138

Electronic Meeting, May 09 – May 20, 2022

Title: 	Discussion of test setup, MU and TT for FR1 NR repeater

Source: 	CATT

Agenda item:	9.5.5.2.1

Document for:	Approval

- Introduction

The NR repeater RF requirements are closed in last RAN4 meeting. There’s one importance issue for the conformance test discussion which is test uncertainty. This contribution provides our analysis for the conducted test uncertainty for NR repeater. The test tolerance is also analyzed and proposed.

- Discussion

2.1 Measurement system set-up

Most of the measurement system set-up can be inherited from either NR BS or E-UTRA repeater type 1-C. After comparing NR BS and E-UTRA repeater, reusing NR BS as much as possible may be simpler. For example, for NR BS, the transmitted power, EVM, frequency error are merged to one set-up, which is simpler than the E-UTRA approach. The followings are the proposal for repeater

- Repeater output power, transmitter ON/OFF power, frequency stability, EVM, unwanted emissions, ACRR for repeater type 1-C

Figure 1: Measuring system set-up for repeater type 1-C output power, frequency stability, EVM, unwanted emissions, ACRR

Note that a repeater is a bi-directional device. The signal generator may need protection.

There’s one issue for the TDD repeater EVM is that if the EVM for downlink and uplink are tested simultaneously, the set up for the EVM of TDD repeater need to be defined specifically. Figure 2 is a candidate set up for TDD repeater EVM test if specific set up is defined.

Hybrid

NR TDD Signal generator 2

Repeater under test

Signal analyser

NR TDD Signal generator 1

Circulator

Circulator

Figure 2: TDD repeater EVM test set up for simultaneous downlink and uplink test

This set up may need group delay information for the repeater to set up the timing of signal generator 1 and signal generator 2, i.e signal generator 2 for uplink need some time advance compared with the timing of signal generator 1. In the real network, signal generator 2 is UE which knows TA from BS. However, signal generator may not have such a function then the group delay information or a proper TA for the repeater is needed if Figure 2 is agreed as a set-up.

- Output intermodulation for repeater type 1-C

Figure 3: Measuring system set-up for repeater type 1-C output intermodulation

Note that a repeater is a bi-directional device. The signal generator may need protection.

- Out of band gain

# Figure 4: Measuring system set-up for out of band gain

Note that a repeater is a bi-directional device. The signal generator may need protection.

- Input intermodulation

# Figure 5: Measuring system set-up for input intermodulation.

Note that a repeater is a bi-directional device. The signal generator may need protection.

2.2 Test uncertainty

In general, when the NR repeater requirements reuse BS requirements, the BS test uncertainty can be reused. For the NR repeater specific requirements, E-UTRA repeater test uncertainty can be reused with some frequency range adaption for NR. Table 1 shows the NR repeater test uncertainty consideration after considering of BS and E-UTRA repeater. For the NR repeater, there’re some requirements difference for downlink and uplink. However, as the test uncertainties are related to frequency or the requirements level, there will be no test uncertainty difference for the downlink and uplink.

In theory, the measurement uncertainty for repeater is greater than BS because there’s input signal for the output. However, comparing the test uncertainty of E-UTRA repeater and E-UTRA BS, there’s no extra uncertainty added to BS for most of the requirements. That means reusing BS test uncertainty for many of the repeater requirements are feasible. Thus through comparing the measurement uncertainties of NR BS and E-UTRA repeater in Table 1, Table 2 summarizes the test uncertainty proposals for NR conducted repeater.

Table 2: NR conducted repeater test uncertainty proposal

| Reuse BS test uncertainty | Reuse E-UTRA repeater test uncertainty |
| --- | --- |
| 6.2 Repeater output power 6.5.2 Adjacent Channel Leakage Power Ratio 6.5.3 Operating band unwanted emissions 6.5.4 Transmitter spurious emissions 6.8 Output intermodulation The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station. The interference signal must have a spurious emission level at least 10 dB below the spurious levels required. 6.10.1 Transmitter OFF power | 6.3 Frequency stability 6.4 Out of band gain (update the frequency range as shown in Table 1) 6.6 Error vector magnitude (No test for low power EVM) 6.7 Input intermodulation 6.7.1 General requirement 6.7.2 Co-location with BS/repeater in other systems 6.9 Adjacent Channel Rejection Ratio (ACRR) |

Table 1: NR repeater test uncertainty consideration

| Clause | NR BS | E-UTRA repeater | NR repeater DL and UL |
| --- | --- | --- | --- |
| 6.2 Repeater output power | ±0.7 dB, f ≤ 3 GHz ±1.0 dB, 3 GHz < f ≤ 6 GHz (Note) ±1.5 dB, for bands n46 and n96 NOTE: Test system uncertainty values for 4.2 GHz < f ≤ 6 GHz apply for BS operates in licensed spectrum only. | ±0,7 dB, f ≤ 3.0 GHz ±1,0 dB, 3.0 GHz < f ≤ 4.2 GHz | Reuse BS |
| 6.3 Frequency stability | ± 12 Hz | ±12 Hz Measurement results of ±500 Hz | Reuse E-UTRA repeater |
| 6.4 Out of band gain | No such requirement | ±0,5 dB, f ≤ 3.0 GHz ±0,8 dB, 3.0 GHz < f ≤ 4.2 GHz Calibration of test set-up shall be made without DUT in order to achieve the accuracy | Reuse E-UTRA repeater and update the frequency range. ±0,5 dB, f ≤ 3.0 GHz ±0,8 dB, 3.0 GHz < f ≤ 6 GHz ±1.3 dB, for bands n46 and n96 Calibration of test set-up shall be made without DUT in order to achieve the accuracy NOTE: Test system uncertainty values for 4.2 GHz < f ≤ 6 GHz apply for BS operates in licensed spectrum only. |
| 6.5.2 Adjacent Channel Leakage Power Ratio | ACLR/ CACLR BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB Absolute power ±2.0 dB, f ≤ 3 GHz Absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) Absolute power ±3 dB, for bands n46 and n96 CACLR BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB CACLR absolute power ±2.0 dB , f ≤ 3 GHz CACLR absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz CACLR absolute power ±3 dB, for bands n46 and n96 (Note) | No such requirement | Reuse BS |
| 6.5.3 Operating band unwanted emissions | ±1.5 dB, f ≤ 3 GHz ±1.8 dB, 3 GHz < f ≤ 6 GHz (Note) ±2.2 dB, for bands n46 and n96 | (except 9.1.5.4) ±1,5 dB, f ≤ 3.0 GHz ±1,8 dB, 3.0 GHz < f ≤ 4.2 GHz The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station according toTS36.141 [11] 9.1.5.4 Protection of the BS receiver in the operating band for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB | Reuse BS The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station according toTS38.141-1 |
| 6.5.4 Transmitter spurious emissions | Transmitter spurious emissions, Mandatory Requirements 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB 19 GHz < f ≤ 26 GHz: ±4.5 dB Transmitter spurious emissions, Additional spurious emission requirements ±2.0 dB for > -60 dBm, f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 4.2 GHz ±3.0 dB, 4.2 GHz < f ≤ 6 GHz ±3.0 dB for ≤ -60 dBm, f ≤ 3 GHz ±3.5 dB, 3 GHz < f ≤ 4.2 GHz ±4.0 dB, 4.2 GHz < f ≤ 6 GHz ±4.0 dB, for bands n46 and n96 Transmitter spurious emissions, Co-location ±3.0 dB | In E-UTRA and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power 9kHz < f <= 4 GHz ±2,0 dB; 4GHz < f <= 19 GHz ±4,0 dB. The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station according to TS36.141 [11] | Reuse BS |
| 6.6 Error vector magnitude | ± 1% | 1% signal analyser, 2% stimulus signal | Reuse E-UTRA repeater |
| 6.7 Input intermodulation 6.7.1 General requirement | No such requirement | ±1,2 dB | Reuse E-UTRA repeater |
| 6.7.2 Co-location with BS/repeater in other systems | ±3.0 dB | ±1,2 dB | Reuse E-UTRA repeater |
| 6.8 Output intermodulation | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests in 6.6.3 (ACLR), 6.6.4 (OBUE) and 6.6.5 (spurious emissions) which have to be carried out in the presence of the interferer. ±1.0 dB | ±2,1 dB operating band unwanted emission The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station For spurious emission: In UTRA and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power; 9 kHz < f £ 4 GHz ±2,0 dB; 4GHz < f £ 19 GHz ±4,0 dB. The interference signal must have a spurious emission level at least 10 dB below the spurious levels required in 9.2. | Reuse BS The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station. The interference signal must have a spurious emission level at least 10 dB below the spurious levels required. |
| 6.9 Adjacent Channel Rejection Ratio (ACRR) | No such requirement | ±0,7 dB | Reuse E-UTRA repeater |
| 6.10.1 Transmitter OFF power | ±2.0 dB , f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) ±3 dB, for bands n46 and n96 | No such requirement | Reuse BS |

2.3 Test tolerance

Similar to test uncertainty, the E-UTRA repeater and BS test tolerance are compared, then NR repeater TT are proposed.

Table 3: NR repeater test tolerance consideration

| Clause | BS | E-UTRA repeater | NR repeater DL and UL |
| --- | --- | --- | --- |
| 6.2 Repeater output power | Normal and extreme conditions: 0.7 dB, f ≤ 3.0 GHz 1.0 dB, 3.0 GHz < f ≤ 6GHz (Note) NOTE 1: TT values for 4.2 GHz < f ≤ 6.0 GHz apply for BS operates in licensed spectrum only. | Normal and extreme conditions: 0.7 dB, f ≤ 3.0 GHz 1.0 dB, 3.0 GHz < f ≤ 4.2GHz | Reuse BS |
| 6.3 Frequency stability | 12 Hz | 12 Hz | Reuse E-UTRA repeater (the same as E-UTRA repeater) |
| 6.4 Out of band gain | N/A | 0,5 dB for for f 3.0 GHz; 0.8 dB for 3.0 GHz < f 4.2GHz | Reuse E-UTRA repeater and update the frequency range. ±0,5 dB, f ≤ 3.0 GHz ±0,8 dB, 3.0 GHz < f ≤ 6 GHz NOTE: TT values for 4.2 GHz < f ≤ 6 GHz apply for BS operates in licensed spectrum only. |
| 6.5.2 Adjacent Channel Leakage Power Ratio | ACLR/CACLR: BW ≤ 20MHz: 0.8dB BW > 20MHz: 1.2 dB Absolute ACLR/CACLR: 0 dB | N/A | Reuse BS |
| 6.5.3 Operating band unwanted emissions | Offsets < 10MHz 1.5 dB, f ≤ 3.0GHz 1.8 dB, 3.0GHz < f ≤ 6GHz (Note) Offsets ≥ 10MHz 0dB | General requirements: 1,5 dB inside 2 x pass band BW < 10 MHz from pass band edge. 0 dB elsewhere in the operating band 0 dB on additional requirements, f ≤ 3.0 GHz General requirements: 1,8 dB inside 2 x pass band BW < 10 MHz from pass band edge. 0,3 dB elsewhere in the operating band. 0,3 dB on additional requirements, 3.0 GHz < f ≤ 4.2 GHz | Reuse E-UTRA repeater, update the frequency range to 6GHz |
| 6.5.4 Transmitter spurious emissions | 0dB | 0dB | Reuse BS (the same as E-UTRA repeater) |
| 6.6 Error vector magnitude | ± 1% | ± 1.25 % | Reuse E-UTRA repeater |
| 6.7 Input intermodulation 6.7.1 General requirement | No such requirement | ±1,2 dB | Reuse E-UTRA repeater |
| 6.7.2 Co-location with BS/repeater in other systems | 0dB | ±1,2 dB | Reuse E-UTRA repeater |
| 6.8 Output intermodulation | 0dB | As in 6.5.3 and 6.5.4 | Reuse E-UTRA repeater |
| 6.9 Adjacent Channel Rejection Ratio (ACRR) | No such requirement | ±0,7 dB | Reuse E-UTRA repeater |
| 6.10.1 Transient period | N/A | N/A | N/A |

Table 4: NR repeater TT proposal

| Reuse BS TT | Reuse E-UTRA TT |
| --- | --- |
| 6.2 Repeater output power 6.5.2 Adjacent Channel Leakage Power Ratio 6.5.4 Transmitter spurious emissions | 6.3 Frequency stability 6.4 Out of band gain (update the frequency range as shown in Table 3) 6.5.3 Operating band unwanted emissions (update the frequency range as shown in Table 3) 6.6 Error vector magnitude (No test for low power EVM) 6.7 Input intermodulation 6.7.1 General requirement 6.7.2 Co-location with BS/repeater in other systems 6.8 Output intermodulation 6.9 Adjacent Channel Rejection Ratio (ACRR) |

- Conclusion

This contribution provides our proposal for NR repeater test set-up, test uncertainty and test tolerance. The proposals are shown in Figure 1-5, Table 2 and Table 4.

Reference

[1] RP-212129, “Revised WID on NR Repeaters”, Qualcomm, RAN#93e

[2] TS 38.106 v1.0.0

[3] TS 38.141-1

[4] TS 38.143