3GPP TSG-RAN WG4 Meeting # 103-e			                                      R4-2208355

Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2314 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to maintain pre-configured measurement gap in TS 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The corresponding RAN2 signalling are still absent in current general requirements of pre-configured measurement gap. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the corresponding indication for UE capabilities of pre-configured gap and network signalling in general requirements of pre-configured measurement gap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The general requirements for pre-configured measurement gap are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

### 9.1.7	Pre-configured measurement gap

#### 9.1.7.1	Introduction

A UE capable of Pre-configured measurement gap (Pre-MG) pattern can be configured with a Pre-MG pattern via RRC signalling [2].

The gap interruption requirements in Section 9.1.2 apply to Pre-MG when Pre-MG is activated, and no gap interruption is expected when Pre-MG is deactivated.

-	The requirements apply for NR standalone operation with single carrier and NR CA.

Editor Note: RAN4 agreed to deprioritize Pre-MG for MR-DC in Rel17.

#### 9.1.7.2	Requirements applicability

The requirements related to pre-configured measurement gap apply provided:

-	UE indicates support of preconfiguredUE-AutonomousMeasGap-r17 and/or preconfiguredNW-ControlledMeasGap-r17, and

-	either a single per-UE measurement gap is pre-configured by the network, or one or two per-FR measurement gaps are pre-configured by the network, and

-	one of measurement gap patterns among measurement gap patterns #0 ~ #25 is configured for pre-configured measurement gap, and

-	UE is in NR SA with single carrier or with NR CA.

A measurement gap is configured as pre-configured measurement gap if preConfigInd-r17 in the corresponding GapConfig is indicated by network.

If UE indicates support of only preconfiguredNW-ControlledMeasGap-r17, UE can expect the network to configure deactivatedMeasGapList-r17..

If a measurement gap is configured as pre-configured measurement gap, the applicability of measurement gap patterns is defined in Table 9.1.2-3.A pre-configured measurement gap may not be sufficient to perform PRS measurements because it is not always activated as determined from the signalling provided by the network or from the autonomous rules to determine the status of the pre-configured measurement gap. In this scenario, the UE will inform the network that it is going to start/stop PRS measurements with the configured pre-configured measurement gap by initiating the existing LocationMeasurementIndication procedure.

#### 9.1.7.3	Requirements

Any of the measurement Gap pattern #0 to #25 defined in Table 9.1.2-1 can be configured as Pre-MG pattern.

The UE can determine the Pre-MG status based on autonomous activation/deactivation mechanism or based on network-controlled activation/deactivation mechanism.

A UE capable of both autonomous and network-controlled mechanisms for activation/deactivation of Pre-MG pattern will not use autonomous rules to determine the activation/deactivation status of the pre-configured MG if the network provides the activation/deactivation status via RRC indication preconfiguredNW-ControlledMeasGap-r17.

##### 9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism

Requirements in this section apply when autonomous mechanism [1] is used for activation/deactivation of Pre-MG pattern.

The UE can autonomously change the Pre-MG status from activation to deactivation or vice versa based on any of the following triggering conditions:

-	DCI, timer or RRC based active BWP switching,

-	Activation/deactivation of SCell(s),

-	Addition/removal of any measurement object(s)

-	Addition/release/change of a SCell in carrier aggregation,

-	Initiation of LocationMeasurementIndication procedure specified in clause 5.5.6 [2].

The UE shall autonomously determine the status of the per-UE Pre-MG pattern as deactivated immediately after the configuration of the per-UE Pre-MG pattern provided that all the configured measurements can be performed without measurement gaps. The UE shall autonomously determine the status of the per-FR Pre-MG pattern as deactivated immediately after the configuration of the per-FR Pre-MG pattern provided that all the configured measurements in the same FR can be performed without measurement gaps.

A measurement can be performed by the UE without measurement gaps if any of the following conditions is met:

-	The UE is configured with SSB based intra-frequency measurements, and the conditions defined for SSB based intra-frequency measurement without gaps in Clause 9.2.1 are met, or

-	The UE is configured with SSB based inter-frequency measurements, and the conditions defined for SSB based inter-frequency measurement without gaps in Clause 9.3.1 are met, or

-	The UE is configured with CSI-RS based intra-frequency measurements.

The UE shall autonomously determine the status of the per-UE Pre-MG pattern as activated immediately after the configuration of the per-UE Pre-MG pattern provided that at least one of the configured measurements cannot be performed without measurement gaps. The UE shall autonomously determine the status of the per-FR Pre-MG pattern as activated immediately after the configuration of the per-FR Pre-MG pattern provided that at least one of the configured measurements in the same FR cannot be performed without measurement gaps.

A measurement cannot be performed by the UE without measurement gaps if any of the following conditions is met:

-	The UE is configured with SSB based intra-frequency measurements, and the conditions defined for SSB based intra-frequency measurement without gaps in Clause 9.2.1 are not met, or

-	The UE is configured with SSB based inter-frequency measurements, and the conditions defined for SSB based inter-frequency measurement without gaps in Clause 9.3.1 are not met, or

-	The UE is configured with any of the following measurements:

-	CSI-RS based inter-frequency measurements, or

-	NR PRS-based positioning measurements, or

-	E-UTRA Inter-RAT measurements, or

-	E-UTRA Inter-RAT RSTD and E-CID measurements, or

-	UTRA Inter-RAT measurements.

##### 9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism

The requirements in this section apply when network-controlled mechanism [1] is used for activation/deactivation of Pre-MG pattern.

For per-UE Pre-configured MG,

-	the UE determines that the Pre-configured MG is activated if deactivatedMeasGapList is absent or any of gap IDs for pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) is not indicated in deactivatedMeasGapList in BWP-DownlinkDedicated for the active DL BWP in any of the activated CCs, or

if for any of the deactivated SCCs deactivatedMeasGapList is absent or any of gap IDs for pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) is not indicated in deactivatedMeasGapList in CellGroupConfig,

-	otherwise, the UE determines that the Pre-configured MG is deactivated

For per-FR Pre-configured MG,

-	the UE determines that the Pre-configured MG is activated if deactivatedMeasGapList is absent or any of gap IDs for pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) is not indicated in deactivatedMeasGapList in BWP-DownlinkDedicated for the active DL BWP in any of the activated CCs in the corresponding FR, or

if deactivatedMeasGapList is absent or any of gap IDs for pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) is not indicated in deactivatedMeasGapList in CellGroupConfig for any of the deactivated SCCs in the corresponding FR,

-	otherwise, the UE determines that the Pre-configured MG is deactivated

##### 9.1.7.3.3	Requirements for reception/transmission during activation/deactivation

The requirements in this section apply when autonomous mechanism or network-controlled mechanism is used for activation/deactivation [1] of Pre-MG pattern.

If per-UE Pre-MG pattern is activated then the UE is not required to conduct reception/transmission from/to the corresponding serving cells during the gap occasion according to the same principles as described for per-UE measurement gaps in clause 9.1.2. Otherwise, the UE can be scheduled for reception/transmission of signals in all the serving cells during the gap occasion.

If per-FR Pre-MG pattern is activated then the UE is not required to conduct reception/transmission from/to the corresponding serving cells during the gap occasion on the same FR according to the same principles as described for per-FR measurement gaps in clause 9.1.2. Otherwise, the UE can be scheduled for reception/transmission of signals in all the serving cells during the gap occasion in the same FR.

# < End of change 1>