3GPP TSG-RAN WG4 Meeting # 103-e											R4-2208365

Electronic Meeting, May 09 – May 20, 2022

Source: 	OPPO

Title: 	Discussion on general requirements on RedCap UE

Agenda Item:	9.19.3.1.1

Document for:	Discussion

# 1 Introduction

In last RAN4 meeting, a way forward on RRM requirements for RedCap UE was agreed in [1]. In this contribution, we provide our further consideration on the remaining open issues of RRM requirements for RedCap UE.

- Discussion

Paging reception

Impact on paging reception requirements for FD-FFD/TDD UEs - Option 1 (Xiaomi, OPPO, CMCC, E///, MTK, vivo): For 1 Rx, there is no impact on the requirement for maximum interruption in paging reception, i.e. Rel-15 requirements shall apply to RedCap. - Option 2 (HW): For RedCap with 1RX, the interruption time shall not exceed TSI-NR + 3*Ttarget_cell_SMTC_period ms.

In current spec, the interruption time for NR to NR cell re-selection is specified as TSI-NR + 2*Ttarget_cell_SMTC_period ms, which consists of the time for UE to read SIB from the target cell and the time for UE to synchronize to the target cell. Companies raised concerns that additional time may be needed if the PO occasions of the target cell are within the RedCap specific initial BWP. In our view, when the PO occasions for RedCap are configured in initial DL BWP, CD-SSB is always expected in the certain BWP. Then, we think the current specification is applicable for RedCap UE.

Proposal 1: For 1 Rx, there is no impact on the requirement for maximum interruption in paging reception, i.e. Rel-15 requirements shall apply to RedCap.

Small data transmission for RedCap

SDT for RedCap with 1 Rx – time window - Option 1 (vivo, E///, MTK): Time windows defining the valid measurements used for TA validation are reused from Rel-17 SDT discussions but need to be updated to reflect the RedCap 1 Rx measurement times (compared to 2 Rx measurement times). - Option 1a (MediaTek): Support reusing the TA validation requirements from SDT rel-17 WI to SDT for RedCap in rel-17 with relaxed accuracy performance for the case of 1Rx RedCap. - Option 2 (CMCC, HW, Apple): Discuss the time windows for 1Rx RedCap UE after the 2Rx requirements are agreed.

As agreed in last meeting, for FDD and TDD, SDT requirements for RedCap UE with 2 Rx are reused from those defined for legacy NR UEs under Rel-17 SDT WI. The time window for 1Rx RedCap UE could be discussed based on 2Rx requirements. We think time windows it is feasible to reuse the TA validation requirements from SDT rel-17 WI to SDT for RedCap in rel-17 with relaxed accuracy performance for the case of 1Rx RedCap

Proposal 2: Reusing the TA validation requirements from SDT rel-17 WI to SDT for RedCap in rel-17 with relaxed accuracy performance for the case of 1Rx RedCap.

- Conclusion

Proposal 1: For 1 Rx, there is no impact on the requirement for maximum interruption in paging reception, i.e. Rel-15 requirements shall apply to RedCap.

Proposal 2: Reusing the TA validation requirements from SDT rel-17 WI to SDT for RedCap in rel-17 with relaxed accuracy performance for the case of 1Rx RedCap

- Reference

[1] R4-2206950, WF on RedCap RRM requirements, Ericsson