3GPP TSG-RAN WG4 Meeting # 103-e										R4-2208408

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	8.1.3

Source: 	CMCC

Title: 	Discussion on UE requirements for 6GHz licensed spectrum

Document for:	Discussion

1. Introduction

In last RAN4 #102 e-meeting, the remaining issues on UE requirements include MPR, ACLR, SEM, REFSENSE and blocking.

In this contribution, we focus on the remaining issues for UE.

2. Discussion

For Tx requirements, the remaining issues are related to co-existence scenario, i.e. ACLR. The main controversy exists in whether to reuse the same requirements in TS 38.101-1 or the requirements in TR 38.921.

For the simulations performed for ACLR in TS 38.101-1, it assumes 2GHz reference frequency. Although the frequency of new 5G band is becoming higher, but the same ACLR is reused considering the frequency difference is not that much. For 6GHz licensed spectrum, new simulation is performed and final simulation shows RF requirements could be relaxed. From our perspective, we don’t have very strong view on which option could be finally approved. Both the ACLR in 38.101-1 and 38.921 are OK for us. But if option 2(relaxed value compared with legacy requirement) is finally approved, it’s suggested to relatively improve the MPR requirements since ACLR is the key factor that determine MPR value. Improved MPR will help to extend UL coverage which is very important for the higher 6GHz frequency.

Observation 1: both the ACLR and SEM in 38.101-1 and 38.921 are OK for us.

Proposal 1: if relaxed ACLR is approved finally, we suggest to improve MPR value to extend UL coverage which is very important for the higher 6GHz frequency.

For Rx blocking requirements, since ACS reuse the same value as defined in 38.101, the same blocking requirements as in 38.101-1 are suggested for 6425-7125MHz.

Proposal 2: the same blocking requirement as in 38.101-1 is also applicable for 6425-7125MHz.

3. Conclusions

In this contribution, our understanding of remaining UE RF requirement of 6GHz is listed with following observations and proposals:

Observation 1: both the ACLR and SEM in 38.101-1 and 38.921 are OK for us.

Proposal 1: if relaxed ACLR is approved finally, we suggest to improve MPR value to extend UL coverage which is very important for the higher 6GHz frequency.

Proposal 2: the same blocking requirement as in 38.101-1 is also applicable for 6425-7125MHz.

4. Reference

[1] R4-2206373, WF on UE RF requirements, Qualcomm.