3GPP TSG-RAN WG4 Meeting #103-e	R4-2208472

# Electronic Meeting, May 09 – May 20, 2022

Agenda Item:	9.15.8.6

Source:	MediaTek Inc.

Title:	Discussion on LBT impacts on RRM requirements in FR2-2

Document for:		Discussion

# Introduction

In this paper, our views on the RRM requirements extended due to LBT impacts in FR2-2 are provided, including the RLM OOS and BFD.

# LBT impacts on RRM requirements

- The step of FR2-2 RRM requirements extension due to missed SMTC/SSB occasions is equal to N SMTC/SSB occasions, where N is RX beam sweeping scaling factor

- The RRM requirements are extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC occasions group consists of N consecutive SSB/SMTC occasions. An SSB/SMTC occasions group is not available, when at least one SSB/SMTC occasion in the group is not transmitted by the gNB.

  - The definition of SSB/SMTC occasion follows Rel-16 NR-U definition

  - FFS how to introduce the test case

  - FFS if agreement applies to RLM OOS and BFD

In the last meeting, the RRM requirements extended due to LBT impacts are discussed in Meeting #102-e [2] as follows.

I can be observed as the following Table 8.1A.2.2-1 and Table 8.5A.2.2-1, in FR1 NR-U, the existing RLM OOS and BFD requirements are extended by a fixed number of samples, rather than extended by the number of SSB/SMTC occasions (groups) not available. It also can be observed, for RLM OOS, the number of extended samples is 3 ~ 7 for SNR >= -7 dB and 6 ~ 14 in SNR < - 7dB; for BFD, the number of extended samples is 2 ~ 5 for SNR >= -7 dB and 3 ~ 7 in SNR < - 7dB.

Table 8.1A.2.2-1: Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA

| Configuration | TEvaluate_out_SSB,CCA (ms) |  | TEvaluate_in_SSB,CCA (ms) |
| --- | --- | --- | --- |
|  | RLM-RS SSB Es/IotNote4 ≥-7 dB | RLM-RS SSB Es/Iot Note4 <-7 dB |  |
| no DRX | Max(200, Ceil(17*P)*TSSB) | Max(200, Ceil(24*P)*TSSB) | Max(100, Ceil((5+Lin)*P)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*15*P)*Max(TDRX,TSSB)) | Max(200, Ceil(1.5*20*P)*Max(TDRX,TSSB)) | Max(100, Ceil(1.5*(5+Lin)*P)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil(13*P)*TDRX | Ceil(16*P)*TDRX | Ceil((5+Lin)*P)*TDRX |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: When DRX is not configured, Lin is the number of RLM-RS SSB occasions which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. When DRX is configured, Lin is the number of DRX cycles in which at least one RLM-RS SSB occasion is not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. The UE is not required to determine the availability of SSB occasions more frequent than once per DRX cycle length, when configured with DRX. NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. NOTE 4: RLM-RS SSB Es/Iot is the averaged Es/Iot over the most recent previous out-of-sync evaluation period. |  |  |  |

Table 8.5A.2.2-1: Evaluation period TEvaluate_BFD_SSB_CCA

| Configuration | TEvaluate_BFD_SSB_CCA (ms) |  |
| --- | --- | --- |
|  | BFD-RS SSB Es/Iot Note2 ≥ -7 dB | BFD-RS SSB Es/Iot Note2 < -7 dB |
| no DRX | Max(50, Ceil((10 P) TSSB)) | Max(50, Ceil((12 P) TSSB)) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 8 P) Max(TDRX,TSSB)) | Max(50, Ceil(1.5 10 P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(7 P) TDRX | Ceil(8 P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length. Note 2: BFD-RS SSB Es/Iot is the averaged BFD-RS SSB Es/Iot over the most recent previous evaluation period. |  |  |

Observation 1: In FR1 NR-U, the existing RLM OOS and BFD requirements are extended by a fixed number of samples, rather than extended by the number of SSB/SMTC occasions (groups) not available.

Therefore, with LBT, we propose the FR2-2 RLM OOS and BFD requirements are extended by K SSB/SMTC occasions groups, where K is the extending parameter which is regardless whether the SSB/SMTC occasions groups are available at UE or not.

Besides, assuming low LBT failure probability in FR2-2, it would be sufficient to extend the delay by one additional SSB/SMTC occasions group, i.e. K=1. The exemplary tables are shown below:

Table 8.5.2.2-2: Evaluation period TEvaluate_BFD_SSB for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil((5+K) P N) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil((7.5+ K) P N) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil((5+ K) P N) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.1.2.2-2: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil((10+ K) P N) TSSB) | Max(100, Ceil(5 P N) TSSB) |
| DRX cycle≤320ms | Max(200, Ceil((15+ K) P N) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P N) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil((10+ K) P N) TDRX | Ceil(5 P N) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Proposal 1: With LBT, the FR2-2 RLM OOS and BFD requirements are extended by K SSB/SMTC occasions groups, where K is the extending parameter which is regardless whether the SSB/SMTC occasions groups are available at UE or not, and K=1.

In additional, one unified requirement can avoid complicated UE behaver and testing complexity, thus we propose not to differentiate the requirements for the case with channel condition of SNR >= -7 dB and the case with channel condition of SNR < - 7dB, i.e. the same K value for the both cases.

Proposal 2: With LBT, for the FR2-2 RLM OOS and BFD requirements, same requirements for the case with channel condition of SNR >= -7 dB and the case with channel condition of SNR < - 7dB.

# Conclusion

In this contribution, we discuss the RRM measurement requirement in FR2-2. We have the following observations and proposals.

Observation 1: In FR1 NR-U, the existing RLM OOS and BFD requirements are extended by a fixed number of samples, rather than extended by the number of SSB/SMTC occasions (groups) not available.

Proposal 1: With LBT, the FR2-2 RLM OOS and BFD requirements are extended by K SSB/SMTC occasions groups, where K is the extending parameter which is regardless whether the SSB/SMTC occasions groups are available at UE or not, and K=1.

Proposal 2: With LBT, for the FR2-2 RLM OOS and BFD requirements, same requirements for the case with channel condition of SNR >= -7 dB and the case with channel condition of SNR < - 7dB.

# References

- R4-2206919, WF on NR extension to 71 GHz RRM requirements (Part 1), Qualcomm.

- R4-2206924, WF on NR extension to 71 GHz RRM requirements (Part 2), Intel Corporation.