3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208723

Electronic Meeting, May 09 – May 20, 2022

Source:	ZTE Corporation

Title:	On extended DRX enhancements for RedCap

Agenda item:	9.19.3.2

Document for:	Discussion

# 1	Introduction

Issue 1-4-1: Inactive state requirements when idle eDRX is longer than 10.24s - Option 1: The inactive UE requirements are based on inactive DRX or inactive eDRX when inactive eDRX is configured (Huawei vivo MTK) - Option 2: Based on the paging monitoring cycle of T agreed in RAN2 (Apple Ericsson xiaomi Nokia) IDLE eDRX[s]; Inactive eDRX[s]; Outside CN PTW or during CN PTW; T. >10.24; Not configured; During CN PTW; Shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer. >10.24; Not configured; Outside CN PTW; RAN paging cycle.. >10.24; ≤10.24; During CN PTW; Shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer. >10.24; ≤10.24; Outside CN PTW; INACTIVE eDRX cycle. Agreement: no consensus

In RAN4 102-e, there were discussions on the extended DRX related requirements for reduced capability UEs. Some options were captured in the WF [1] which are copy pasted here.

- This paper discusses the issues listed above and provide our views.

2	Discussions

Issue 1-4-1: Inactive state requirements when idle eDRX is longer than 10.24s

eDRX has been agreed to be supported for RedCap UEs by RAN2. In the TR 38.875 [2], eDRX is introduced with a reference to LTE systems. In our view, the inactive DRX from LTE can be seen as a baseline for our discussion here for NR RedCap systems.

8.3 Extended DRX for RRC Inactive and/or Idle 8.3.1 Description of feature In LTE connected to EPC, the UE may be configured with an extended DRX (eDRX) cycle. The UE may operate in eDRX only if the UE is configured by NAS and the cell indicates support for eDRX in System Information (note that there is no System Information indication for NB-IoT). In RRC_IDLE, the eDRX cycle has the maximum value of 2621.44 seconds (43.69 minutes). For NB-IoT the maximum value of eDRX cycle is 10485.76 seconds (2.91 hours). Hyper SFN (H-SFN) is broadcasted in System Information and incremented by one when SFN wraps around. The Paging Hyperframe (PH) refers to the H-SFN in which the UE starts to monitor for paging during a Paging Time Window (PTW), see e.g. [12]. RAN2 has studied the following topics related to extended DRX for RRC_IDLE and RRC_INACTIVE: - Analysis of UE power saving - Analysis of upper and lower bound of extended DRX cycles - Analysis of mechanisms for extended DRX

Considering the design in LTE, we think that measurement (here discussed in the RRM session) is more related to DRX cycle than paging reception windows, and thus, we propose that the inactive state requirements were defined based on inactive DRX or inactive eDRX when inactive eDRX is configured.

The inactive state requirements were defined based on inactive DRX or inactive eDRX when inactive eDRX is configured.

3	Conclusion

- Proposal 1: The inactive state requirements were defined based on inactive DRX or inactive eDRX when inactive eDRX is configured.

References

- R4-2206974

- TR 38.875