3GPP TSG-RAN WG4 Meeting # 103-e 	R4-2208556

# Electronic Meeting, May 09 – May 20, 2022

Source: 	Huawei, HiSilicon

Title: 	TP for overlapping CA

Agenda Item:	11.2.3.3

Document for:	Approval

1 Introduction

A new SI has been approved on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths [1]. One of the objective is to study the use of overlapping UE channel bandwidths.

Study the use of overlapping UE channel bandwidths (from both UE and network perspective) to cover operator’s license spectrum for both UL and DL, and if new gNB channel bandwidths are needed.

NOTE:	For all considered solutions, new (dedicated) channel filters (e.g. non-integer-multiples of 5MHz) are not considered for the UE and not prioritized for the gNB.

In this contribution, we provide further text proposal for the TR 38.844 for the following sub-clauses.

- 6.2.3.2      Configuration and signalling aspects

- 6.4	Generic solutions guidance

- 6.5	RAN1 and RAN2 impact

3 Conclusions

It is proposed to approve the TP on the overlapping CA approach for the TR.

References

RP-202103, New SID: Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, T-Mobile USA, Ericsson

R4-2201512, RF requirements for overlapping CA, , Huawei

Text Proposal for TR 38.844

==== Start of Changes ===

#### 6.2.3.2      Configuration and signalling aspects

In this approach, network configures two cell-defining SSB in the allocated irregular spectrum block and the two cells are configured with overlapping regular channel bandwidth. From procedure wise, the network can indicate UE the location and bandwidth by using the existing procedures on individual CC. From UE perspective, there are two independent cells with overlapped bandwidth. For a particular UE, CA with overlapped cells can be configured to fully utilize the network bandwidth.

==== next changed section ===

6.4.3	Overlapping CA	 (two cell)

This approach can be applied to various size of  irregular channel bandwidths, and it does not require to define new channel filters to be designed and tested for BS and UE.

==== next changed section ===

#### 6.5.4	Overlapping CA (two cells)

CSI-RS measurement and reporting for the component carriers are specified in TS 38.213 and to be performed independently per-carrier, similarly for PDCCH.  It is up to the gNB scheduler to avoid collisions of different transmissions for the overlapping part with overlapping CA cross-carrier scheduling.

Existing UE capability signalling does not provide any possibility for UE to indicate support of overlapped CA.  New signalling would be required in order for this approach to become feasible.

==== End of Changes ===