3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211674

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.7.2.2

Source: 	CATT

Title: 	Discussion on remaining issues of timing and signal characteristics requirements for HST FR2

Document for:	Discussion

Introduction

RAN4 has completed UE RRM requirements for HST FR2. However, there are still some open issues for RRM core requirements for HST FR2. WF on RRM requirements for NR HST FR2 was approved in [1].

In this contribution, we provide our consideration of open issue and give our proposal.

Discussion

In last meeting, the scheduling restriction after TCI state switch is still open as listed in [1]:

Issue 1-3-2: Scheduling/Transmit restriction after TCI state switch Way Forward: Further discussion is needed whether and how to define transmit or scheduling restriction for UL after the TCI state switch when highSpeedLargeOneStepUL-TimingFR2-r17 is disabled. - Option 2: No impact on UE behavior - Option 3: Define scheduling restriction on DL and UL after inter-RRH TCI state switch and before PRACH transmission when highSpeedLargeOneStepUL-TimingFR2-r17 is disabled - Option 4: after the TCI state switch, the UE shall not transmit except for RACH preamble in the new target TCI before one of the following conditions is fulfilled: - the new timing advance is acquired and applied in the target TCI state according to the requirements in clause 7.3; - the UL transmission is scheduled by the gNB. In this case, the requirements in clause 7.1.2.1 apply. - Other options are not precluded

In previous meeting, it is agreed that no indication of intra-RRH and inter-RRH. When highSpeedLargeOneStepUL-TimingFR2-r17 is disabled, UE has no information whether it is intra-RRH or inter-RRH switch. Option 3 doesn’t work. The network knows the TCI state of intra-RRH and inter-RRH. If it is intra-RRH TCI state, it can be controlled by NW for the scheduling. Option 2 and option 4 are both OK. If it is inter-RRH TCI state, the UE should not transmit UL signal until new correct timing. Option 4 is workable.

Proposal 1: When highSpeedLargeOneStepUL-TimingFR2-r17 is disabled, after the TCI state switch, option 4 is acceptable.

Conclusion

In this contribution, we provide our consideration of open issue for HST FR2 RRM requirements and our proposal is:

Proposal 1: When highSpeedLargeOneStepUL-TimingFR2-r17 is disabled, after the TCI state switch, option 4 is acceptable.

Reference

[1] R4-2210608, WF on HST FR2 RRM Core Requirement Maintenance, Nokia