3GPP TSG RAN WG4 Meeting #104-e                       R4-2211691

Electronic Meeting, August 15th - August 26th, 2022

Agenda item:			9.18.4.1

Source:	CATT

Title:	Discussion on general for RRM performance requirements in RedCap devices

Document for:	Discussion

- Introduction

In the last RAN4 meeting, we initially discussed RRM performance requirements for RedCap UEs. The workplan and test case list for RedCap RRM performance part were approved in [1] and [2], but there is still no conclusion about the test configurations. Therefore, in this contribution, some considerations on the test configurations for RedCap UEs are further discussed.

- Discussion

SSB Configurations

In section A.3.10.1 in current 3GPP TS 38.133 [3], the channel bandwidth of SSB pattern 2, SSB pattern 4 and SSB pattern 6 is configured as 40MHz, while the maximum bandwidth of RedCap UEs is only 20MHz for FR1, so the channel bandwidth in existing SSB configurations may exceed the maximum bandwidth for RedCap UEs in FR1. It is suggested to specify SSB configurations for RedCap UEs in a separate chapter, and a new configuration of '30 kHz SCS and 20 MHz BW' should be introduced for RedCap UEs in FR1.

Observation 1: In current spec, the channel bandwidth configured in SSB pattern 2, SSB pattern 4 and SSB pattern 6 exceed the maximum bandwidth for RedCap UEs in FR1.

Proposal 1: It is suggested to specify SSB configurations for RedCap UEs in a separate chapter.

Proposal 2: It is suggested to a new configuration of '30 kHz SCS and 20 MHz BW' for RedCap UEs in FR1.

BWP configurations

In the last meeting, it was suggested that RAN4 to decide on whether to introduce new configuration for RedCap specific BWP based on type of test cases agreed for RedCap [4].

In section A.3.9A in 3GPP TS 38.133 [3], we need to focus on whether the existing bandwidth in BWP configuration exceeds the maximum transmission bandwidth for RedCap UEs. According to the correspondence of the maximum transmission bandwidth configuration NRB, UE channel bandwidth and subcarrier spacing specified in Table 5.3.2-1 in 3GPP TS 38.101-1 [5], the bandwidth defined in Table A.3.9.2.2-1 and Table A.3.9.3.2-1 [3] do not exceed the maximum transmission bandwidth for RedCap UEs, therefore we suggest to reuse existing BWP configurations defined in A.3.9.2 and A.3.9.3 in 3GPP TS 38.133 for RedCap UEs and not to introduce new configuration for RedCap specific BWP.

Observation 2: Neither the bandwidth defined in section A.3.9.2 nor section A.3.9.3 in 3GPP TS 38.133 exceeds the maximum transmission bandwidth for RedCap UEs.

Proposal 3: It is suggested that existing BWP configurations defined in A.3.9.2 and A.3.9.3 in TS 38.133 are reused for RedCap UEs, and no new configuration is needed for RedCap specific BWP.

- Summary

In this paper, we provide our views on the test configurations for RedCap UEs. From this discussion we have derived the following observations and proposals:

Observation 1: In current spec, the channel bandwidth configured in SSB pattern 2, SSB pattern 4 and SSB pattern 6 exceed the maximum bandwidth for RedCap UEs in FR1.

Proposal 1: It is suggested to specify SSB configurations for RedCap UEs in a separate chapter.

Proposal 2: It is suggested to a new configuration of '30 kHz SCS and 20 MHz BW' for RedCap UEs in FR1.

Observation 2: Neither the bandwidth defined in section A.3.9.2 nor section A.3.9.3 in 3GPP TS 38.133 exceeds the maximum transmission bandwidth for RedCap UEs.

Proposal 3: It is suggested that existing BWP configurations defined in A.3.9.2 and A.3.9.3 in TS 38.133 are reused for RedCap UEs, and no new configuration is needed for RedCap specific BWP.

- References

- R4-2209911 WI workplan for RedCap for RRM performance part, Ericsson

- R4-2210595 Test case list for RedCap RRM performance part, Ericsson

- 3GPP TS 38.133, Requirements for support of radio resource management

- R4-2210286 Email discussion summary for [103-e][214] NR_redcap_RRM_1, Moderator (Ericsson)

- 3GPP TS 38.101-1 V17.5.0, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone