3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211842

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.8.2.2

Source:	Apple

Title:	On testing configuration for HO with PSCell

Document for:	Discussion

# Introduction

In RAN4 #103e meeting, the WF[1] for HO with PScell was agreed, but there are still some open issues for testing configuration as followings,

Issue 2-1-1: Test cases design principle - FR1+FR2 test cases - Option 1: Test case design is delayed until testability issues are solved - Option 2: Introduce the test case and define the applicability - Option 3: Do not introduce the test

In this contribution, we discuss the above remaining issues from last RAN4 meeting.

# Discussion

In previous RAN4 meeting, the WF on Rel-15 NR RRM test case related issues was agreed in [2], and main conclusion for FR1+FR2 testability is as following:

## Criteria for selecting FR1/LTE+FR2 test with OTA testability problem - FR1/LTE+FR2 test has OTA testability problem if at least one of the following criteria is met: - Tests where any requirement is tested for FR1/LTE, - Tests where UE receives any DL message (e.g. RRC/DCI/MAC-CE configuration message/command etc) on FR1/LTE between the starting point and ending point of the test, and - Tests where UE transmits any UL signal (e.g. measurement report, ACK/NACK, CSI etc) b on FR1/LTE between the starting point and ending point of the test.

In the following test cases, the criteria for identifying testability problem is met:

- FR1+FR2 NR-DC to FR1+FR2 NR-DC: FR1 HO is tested and RRC command is received on FR1

- EN-DC with FR1 PSCell to EN-DC with FR2 PSCell: LTE HO is tested and RRC command is received on LTE

- EN-DC with FR2 PSCell to EN-DC with FR1 PSCell: LTE HO is tested and RRC command is received on LTE

- EN-DC with FR2 PSCell to EN-DC with FR2 PSCell: LTE HO is tested and RRC command is received on LTE

- NR-SA FR2 to EN-DC with FR1 PSCell: FR2 to LTE HO is tested as well as FR1 PSCell addition, and RACH on LTE

- NR-SA FR2 to EN-DC with FR2 PSCell: FR2 to LTE HO is tested as well as FR2 PSCell addition, and RACH on LTE

- NR-SA FR1 to EN-DC with FR2 PSCell: FR1 to LTE HO is tested as well as FR2 PSCell addition, and RACH on LTE

Thus, we think FR1+FR2 test cases for HO with PSCell shall be delayed until testability issues are solved, including:

- FR1+FR2 NR-DC to FR1+FR2 NR-DC

- EN-DC with FR1 PSCell to EN-DC with FR2 PSCell

- EN-DC with FR2 PSCell to EN-DC with FR1 PSCell

- EN-DC with FR2 PSCell to EN-DC with FR2 PSCell

- NR-SA FR2 to EN-DC with FR1 PSCell

- NR-SA FR2 to EN-DC with FR2 PSCell

- NR-SA FR1 to EN-DC with FR2 PSCell

Proposal 1:

FR1+FR2 test cases for HO with PSCell shall be delayed until testability issues are solved, including:

- FR1+FR2 NR-DC to FR1+FR2 NR-DC

- EN-DC with FR1 PSCell to EN-DC with FR2 PSCell

- EN-DC with FR2 PSCell to EN-DC with FR1 PSCell

- EN-DC with FR2 PSCell to EN-DC with FR2 PSCell

- NR-SA FR2 to EN-DC with FR1 PSCell

- NR-SA FR2 to EN-DC with FR2 PSCell

- NR-SA FR1 to EN-DC with FR2 PSCell

# Conclusion

In this contribution, we discuss the above remaining issues for HO with PSCell testing from last RAN4 meeting.

Proposal 1:

FR1+FR2 test cases for HO with PSCell shall be delayed until testability issues are solved, including:

- FR1+FR2 NR-DC to FR1+FR2 NR-DC

- EN-DC with FR1 PSCell to EN-DC with FR2 PSCell

- EN-DC with FR2 PSCell to EN-DC with FR1 PSCell

- EN-DC with FR2 PSCell to EN-DC with FR2 PSCell

- NR-SA FR2 to EN-DC with FR1 PSCell

- NR-SA FR2 to EN-DC with FR2 PSCell

- NR-SA FR1 to EN-DC with FR2 PSCell

# References

[1] R4-2210583, WF on further RRM enhancement for NR and MR-DC – HO with PSCell, vivo, RAN4 #103e.

[2] R4-2115240, WF on Rel-15 NR RRM test case related issues, Ericsson, RAN4 #100e