3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212035

Electronic Meeting, August 15 – August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to pre-MG TC1-5 for Intra-frequency measurement test with SA event triggered reporting tests: autonomous activation/deactivation of pre-MG triggered by MO addition/release in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Perf |  |  |  |  |  |  | Date: |  | 2022-08-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Pre-configured measurement gap (Pre-MG) was introduced for intra-frequency measurements and UE could autonomously activate/deactivate the pre-MG when receiving RRC reconfiguration, e.g. to add or remove measurement objects. The corresponding test case should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case for SSB based measurement with SA event triggered reporting with autonomous activation/deactivation of Pre-MG in FR1 triggered by MO addition/release is defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM requirements cannot be verified in the realistic environment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.6.6.X1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.533 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

A.6.6.X1.3	Intra-frequency measurement with SA event triggered reporting tests under non-DRX: with autonomous activation/deactivation of Pre-MG in FR1 triggered by MO addition/release

A.6.6.X3.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event with autonomous activation/deactivation of Pre-MG triggered by MO addition/release. This test will partly verify the intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

A.6.6.X3.1.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1) on NR RF channel 1, a FR1 neighbour cell (Cell 2) on the same frequency as the PCell (NR RF channel 1), and a FR1 neighbour cell (cell 3) on NR RF channel 2. The test parameters for PCell are given in Table A.6.6.X3.1.2-1 and A.6.6.X3.1.2-1 below.

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and

- UE is configured with BWP1, which contains the cell defining SSB. During the whole test, BWP1 is always scheduled as the active BWP the UE.

- UE shall not have any information of Cell 2 and Cell 3.

The TE schedules continuous DL data on PCell throughout the test.

The test consists of 4 successive time periods, with durations of T1, T2, T3 and T4, respectively.

The time period T1 starts when the measurement control information via RRC message, sent from the TE to the UE, is received at the UE side in PCell’s slot # denoted i. In the measurement control information, the following two measurement objects are configured.

- MO1 containing SSB from neighbour Cell 2, which completely contained in the active BWP and could be measured without gap,

- MO2 containing SSB from neighbour Cell 3, which is on a different frequency of the PCell and requires gap.

And it is indicated to the UE that event-triggered reporting with Event A3 is used for MO1. The pre-MG is supposed to be activated during T2.

The time period T3 starts when the measurement control information via RRC message, sent from the TE to the UE, is received at the UE side in PCell’s slot # denoted j. In the measurement control information, the measurement object MO2 containing SSB from neighbour Cell 3 is removed. Then UE is only required to measure the MO1 during T4, the pre-MG is supposed to be deactivated.

Table A.6.6.X3.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.X3.1.2-2: General test parameters for SA intra-frequency event triggered reporting with autonomous activation/deactivation of pre-MG in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 and Cell 3 | Cell 2 is to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
|  |  |  | 2: Cell 3 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE |  |
| Gap pattern ID |  | 1, 2, 3 | 0 |  |
| Measurement gap repetition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 and Cell 3 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 1 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| T3 | s | 1, 2, 3 | 1 |  |
| T4 | s | 1, 2, 3 | 5 |  |

Table A.6.6.X3.1.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with autonomous activation/deactivation of pre-MG in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| TDD configuration |  | 1 | N/A |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 2 | CCR.1.2 TDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  |  | OP.1 |  |  |  | OP.1 |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  |  | N/A |  |  |  | N/A |  |  |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |  |  | N/A |  |  |  | N/A |  |  |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |  |  | N/A |  |  |  | N/A |  |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  | SSB |  |  |  | SSB |  |  |  |
| SSB parameters |  | 1 | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |  |
| SMTC configuration defined in A.3.11 |  | 1 | SMTC.2 |  |  |  | SMTC.2 |  |  |  | SMTC.2 |  |  |  |
|  |  | 2 | SMTC.1 |  |  |  | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
|  |  | 3 | SMTC.1 |  |  |  | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | 4 | -1.46 | -Inf | -1.46 | -Inf | -1.46 | -Inf | 7 | -Inf | -Inf |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 4 | 4 | 4 | 4 | -Inf | 4 | -Inf | 4 | -Inf | 7 | -Inf | -Inf |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -94 | -94 | -Inf | -94 | -Inf | -94 | -Inf | -91 | -Inf | -Inf |
|  |  | 2 | -94 | -94 | -94 | -94 | -Inf | -94 | -Inf | -94 | -Inf | -91 | -Inf | -Inf |
|  |  | 3 | -91 | -91 | -91 | -91 | -Inf | -92 | -Inf | -92 | -Inf | -88 | -Inf | -Inf |
| Io | dBm/9.36 MHz | 1 | -64.6 | -64.6 | -64.6 | -62.3 | -64.6 | -62.25 | -64.6 | -62.25 | -70.5 | -62.2 | -70.5 | -70.5 |
|  | dBm/9.36 MHz | 2 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 | -64.60 | -62.25 | -70.5 | -62.2 | -70.5 | -70.5 |
|  | dBm/38.16 MHz | 3 | -58.50 | -58.50 | -58.50 | -56.16 | -58.50 | -56.16 | -58.50 | -56.16 | -63.94 | -56.15 | -63.94 | -63.94 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.6.6.X3.1.3	Test Requirements

During T1 and T2, the UE shall not report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions, starting from the 1st complete Pre-MG occasion after the beginning of PCell’s DL slot (i+TRRCProccesingDelay) + [5]ms as defined in clause 8.19.4.

During T3 and T4, the UE shall be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions, starting from the 1st complete Pre-MG occasion after the beginning of PCell’s DL slot (j+ TRRCProccesingDelay) + [5]ms as defined in clause 8.19.4.

The UE shall send one Event A3 triggered measurement report containing the measurement results for neighbour Cell 2, with a measurement reporting delay less than 800 ms from the beginning of time period T2.

The UE shall send one Event A3 triggered measurement report containing the measurement results for neighbour Cell 2, with a measurement reporting delay less than 200 ms from the beginning of time period T4.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

# < End of change 1>