3GPP TSG RAN WG4 #104-e														     R4-2212115

e-Meeting, August 15th – 26th, 2022

Agenda Item:		13.1.1

Source:				InterDigital Communications

Title:						On Multiple TA for multi-TRP MRTD MTTD limits

Document for:		Approval

# Background

RAN plenary #94e approved the WID in [1] for Rel-18 MIMO enhancements. As described in WID, one of the goals in Objective 7 is to study and specify the operation related to multi-DCI multi-TRP deployment allowing for two timing advance TAs loops. In this context, for the case of simultaneous UL transmissions, the operation is limited to the description of Objective 6 in WID.  RAN1 initiated discussions in the last meeting led to an LS to RAN4 in [2] regarding to the MTTD limits that RAN1 shall further consider for this feature design.

We are listing below for convenience the objectives 6 and 7 as they are part of the following discussion:

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

Additionally, we are listing below the content of the LS for convenience as it is the main subject of this contribution:

In Rel-18 discussions on MIMO Evolution for Downlink and Uplink, RAN1 has agreed to support multi-DCI multi-TRP operation with two TAs. With regards to the maximum uplink timing difference between the two TAs for multi-DCI multi-TRP operation, RAN1 made the following conclusion in RAN1#109-e: Conclusion: For multi-DCI multi-TRP operation with two TAs, the decision on the maximum uplink timing difference is left up to RAN4. - send an LS to RAN4 asking them the maximum uplink timing difference RAN1 can assume between the two TAs for multi-DCI multi-TRP operation. RAN1 would kindly like to ask RAN4 to provide feedback on what maximum uplink timing difference that RAN1 can assume between the two TAs for multi-DCI multi-TRP operation.

# Discussion

In this contribution, we share our analysis and propose an answer for the LS reply to RAN1.

# Multi-TRP deployment scenarios analysis

In Rel-17, for both intra and inter-cell, multi-TRP operations were limited to within a CP reception. That means, no major UE timing adjustments in downlink were expected. For convenience we are listing here a table with the CP and symbol length for different SCS options:

| Parameter / Numerology (µ) | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier Spacing (KHz) | 15 | 30 | 60 | 120 | 240 |
| OFDM Symbol Duration (us) | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 |
| Cyclic Prefix Duration (us) | 4.69 | 2.34 | 1.17 | 0.57 | 0.29 |
| OFDM Symbol including CP (us) | 71.35 | 35.68 | 17.84 | 8.92 | 4.46 |

Table 1. Symbol and CP length for different numerologies

As we can see in Table 1, for 120KHz and 240KHz SCS, since CP size is quite small, employing two independent TA values in FR2 is required for supporting a viable multi-TRP operation.

Currently, for non-collocated transmission points, RAN4 specifications (38.133) defines some time restrictions in terms of Maximum Receive Time Difference (MRTD) and Maximum Transmit Time Difference (MTTD). These limits are imposed due to the different UE capabilities in terms of baseband processing and/or network synchronization cases.

For example, for NR inter-band CA within FR2, the maximum receive time difference is 8us. This corresponds roughly a bit less that a full symbol for 120KHz or two symbols for 240KHz numerologies.

Observation 1:  For NR inter-band CA within FR2 the maximum receive time difference defined in 38.133 specification is 8us.

Observation 2: The baseband and time processing may impose limits of the Maximum Receive Time Difference and Maximum Transmit Time Difference for the multi-TRP scenario with 2 TAs scenario.

Defining the deployment conditions/scenarios can be a first step into this study considering the applicable frequency range. Considering all uncertainties and the non-collocated TRPs scenario, it needs to be checked whether an 8us receive time difference would be sufficient.

We believe that maintaining a certain coherence between multi-TRP deployments and CA case would create synergies that would allow an easier UE implementation, and thus we propose to adopt some common deployment scenarios requirements in terms of difference timing limitations.

The requirement for MTTD (between different TAGs) for NR CA within FR2 is specified in 38.133 subclause 7.5.4, Table 7.5.4-1, as described below:

Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 Note1 |
| Between FR1 and FR2-1 | 26.1 |
| Between FR1 and FR2-2 | TBD |
| Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

Proposal 1: For m-TRP with two TAs Maximum Transmit Time Difference limits adopt NR inter-band CA cases as baseline.

According to the above table the MTTD within FR1 is 34.6us while within FR2-1 is 8.5us.

Proposal 2: Adopt for intra-cell m-TRP scenario with two TAs the MTTD within FR1 as 34.6us and within FR2-1 as 8.5us.

In conclusion, we suggest that the LS reply is aligned with the suggested values in proposal 2.

Proposal 3: The LS reply mention that for intra-cell m-TRP scenario with two TAs RAN4 agreed that the MTTD within FR1 is 34.6us and within FR2-1 is 8.5us like the current NR inter-band UL CA timing requirements.

As the MTTD requirements are tightly connected to the MRTD, we suggest adopting the corresponding MRTD as well and eventually convey this information to RAN1 as well.

Here is the corresponding RRM table for convenience:

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 note1 |
| Between FR1 and FR2-1 | 25 |
| Between FR1 and FR2-2 | TBD |
| Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

Proposal 4: Adopt for intra-cell m-TRP scenario with two TAs the MRTD within FR1 as 33us and within FR2-1 as 8us and covey this information to RAN1 in the LS reply.

# Conclusions

This contribution discussed the MTTD limits for intra-cell M-TRP operation with two TAs loops. Based on the presented discussion, we made the following observations and proposals:

Observation 1:  For NR-DC synchronous case or NR inter-band CA within FR2 the maximum receive time difference defined in 38.133 specification is 8us.

Observation 2: The baseband and time processing may impose limits of the Maximum Receive Time Difference and Maximum Transmit Time Difference for the multi-TRP scenario with 2 TAs scenario.

Proposal 1: For m-TRP with two TAs Maximum Transmit Time Difference limits adopt NR inter-band CA cases as baseline.

Proposal 2: Adopt for intra-cell m-TRP scenario with two TAs the MTTD within FR1 as 34.6us and within FR2-1 as 8.5us.

Proposal 3: The LS reply mention that for intra-cell m-TRP scenario with two TAs RAN4 agreed that the MTTD within FR1 is 34.6us and within FR2-1 is 8.5us like the current NR inter-band UL CA timing requirements.

Proposal 4: Adopt for intra-cell m-TRP scenario with two TAs the MRTD within FR1 as 33us and within FR2-1 as 8us and covey this information to RAN1 in the LS reply.

# References

- RP-213598, “New WID: MIMO evolution for Downlink and Uplink”, Samsung, 3GPP RAN Meeting #94e, December 6-17, 2021

- R4-2211512 “LS on maximum uplink timing difference for Multi-DCI Multi-TRP with two TAs “, 3GPP RAN Meeting #109-e, May 2022

- 3GPP 38.133, NR; Requirements for support of radio resource management, Rel-17, 2021