3GPP TSG-RAN4 Meeting #104-e	R4-2212262

# Online meeting, 15 August – 26 August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | 0014 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.106: NR repeater ACRR requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For ACRR requirements, nominal channel bandwidth of min(100 MHz, BWpassband) for FR1 and min(400 MHz, BWpassband) for FR2 are assumed. It is also assumed that filter bandwidth equals to the transmission bandwidth configuration corresponding to the nominal cahnnel bandwidth. However, if min(100 MHz, BWpassband) or min(400 MHz, BWpassband) is not defined for the BS channel bandwidth for the operating band, filter bandwidth for the adjacent channel frequency cannot be decided and ACRR requirements are not clear.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | if min(100 MHz, BWpassband) or min(400 MHz, BWpassband) is not defined for the BS channel bandwidth for the operating band, adopt he widest channel bandwidth for the operating band which is narrower than BWpassband. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ACRR requirements are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.1, 6.9.2, 7.8.1, 7.8.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------- start of change -----------

6.9	Adjacent Channel Rejection Ratio (ACRR)

### 6.9.1	General

Adjacent Channel Rejection Ratio (ACRR) is the ratio of the average gain over a carrier of the repeater in the passband to the average gain of the repeater over an adjacent channel outside the repeater passband. The carrier in the passband and in the adjacent channel shall be of the same type (reference carrier) and both are assumed to have a bandwidth of min{100 MHz, passband BW}. If this bandwidth is not defined for BS channel bandwidth for the operating band, bandwidths are assumed to be equal to the widest channel bandwidth for the operating band which is narrower than passband BW.

The requirement shall apply to the uplink and downlink of the Repeater, where the donor link is maintained via antennas (wireless Repeater).

The requirement is differentiated between uplink and downlink.

The requirement shall apply during the transmitter ON period.

### 6.9.2	Minimum Requirements

For a repeater operating at passband below 2496 MHz, the ACRR requirements in table 6.9.2.1-1 shall apply in downlink. In normal conditions the ACRR for downlink shall be higher than the value specified in the Table 6.9.2.1-1.

Table 6.9.2.1-1: Repeater Downlink ACRR below 2496MHz

| Co-existence with other systems | Repeater Class | Channel offset from frequency edge of passband (MHz) | ACRR limit |
| --- | --- | --- | --- |
| UTRA, E-UTRA, NR | Wide Area repeater | min{100 MHz, passband BW}/2 (Note 2) | 45 |
|  | Medium Range repeater | min{100 MHz, passband BW}/2 (Note 2) | 45 |
|  | Local Area repeater | min{100 MHz, passband BW}/2 (Note 2) | 33 (Note 1) |
| NOTE 1: This requirement does not applicable if the passband occupies the entire operating band. NOTE 2: If min{100 MHz, passband BW} is not defined for BS channel bandwidth for the operating band, {the widest channel bandwidth for the operating band which is narrower than passband BW}/2 shall be used. |  |  |  |

For a repeater operating at passband above 2496 MHz, the ACRR requirements in table 6.9.2.1-1a shall apply in downlink. In normal conditions the ACRR for downlink shall be higher than the value specified in the Table 6.9.2.1-1a.

Table 6.9.2.1-1a: Repeater Downlink ACRR above 2496 MHz

| Co-existence with other systems | Repeater Class | Channel offset from frequency edge of passband (MHz) | ACRR limit |
| --- | --- | --- | --- |
| UTRA, E-UTRA, NR | Wide Area repeater | min{100 MHz, passband BW}/2 (Note 2) | 33dB |
|  | Medium Range repeater | min{100 MHz, passband BW}/2 (Note 2) | 33dB |
|  | Local Area repeater | min{100 MHz, passband BW}/2 (Note 2) | [33dB] (Note 1) |
| NOTE 1: This requirement does not applicable if the passband occupies the entire operating band. NOTE 2: If min{100 MHz, passband BW} is not defined for BS channel bandwidth for the operating band, {the widest channel bandwidth for the operating band which is narrower than passband BW}/2 shall be used. |  |  |  |

For a repeater operating at passband below 2496 MHz, the ACRR requirements in table 6.9.2.1-2 shall apply in uplink. In normal conditions the ACRR for uplink shall be higher than the value specified in the Table 6.9.2.1-2.

Table 6.9.2.1-2: Repeater Uplink ACRR below 2496 MHz

| Co-existence with other systems | Repeater Class | Channel offset from frequency edge of passband (MHz) | ACRR limit |
| --- | --- | --- | --- |
| UTRA, E-UTRA, NR | Wide Area repeater | min{100 MHz, passband BW}/2 (Note 2) | 33dB |
|  | Local Area repeater | min{100 MHz, passband BW}/2 (Note 2) | 33dB (Note 1) |
| NOTE 1: This requirement does not applicable if the passband occupies the entire operating band. NOTE 2: If min{100 MHz, passband BW} is not defined for BS channel bandwidth for the operating band, {the widest channel bandwidth for the operating band which is narrower than passband BW}/2 shall be used. |  |  |  |

For a repeater operating at passband above 2496 MHz, the ACRR requirements in table 6.9.2.1-2a shall apply in uplink. In normal conditions the ACRR for uplink shall be higher than the value specified in the Table 6.9.2.1-2a.

Table 6.9.2.1-2a: Repeater Uplink ACRR above 2496 MHz

| Co-existence with other systems | Repeater Class | Channel offset from frequency edge of passband (MHz) | ACRR limit |
| --- | --- | --- | --- |
| UTRA, E-UTRA, NR | Wide Area repeater | min{100 MHz, passband BW}/2 (Note 3) | 33dB |
|  | Local Area repeater | 5MHz | 20dBc (Note 1, Note 2) |
|  |  | min{100 MHz, passband BW}/2 (Note 3) | 33dBc (Note 1) |
| NOTE 1: This requirement does not applicable if the passband occupies the entire operating band. NOTE 2: In this case, the channel within the passband and the adjacent channel are assumed to have a bandwidth of 10 MHz NOTE 3: If min{100 MHz, passband BW} is not defined for BS channel bandwidth for the operating band, {the widest channel bandwidth for the operating band which is narrower than passband BW}/2 shall be used. |  |  |  |

--------- next change -----------

7.8	OTA Adjacent Channel Rejection Ratio (ACRR)

7.8.1	General

OTA Adjacent Channel Rejection Ratio (ACRR) is the ratio of the average gain over a carrier of the repeater in the passband to the average gain of the repeater over an adjacent channel outside the repeater passband. The requirement shall apply to the uplink and downlink of the Repeater. The bandwidth of the channel inside the passband and the adjacent channel are assumed to be minimum {400MHz, passband BW}. If this bandwidth is not defined for BS channel bandwidth for the operating band, bandwidths are assumed to be equal to the widest channel bandwidth for the operating band which is narrower than passband BW.

The requirement is differentiated between downlink and uplink.

The requirement shall apply during the transmitter ON period.

#### 7.8.1.1	Minimum Requirements

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal in the passband and a received signal on an adjacent channel outside repeater passband is from the same direction and are the same as the TX reference direction for the opposite DL/UL setting.

For a repeater operating at passband operating in FR2, the ACRR requirements in table 7.8.1.1-1 shall apply in downlink. In normal conditions the ACRR for downlink shall be higher than the value specified in the Table 7.8.1.1-1.

Table 7.8.1.1-1: Repeater Downlink ACRR

| Co-existence with other systems | Repeater Class | Channel offset from frequency edge of passband (MHz) | ACRR limit |
| --- | --- | --- | --- |
| NR | Wide Area repeater | minimum {400MHz, passband BW}/2 (Note 4) | 28 (Note 2) 26 (Note 3) |
|  | Medium Range repeater | minimum {400MHz, passband BW}/2 (Note 4) | 28 (Note 2) 26 (Note 3) |
|  | Local Area repeater | minimum {400MHz, passband BW}/2 (Note 4) | 28 (Note 1, 2) 26 (Note 1, 3) |
| NOTE 1: This requirement does not applicable if the passband occupies the entire operating band. NOTE 2: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 3: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz NOTE 4: If min{400 MHz, passband BW} is not defined for BS channel bandwidth for the operating band, {the widest channel bandwidth for the operating band which is narrower than passband BW}/2 shall be used. |  |  |  |

For a repeater operating at passband operating in FR2, the ACRR requirements in table 7.8.1.1-2 shall apply in uplink. In normal conditions the ACRR for uplink shall be higher than the value specified in the Table 7.8.1.1-2.

Table 7.8.1.1-2: Repeater Uplink ACRR

| Co-existence with other systems | Repeater Class | Channel offset from frequency edge of passband (MHz) | ACRR limit |
| --- | --- | --- | --- |
| NR | Wide Area repeater | minimum {400MHz, passband BW}/2 (Note 4) | 28 (Note 2) 26 (Note 3) |
|  | Local Area repeater | minimum {400MHz, passband BW}/2 (Note 4) | 17 (Note 1, 2) 16 (Note 1, 3) |
| NOTE 1: This requirement does not applicable if the passband occupies the entire operating band. NOTE 2: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 3: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz NOTE 4: If min{400 MHz, passband BW} is not defined for BS channel bandwidth for the operating band, {the widest channel bandwidth for the operating band which is narrower than passband BW}/2 shall be used. |  |  |  |

--------- end of change -----------