3GPP TSG-RAN WG4 Meeting # 104-e      	R4-2212299

Electronic Meeting, Aug 15 – Aug 26, 2022

Agenda Item:	9.12.1

Source: 	CMCC

Title: 			 Discussion on RLM/BFD relaxation for NR power saving enhancement

Document for:	Discussion

- Introduction

After RAN4#103-e meeting, a way forward was approved in [1], and a remaining issue of low mobility criteria evaluation still needs FFS. In this contribution, we provide our views on the low mobility criteria.

- Discussion

Issue 1-8: Clarifications for Low mobility criteria evaluation

For NR-DC, the following proposals can be discussed in R4#104-e:

- Proposal 1-1: For NR-DC, UE may also evaluate low mobility criteria in PSCell if network configures, and apply the evaluated low mobility state in the corresponding cell group.

- Proposal 1-2:

  - In NR-DC, the UE configured with low mobility criteria evaluates the low mobility criterion only on SpCell.

  - In NR-DC, if the relaxed measurement criterion (low mobility criteria) is met on a SpCell then the UE assumes that the relaxation criterion is met on all serving cells in the CG of that SpCell

- Proposal 1-3: keep the previous agreement that evaluate low mobility criteria on PCell.

In the previous meetings, we have achieved several agreements about the low mobility criterion.

First, in RAN4#102-e meeting, RAN4 agreed that the low mobility criterion is evaluated on the NR PCell for the case of NR-DC [2].

- The low mobility criteria is evaluated on the - NR PCell for the case of NR single carrier, NR CA, NE-DC - NR PSCell for the case of EN-DC - NR PCell for the case of NR-DC - the low mobility criteria is evaluated on SpCell (s), regardless whether BFD is configured in Scell or not

Following the above agreement, RAN2 approved the related signaling lowMobilityEvaluationConnected. The description in TS38.331 clarifies that low mobility criterion is configured in NR PCell for the case of NR-DC [3].

lowMobilityEvaluationConnected Indicates the criterion for a UE to detect low mobility in RRC_CONNECTED in an SpCell. The s-SearchDeltaP-Connected is the parameter "SSearchDeltaP-connected". Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on. The t-SearchDeltaP-Connected is the parameter "TSearchDeltaP-Connected". Value s5 means 5 seconds, value s10 means 10 seconds and so on. Low mobility criterion is configured in NR PCell for the case of NR SA/ NR CA/ NE-DC/NR-DC, and in the NR PSCell for the case of EN-DC.

Besides, in RAN4#101-bis-e meeting, RAN4 have agreed that low mobility criterion is configured on per-UE basis [4].

- Low mobility criterion is configured on per-UE basis.

From the above, in NR-DC scenario, the low mobility criterion is configured and evaluated on the PCell. If the low mobility criterion is met on the PCell, then the UE assumes that the low mobility criteria is met on all serving cells in both MCG and SCG.

Observation 1: Following the previous agreements, the low mobility criterion should be configured and evaluated on the PCell. If the low mobility criterion is met on the PCell, UE could apply the evaluated low mobility state in both MCG and SCG.

In the last meeting, some companies supported that low mobility criterion could be also configured and evaluated on the PSCell. We are fine with this proposal. However, we think the UE behavior under different configuration should be clarified.

If the low mobility criterion is configured only on the PCell, then UE should evaluate the low mobility criterion on the PCell, and apply the evaluated low mobility state in both MCG and SCG.

If the low mobility criterion is configured on the PCell and PSCell, then UE should evaluate the low mobility criterion on the PCell and PSCell respectively, and apply the evaluated low mobility state in the corresponding cell group.

Proposal 1: We are fine with network could also configure the low mobility criteria in PSCell, UE behavior under different configuration should be:

- If the low mobility criterion is configured only on the PCell

  - UE should evaluate the low mobility criterion on the PCell, and apply the evaluated low mobility state in both MCG and SCG.

- If the low mobility criterion is configured on the PCell and PSCell

  - UE should evaluate the low mobility criterion on the PCell and PSCell respectively, and apply the evaluated low mobility state in the corresponding cell group.

- Conclusion

In this contribution, we discuss the remaining issue about low mobility criteria. The proposals are:

Observation 1: Following the previous agreements, the low mobility criterion should be configured and evaluated on the PCell. If the low mobility criterion is met on the PCell, then the UE assumes that the low mobility criteria is met on all serving cells in both MCG and SCG.

Proposal 1: We are fine with network could also configure the low mobility criteria in PSCell. However, UE behavior under different configuration should be:

- If the low mobility criterion is configured only on the PCell

  - UE should evaluate the low mobility criterion on the PCell, and apply the evaluated low mobility state in both MCG and SCG.

- If the low mobility criterion is configured on the PCell and PSCell

  - UE should evaluate the low mobility criterion on the PCell and PSCell respectively, and apply the evaluated low mobility state in the corresponding cell group.

- Reference

[1] R4-2210613, WF on RLM/BFD relaxation for UE Power Saving enhancements, MediaTek.

[2] R4-2206909, WF on RLM/BFD relaxation for UE Power Saving enhancements, MediaTek.

[3] 3GPP TS 38.133: "NR, Radio Resource Control (RRC) protocol specification".

[4] R4-2202640, WF on RLM/BFD relaxation for UE Power Saving enhancements, MediaTek.