3GPP RAN WG4 Meeting #104-e		R4-2212355

Online, August 15th – 26th, 2022

Agenda item:	9.4.4

Source:	Apple

Title:	Signaling enhancement for FR2 new CA BW classes

WI/SI:	NR_RF_FR2_req_enh2-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

In the need of supporting more than 1GHz contiguous spectrum in certain FR2 band which has been made available in US [1], in RAN #91-e meeting, an objective of introducing new FR2 CA BW classes and the related Rx requirements to support contiguous downlink aggregated channel BW up to 1600 MHz was approved to be included in the Rel-17 WID on further enhancements of NR RF requirements for frequency range 2 [2]. In RAN4 #99-e meeting, it was first agreed to introduce 4 new CA BW classes R, S, T, and U in fallback group 2 (FBG2) to support aggregated channel BW up to 1600 MHz [3]. However, in consideration of the backward compatibility with the existing FR2 cells at 100 MHz in fallback group 3 (FBG3), in past few RAN4 meetings, there have been several variants of new FR2 CA BW classes proposed based on the combinations of 100MHz cells and 200MHz cells to support aggregated channel BW up to 1600 MHz or more [4-8], among which the proposal for introducing 11 new CA BW classes (R2 – R12) in the new fallback group 5 (FBG5) was agreed in last RAN4 meeting [9]. The newly introduced CA BW classes in FBG5 though with the merits of providing the most flexibility in combining the 100MHz and 200MHz cells as well as having the prescience for future aggregated BW extension up to 2400 MHz, it also has a potential issue when UE’s maximum aggregated BW is less than the CA BW class upper BW limit where UE’s full capability in the fallback BW classes can only be indicated by the rather complicated FeatureSet signalling, and that would result in substantial signalling overhead, as has been elaborated in [10]. In this contribution, we discuss the signalling aspect on how UE’s full capability can be indicated to the network for the new FR2 CA BW classes in FBG5 and propose to introduce a new IE to indicate UE’s maximum aggregated BW capability to avoid the complicated FeatureSet signalling.

# Discussion

The virtue of the CA BW classes and the associated fallback rule lies in that UE only needs to signal the highest order CA BW class to indicate its capability for supporting intra-band contiguous CA and all the lower order CA BW classes are expected to be supported without additional signaling, in a way to minimize the signaling overhead. The CA BW class contains two key parameters, number of component carriers (CCs) and aggregated BW range. The number of CCs is associated with UE’s capability on processing the number of carriers simultaneously. The aggregated BW upper limit is normally determined by the number of CCs multiplying the maximum supported BW per carrier assuming all carriers have the same maximum supported BW. Nevertheless, UE’s capability on maximum aggregated BW support can be less than its supported number of CCs multiplying the maximum supported BW per carrier, such as limited by its overall digital processing capacity. In that case, for the traditional CA BW classes where all the cells in the same fallback group have the same maximum BW capability, UE may just signal the next lower order CA BW class to align with its maximum aggregated BW capability. However, for the newly introduced FR2 CA BW classes in FBG5 as shown in Table 2-1, where the aggregated BW can be combined with a mixture of 100MHz cells and 200MHz cells, the signaling of UE’s maximum aggregated BW capability which is less than the CA BW class upper limit to the network could be rather complicated as UE is assumed to have no prior knowledge on how the CA would be configured by the network and UE may have to signal multiple FeatureSets in order to cover all the possible network configurations. Such signaling complexity is further exemplified below.

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 |  |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| R4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| R5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| R6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| R7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 |  |
| R8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| R9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| R10 | 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| R11 | 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| R12 | 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3, 4 and 5 are 400 MHz, 200 MHz, 100 MHz, 100 MHz and 200 MHz respectively except for CA bandwidth class A. For CA bandwidth classes of fallback group 5, requirements apply for non-interlaced 100 MHz and 200 MHz channel bandwidths (each CA bandwidth class consisting of up to two contiguous sub-blocks each with component carriers of a single channel bandwidth). NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. NOTE 3: In this release of the specification, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz (this note is not relevant for UE capability parsing by the network). |  |  |  |

Table 2-1 The newly introduced FR2 CA BW classes R2 – R12 in FBG5 as highlighted

For example, if a UE can support 12 CCs and each CC’s maximum BW is 200 MHz, whereas its maximum aggregated BW is only 1200 MHz, how would the UE report its capability to the network based on the current signaling design? One possibility is to only signal BW class “R6” as its BW upper range aligns with UE’s maximum aggregated BW capability. However, the 1200MHz on the network side may only be configured with either one of the combinations below,

- 2x100MHz + 5x200MHz (7 CCs)

- 4x100MHz + 4x200MHz (8 CCs)

- 6x100MHz + 3x200MHz (9 CCs)

- 8x100MHz + 2x200MHz (10 CCs)

- 10x100MHz + 1x200MHz (11 CCs)

- 12x100MHz (12 CCs)

As a result, 1200MHz could not be configured to the UE which only signals “R6” despite UE does have the capability to support all other configurations. In order to cover all possible 1200MHz network configurations, the UE would have to signal all above FeatureSets in addition to BW class “R12” and that would seemingly defeat the merit of CA BW classes and its fallback rule. On the other hand, considering that the intended FR2 band supporting FBG5 may be aggregated with FR1 bands where its maximum aggregated BW could also vary depending on FR1 bands’ aggregated BW, the signalling complexity and overhead could be rather overwhelming.

The issue with existing signalling design using FeatureSet is that UE’s maximum aggregated BW capability is only represented by a particular carrier BW configuration under a CA BW class. The UE’s true capability on each supported CC cannot be fully indicated by a single FeatureSet. For instance, in the above example if UE signals R12 and only the FeatureSet of 12x100MHz, the network could misinterpret that UE can only support maximum 100MHz in each of the 12 CCs. But UE’s capability for each CC can actually be 200 MHz.

To mitigate this issue, a new IE maxaggregatedBW can be introduced to indicate UE’s maximum aggregated BW capability which would be independent from UE’s per CC BW capability. With the availability of the new IE, there can be three potential signalling solutions:

Alternative 1

The FeatureSet under the highest order CA BW class supported is used to indicate each CC’s true maximum BW capability. Using the same example above, UE only needs to signal the following parameters to the network,

CA BW Class: R12

maxaggregatedBW = 1200 MHz

{CC1, CC2, CC3, CC4, CC5, CC6, CC7, CC8, CC9, CC10, CC11, CC12} =

{200, 200, 200, 200, 200, 200, 200, 200, 200, 200, 200, 200} (FeatureSet)

Based on this information, the network should understand that the UE can support any combinations of 100MHz and 200MHz CCs for the CA provided that the total aggregated BW is equal to or less than 1200 MHz.

The concern with this signalling approach is that the original definition of FeatureSet would be altered as it is used to indicate the envelope capability of the combination, not the per CC capability when not used in a combination. It is unsure whether there would be a backward compatibility issue.

Alternative 2

A simpler solution is that UE only needs to signal CA BW class and maxaggregatedBW to the network without the FeatureSet indication. The default assumption would be that the maxaggregatedBW is also supported by the UE for all the lower order CA BW classes where their aggregated BW upper limit is higher than or equal to maxaggregatedBW. For example, if UE signals R12 with maxaggregatedBW = 1600 MHz, the network should understand that the UE can support the following configurations as the highest envelope in each CA BW class:

R12: 8x100MHz + 4x200MHz (follow maxaggregatedBW)

R11: 6x100MHz + 5x200MHz (follow maxaggregatedBW)

R10: 4x100MHz + 6x200MHz (follow maxaggregatedBW)

R9: 2x100MHz + 7x200MHz (follow maxaggregatedBW)

R8: 8x200MHz (follow maxaggregatedBW and CA BW class)

R7: 7x200MHz (follow CA BW class)

R6: 6x200MHz (follow CA BW class)

R5: 5x200MHz (follow CA BW class)

R4: 4x200MHz (follow CA BW class)

R3: 3x200MHz (follow CA BW class)

R2: 2x200MHz (follow CA BW class)

On the other hand, if a UE does not follow the default assumption, that is, its maximum aggregated BW for lower order CA BW classes is less than maxaggregatedBW, the FeatureSet can still be used to indicate its highest envelope in each CA BW class.

Alternative 3

The alternative 3 solution is similar to Alternative 2, but without the default assumption that the maxaggregatedBW is also supported by the UE for all the lower order CA BW classes where their aggregated BW upper limit is higher than or equal to maxaggregatedBW. The UE would rely on the additional signalling for CA BW class in FBG2 to indicate the maximum number of 200MHz CCs supported. For example, for a UE supports R12 with maxaggregatedBW = 1600 MHz, it can further signal the support of CA BW class “U” in FBG2 to indicate that out of the 12 supported CCs, at least 8 individual CC can support 200MHz. As a result, network should recognize that the UE can support the following configurations as the highest envelope in each CA BW class:

R12: 8x100MHz + 4x200MHz (follow maxaggregatedBW)

R11: 6x100MHz + 5x200MHz (follow maxaggregatedBW)

R10: 4x100MHz + 6x200MHz (follow maxaggregatedBW)

R9: 2x100MHz + 7x200MHz (follow maxaggregatedBW)

R8: 8x200MHz (follow maxaggregatedBW, CA BW class R8 and U)

R7: 7x200MHz (follow CA BW class R7 and T)

R6: 6x200MHz (follow CA BW class R6 and S)

R5: 5x200MHz (follow CA BW class R5 and R)

R4: 4x200MHz (follow CA BW class R4 and F)

R3: 3x200MHz (follow CA BW class R3 and E)

R2: 2x200MHz (follow CA BW class R2 and D)

Another example is that if UE supports R12 with maxaggregatedBW = 1200 MHz, it can further signal the support of CA BW class “S” to indicate that out of the 12 supported CCs, at least 6 individual CC can support 200MHz. As a result, network should recognize that the UE can support the following configurations as the highest envelope in each CA BW class:

R12: 12x100MHz (follow maxaggregatedBW)

R11: 10x100MHz + 1x200MHz (follow maxaggregatedBW)

R10: 8x100MHz + 2x200MHz (follow maxaggregatedBW)

R9: 6x100MHz + 3x200MHz (follow maxaggregatedBW)

R8: 4x200MHz + 4x200MHz (follow maxaggregatedBW)

R7: 2x200MHz + 5x200MHz (follow maxaggregatedBW)

R6: 6x200MHz (follow maxaggregatedBW, CA BW class R6 and S)

R5: 5x200MHz (follow CA BW class R5 and R)

R4: 4x200MHz (follow CA BW class R4 and F)

R3: 3x200MHz (follow CA BW class R3 and E)

R2: 2x200MHz (follow CA BW class R2 and D)

The Alternative 3 would require CA BW classes R, S, T, and U to be retained, despite they can be covered by R5, R6, R7, and R8 respectively. In that case the newly introduced CA BW classes in FBG5 may need to be renamed from R2 – R12 to V2 – V12 to avoid using the same letter “R” with CA BW class R.

In summary, to avoid the complicated FeatureSet signalling and the signalling overhead to indicate UE’s full capability in the fallback BW classes when UE’s maximum aggregated BW is less than the CA BW class upper BW limit, it is proposed to introduce a new IE maxaggregatedBW to indicate UE’s maximum aggregated BW capability which would be independent from UE’s per CC BW capability. The signalling solution can be down-selected from the aforementioned three alternatives.

Proposal 1: Introduce a new IE maxaggregatedBW to indicate UE’s maximum aggregated BW capability when UE’s maximum aggregated BW is less than the CA BW class upper BW limit.

Proposal 2: The signalling solution is down-selected from the following three alternatives (using R12 in FBG5 and maximum aggregated BW = 1600 MHz as an example):

Alternative 1: The following parameters are signalled,

CA BW Class: R12

maxaggregatedBW = 1600 MHz

{CC1, CC2, CC3, CC4, CC5, CC6, CC7, CC8, CC9, CC10, CC11, CC12} =

{200, 200, 200, 200, 200, 200, 200, 200, 200, 200, 200, 200} (FeatureSet)

Alternative 2: The following parameters are signalled,

CA BW Class: R12

maxaggregatedBW = 1600 MHz

Note: The default assumption is that the maxaggregatedBW is also supported by the UE for all the lower order CA BW classes where their aggregated BW upper limit is higher than or equal to maxaggregatedBW.

Alternative 3: The following parameters are signalled,

CA BW Class: R12

maxaggregatedBW = 1600 MHz

CA BW Class: U

# 3	Conclusion

In this contribution, we discuss the signalling aspect on how UE’s full capability can be indicated to the network for the new FR2 CA BW classes in FBG5 and propose to introduce a new IE to indicate UE’s maximum aggregated BW capability to avoid the complicated FeatureSet signalling.

Proposal 1: Introduce a new IE maxaggregatedBW to indicate UE’s maximum aggregated BW capability when UE’s maximum aggregated BW is less than the CA BW class upper BW limit.

Proposal 2: The signalling solution is down-selected from the following three alternatives (using R12 in FBG5 and maximum aggregated BW = 1600 MHz as an example):

Alternative 1: The following parameters are signalled,

CA BW Class: R12

maxaggregatedBW = 1600 MHz

{CC1, CC2, CC3, CC4, CC5, CC6, CC7, CC8, CC9, CC10, CC11, CC12} =

{200, 200, 200, 200, 200, 200, 200, 200, 200, 200, 200, 200} (FeatureSet)

Alternative 2: The following parameters are signalled,

CA BW Class: R12

maxaggregatedBW = 1600 MHz

Note: The default assumption is that the maxaggregatedBW is also supported by the UE for all the lower order CA BW classes where their aggregated BW upper limit is higher than or equal to maxaggregatedBW.

Alternative 3: The following parameters are signalled,

CA BW Class: R12

maxaggregatedBW = 1600 MHz

CA BW Class: U

# 4	References

R4-2100264 "Release 17 FR2 bandwidth class", Verizon, 3GPP TSG-RAN WG4 Meeting #98-e, January 25th - February 5th, 2021

RP-210914 "Revised WID Further enhancements of NR RF requirements for frequency range 2 (FR2)", Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN Meeting #91-e, March 22nd - 26th, 2021

R4-2107860 "CR for TS 38.101-2: Introduction of FR2 new CA BW classes", Qualcomm, TSG-RAN WG4 Meeting #99-e, May 19th - 27th, 2021

R4-2114963 "WF on FR2 new CA BW Class", Nokia, TSG-RAN WG4 Meeting #100-e, August 16th - 27th, 2021

R4-2117541 "Fallback behavior of FBG3+2", Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 Meeting #101-e, November 1st - 12th, 2021

R4-2200909 "Alternatives of FR2 new CA BW classes", Apple, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th - 25th, 2022

R4-2200857 "FR2 bandwidth classes covering up to 1600 MHz aggregated bandwidth with mixed carrier bandwidths", Ericsson, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th - 25th, 2022

R4-2202347 "WF on FR2 CA BW classes", Xiaomi, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th - 25th, 2022

R4-2210783 “FR2 CA BW classes up to 2400 MHz aggregated BW with mixed channel bandwidths”, Ericsson, Verizon, 3GPP TSG-RAN WG4 Meeting #103-e, May 9th – 20th, 2022

R4-2207680 “Further views on new FR2 CA BW classes”, Apple, 3GPP TSG-RAN WG4 Meeting #103-e, May 9th – 20th, 2022