3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212589

# Online meeting, 15-26 Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Rel-17 38.101-2 to correct the notation for FBG5 CA BW class |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The notation Rx for FBG5 may make confusion with class R in FBG2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the notation as Vx |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | if not, it will cause understanding confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

5.3A.4	UE channel bandwidth per operating band for CA

For intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting a carrier aggregation bandwidth class with associated bandwidth combination sets specified in clause 5.5A.1. For each carrier aggregation configuration, requirements are specified for all aggregated channel bandwidths contained in a bandwidth combination set, UE can indicate support of several bandwidth combination sets per carrier aggregation configuration. The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier.

For intra-band non-contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting two or more sub-blocks, each supporting a carrier aggregation bandwidth class. The requirements are applicable only when Uplink CCs in each UL sub-block are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier of a DL sub-block.

Frequency separation class (Fs) specified in Table 5.3A.4-2 indicates the maximum frequency span between lower edge of lowest component carrier and upper edge of highest component carrier that UE can support per band in downlink or uplink (DL Fs or UL Fs) respectively in non-contiguous intra-band operation within the bidirectional spectrum.

The DL-only frequency spectrum is the width of UE frequency spectrum available to network to configure DL CCs only, and it extends on one-side of the bidirectional spectrum in contiguous manner with no frequency gap between the two. Frequency separation class for DL-only spectrum (Fsd) specified in Table 5.3A.4-3 and is declared per band. The frequency separation class for DL-only spectrum (Fsd) can be equal but not larger than the frequency separation (DL Fs). The combined downlink spectrum (DL Fs + Fsd) cannot exceed 2400 MHz. A UE may configure DL-only spectrum  only if the combined downlink spectrum (DL Fs + Fsd) exceeds 1400 MHz. When a UE configures DL-only spectrum, it shall not expect a CC to be configured across the boundary between bidirectional spectrum and DL-only spectrum UE can support respectively.

For inter-band carrier aggregation, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class.

Table 5.3A.4-1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 |  |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| V2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| V3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| V4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| V5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| V6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| V7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 |  |
| V8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| V9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| V10 | 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| V11 | 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| V12 | 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3, 4 and 5 are 400 MHz, 200 MHz, 100 MHz, 100 MHz and 200 MHz respectively except for CA bandwidth class A. For CA bandwidth classes of fallback group 5, requirements apply for non-interlaced 100 MHz and 200 MHz channel bandwidths (each CA bandwidth class consisting of up to two contiguous sub-blocks each with component carriers of a single channel bandwidth). NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. NOTE 3: In this release of the specification, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz (this note is not relevant for UE capability parsing by the network). |  |  |  |

< end of changes >