3GPP TSG-RAN WG4 Meeting #104-e	R4-2212753

Electronic Meeting, Aug 15- Aug 26, 2022

Agenda Item:	9.18.3.1.5

Source: 	Ericsson

Title:	Discussions on RedCap measurement procedure

Document for:	Discussion

# Introduction

In last meeting, the RedCap WI has closed and most measurement issues are agreed[1]. In this contribution, we’ll discuss the remaining issues for RedCap measurement.

# NCD-SSB measurement

Per-FR gap

In last meeting, the remaining open issue for measurement is as follow.

Whether to support for per-FR/per-UE gap Candidate options: - Option 1 (OPPO, Ericsson, CMCC, Nokia): If a RedCap UE support both FR1 and FR2, whether RedCap UE can support per-FR gap(e.g., independentGapConfigdf) depends on UE capability. - Option 2 (MTK, vivo, Apple): If MG is needed, both per-UE gap and per-FR gap can be supported, but UE behavior is same as per-UE gap.

From our understanding, RedCap UE can also support per-FR gap. When UE camps on a FR1 serving cell and NW configures a FR2 measurement, there is no gap interruption for serving cell if UE supports per-FR gap. Furthermore, considering forward compatibility, it’s better to keep the per-FR gap capability.

Proposal 1: If a RedCap UE support both FR1 and FR2, UE can support per-FR gap capability.

# Summary

In the contribution, we have discussed the CONNECTED mode measurement procedures for RedCap UE. Based on the discussions, we have made following proposals:

Proposal 1: If a RedCap UE support both FR1 and FR2, UE can support per-FR gap capability.

# References

- R4-2210592, “WF on RedCap RRM requirements”, Ericsson