GPP TSG-RAN WG4 Meeting # 104-e	R4-2212956

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.14.6.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on general RRM requirements for NR operation extending to 71GHz

Document for:	Discussion

# Introduction

In last RAN4 meeting, most of the remaining issues are completed for RRM requirements for FR2-2 with agreement captured in [1]. In this paper, we further provide our views on some remaining issues.

# Discussion

In lasting meeting, it is agreed to extend the Rx beam sweeping factor to 12 for FR2-2 without introducing new UE capabilities. And it is also agreed that the capability maxNumberRxBeam should be extended by the same number. The latest status are summarized as follows:

- Sub-topic 1-1: Rx beam sweeping scaling factor Agreement: Define an Rx beam sweeping scaling factor of 12 for FR2-2 Agreement: If Rx Beam Sweeping Scaling factor is extended, maxNumberRxBeam should be extended by the same number

Regarding the extension of capability maxNumberRxBeam, there is RAN2 impact to update the signalling. RAN4 should inform RAN2 to extend maxNumberRxBeam. A draft LS is attached in appendix.

Proposal 1: RAN4 should inform RAN2 to extend maxNumberRxBeam to 12 according to RAN4’s agreement.

In the agreed Big CR in last meeting, the beam sweeping factor has been updated for some requirements. However, there are some requirements where the beam sweeping factor is implicitly reflected, i.e. SCell activation. And the scaling factor in IDLE mode remains the same as that for FR2-1. It could be observed that a lot of requirements involving beam sweeping needs to be updated (Rel-15 – Rel-17). For Rel-17 features which are discussed in parallel, it is straightforward that FR2-2 is not considered in these WIs. For other requirements, RAN4 should discussed the impacts of beam sweeping factor on RRM requirements which needs to be updated.

Proposal 2: Identify the RRM requirements which needs to be updated for beam sweeping factors.

Another remaining issue is about frame boundary alignment tolerance of PDSCH symbols when deriveSSB-IndexFromCell is disabled. The options are as follows:

Way forward: (To be handled during maintenance phase) Frame boundary alignment tolerance of PDSCH symbols when deriveSSB-IndexFromCell is disabled - Option 1 SSB SCS (KHz); Data SCS (KHz); Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled). 960; 120; 1 120KHz symbol. 960; 480; 3 480KHz symbols. 960; 960; 6 960KHz symbols. - Option 2: SSB SCS (KHz); Data SCS (KHz); Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled). 960; 120; 1 120KHz symbol. 960; 480; 3 480KHz symbols. 960; 960; 7 960KHz symbols.

Option 1 was agreed in previous meetings. The motivation to have frame boundary alignment tolerance is to facilitate SSB index acquisition without PBCH decoding. As explained in last meeting, for 960 and 960 KHz, even the tolerance is extended to 7 symbols, it can still serve the purpose and give more freedom in deployment. Thus, it seems option 2 is a reasonable value to be considered.

Proposal 3:  Define frame boundary alignment tolerance of PDSCH symbols as 7 symbols for (960, 960) when deriveSSB-IndexFromCell is disabled.

An LS is sent to RAN4 about TCI assumption for RSSI measurement for FR2-2 [2]. Different from NR-U RSSI measurement, beam-based RSSI measurement is assumed in FR2-2. According to RAN1 agreement, the Rx beam for RSSI measurement can be either explicitly configured or use default beam. The TP are also included in RAN1 LS. It is straightforward to capture the TP in RAN4 spec.

Proposal 4: Capture the TP in RAN1 LS in TS 38.133.

The measurement relaxation of cell-detection was discussed in last meeting with following agreements:

Agreement: RAN4 to relax the cell-detection (PSS/SSS detection) delay requirements by a factor of 2 for 480kHz SCS and by a factor of 3 for 960kHz SCS to avoid increasing the HW and memory requirements

The motivation is to mitigate the hardware and memory burden of UE due to higher sampling rate with 480 and 960 KHz. Take 480 KHz for example, considering the extended RX beam sweeping factor and the relaxation factor above, the cell detection delay is tripled compared with that in FR2-1. For instance, when DRX is configured, the cell detection time may be extended by tens of seconds compared with FR2-1. Thus, we would like to further evaluate whether it is that serve to extend the delay significantly. We take 120 KHz in FR2-1 and 480 KHz in FR2-2 for comparison. It is assumed that 64 SSB indexes are configured to be measure to consider the worst case. The SSB patterns are shown as follows. For 480 kHz, as UE is required to monitor 3 symbols before and after SSB, then it almost means that UE needs to monitor all symbols of 32 slots. For 120 kHz in FR2-1, UE is required to measure one symbol before and after SSB, then it means UE needs to monitor 10 symbols in each slot. Considering same FFT size, the number of samples that UE needs to processed in 480 kHz is increased by 30-40% at the wort case. The situation is same for 960 kHz as UE will monitor all symbols at the worst case.

Fig. 1 Measurement window for FR2-1 120 kHz and FR2-2 480 kHz

Observation 1: The demand on hardware and memory for cell detection in 480 kHz and 960 kHz is increased compared with FR2-1 but not significantly.

On the other hand, if UE supports the operation in 480 kHz and 960 kHz, the UE shall be more powerful to support not only RRM measurement but also other procedures (e.g. demodulation). Thus, we don't think the RRM measurement put extra burdens on UE which supports 480 kHz and 960 kHz.

Observation 2: For powerful UE supporting 480 kHz and 960 kHz, the delay will be even longer than 120 kHz.

However, we also understand the consideration from companies and agreements reached in last meeting. We would like to consider to introduce a corresponding UE capability. The relaxed requirements only apply to UE incapable of the capability.

Proposal 5: Introduce UE capability on relaxed requirements for cell detection.

# Conclusions

Proposal 1: RAN4 should inform RAN2 to extend maxNumberRxBeam to 12 according to RAN4’s agreement.

Proposal 2: Define frame boundary alignment tolerance of PDSCH symbols as 7 symbols for (960, 960) when deriveSSB-IndexFromCell is disabled.

Proposal 3: Capture the TP in RAN1 LS in TS 38.133.

Proposal 4: Introduce scheduling restriction on DL during RSSI measurement in FR2-2.

Observation 1: The demand on hardware and memory for cell detection in 480 kHz and 960 kHz is increased compared with FR2-1 but not significantly.

Observation 2: For powerful UE supporting 480 kHz and 960 kHz, the delay will be even longer than 120 kHz.

Proposal 5: Introduce UE capability on relaxed requirements for cell detection.

# References

[1] R4-2210590 WF on NR extension to 71 GHz RRM requirements (Part 1), Qualcomm

[2] R1-2205582 LS on TCI assumption for RSSI measurement for FR2-2, RAN1

# Appendix

3GPP TSG-RAN WG4 Meeting #104e	R4-22xxxxx

Electronic Meeting, August 15 – August 26, 2022

Title:	 LS on updating maxNumberRxBeam in FR2-2

Response to:

Release:	Rel-17

Work Item:	NR_ext_to_71GHz-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:	Zhongyi Shen

E-mail:	shenzhongyi3@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

In RAN4#103e, it is agreed that the RX beam sweeping scaling factor is defined as 12 for FR2-2 for RRM requirements in TS 38.133. It is further identified that maxNumberRxBeam should be extended by the same number as the maximum value of maxNumberRxBeam is 8 in existing specifications.

Therefore, RAN4 respectfully asks RAN2 to consider the design of the corresponding parameters to extend maxNumberRxBeam to 12 for FR2-2.

2. Actions:

To RAN1 and RAN2 group:

ACTION: 	RAN4 would like to kindly ask RAN2 to consider the design of the corresponding parameters.

3. Dates of next TSG-RAN WG4 meetings:

RAN4#104-bis-e	10 – 19 Oct 2022			e-Meeting