3GPP TSG-RAN WG4 Meeting #104-e	R4-2213094

Electronic Meeting, 15 August – 26 August 2022

Source:	Ericsson, Telstra

Title:	TP for TR 37.718-21-11: Including band combinations DC_3-(n)7

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-21-11 to add DC_3A-(n)7AA.

# 2. Text Proposal

---Start of changes---

## 5.x	DC_3-(n)7

### 5.x.1	Operating bands for EN-DC

Table 5.x.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-(n)7 | CA_3-7 | n7 |  |

### 5.x.2	Configuration for DC

Table 5.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-(n)7AA | DC_3A-7A | CA_3A-7A | n7A |
| DC_3C-(n)7AA | DC_3A-7A | CA_3C-7A | n7A |

### 5.x.3		∆TIB and ∆RIB values

For DC_3-(n)7, the TIB,c and RIB,c values are reused from DC_3_n7 and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-(n)7 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n7 | 0.5 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-(n)7 | 3 | 0 |
|  | 7 | 0 |
|  | n7 | 0 |

### 5.x.4	REFSENS requirements

There are possible IMD4 impact from UL 3_7 affecting band n7.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-(n)7AA DC_3C-(n)7AA | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n7 | 2515 | 5 | 25 | 2635 | 10.2 | IMD4 |

---End of changes---

# Reference

[1]		RP-221832, New WID: Rel-18 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon