3GPP TSG-RAN4 Meeting # 104-e	R4-2213274

# Electronic Meeting, ,  15-26 August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test set 4-3 CR to introduce RSTD accuracy test case with reduced number of samples in FR1 in RRC_CONNECTED state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce RSTD accuracy test case with reduced number of samples in FR1 in RRC_CONNECTED state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test set 4-3 is implemented as agreed in RAN4#103-e R4-2211060. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RSTD accuracy with reduced number of samples cannot be verified in RRC_CONNECTED FR1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.7.13.X1 and A.6.7.13.X2 are added. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------START OF CHANGE-------------------------------------------------

#### A.6.7.13.X1	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state

##### A.6.7.13.X1.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements for reduced number of samples specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.7.13.X1.1-1.

Table A.6.7.13.X1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cells. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be longer than the UE measurement period as defined in clause 9.9.2.7.

Table A.6.7.13.X1.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| Duplex mode | 1 |  | FDD |  |  | FDD |  |
|  | 2 |  | TDD |  |  | TDD |  |
|  | 3 |  | TDD |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  | - | 3 |
|  | 2,3 |  | - | 3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.2 FR1 |  |  | PRS.1.2 FR1 |  |
|  | 2 |  | PRS.1.2 FR1 |  |  | PRS.1.2 FR1 |  |
|  | 3 |  | PRS.2.2 FR1 |  |  | PRS.2.2 FR1 |  |
| PRS muting info | 1~3 |  | ‘10’ | ‘01’ |  | ‘10’ | ‘01’ |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  | -98 |  |
|  | 3 |  | -95 |  |  | -95 |  |
|  | 1~3 | dB | -3 | 4 |  | -3 | 4 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -101 | -94 |  | -101 | -94 |
|  | 3 |  | -98 | -91 |  | -98 | -91 |
| SS-RSRPNote3 | 1,2 | dBm/SCS | -88 | -88 |  | -88 | -88 |
|  | 3 |  | -85 | -85 |  | -85 | -85 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -69.5 | -67.9 |  | -69.5 | -67.9 |
|  | 3 | dBm/ 38.16MHz | -63.4 | -61.8 |  | -63.4 | -61.8 |
|  | 1~3 | dB | -3 | 4 |  | -3 | 4 |
| Propagation condition | 1~3 | - | AWGN |  |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

##### A.6.7.13.X1.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.7.13.X2	RSTD measurement accuracy test case with reduced number of samples for dual positioning frequency layers in FR1 in RRC_CONNECTED state

##### A.6.7.13.X2.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.7.13.X2.1-1.

Table A.6.7.13.X2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell on NR RF channel #1 in FR1. Cell 2 is a neighbour cell on a different NR RF channel #2 in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be longer than the UE measurement period as defined in clause 9.9.2.7.

Table A.6.7.13.X2.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  | 2 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| Duplex mode | 1 |  | FDD |  | FDD |  |
|  | 2 |  | TDD |  | TDD |  |
|  | 3 |  | TDD |  | TDD |  |
| TDD configuration | 1 |  | N/A |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 | - | 3 |
|  | 2,3 |  | - | 3 | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  | 2 |  | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  | 3 |  | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info | 1~3 |  | ‘10’ | ‘01’ | ‘10’ | ‘01’ |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  | -98 |  |
|  | 3 |  | -95 |  | -95 |  |
|  | 1~3 | dB | -3 | 4 | -3 | 4 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -101 | -94 | -101 | -94 |
|  | 3 |  | -98 | -91 | -98 | -91 |
| SS-RSRPNote3 | 1,2 | dBm/SCS | -88 | -88 | -88 | -88 |
|  | 3 |  | -85 | -85 | -85 | -85 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -69.5 | -67.9 | -69.5 | -67.9 |
|  | 3 | dBm/ 38.16MHz | -63.4 | -61.8 | -63.4 | -61.8 |
|  | 1~3 | dB | -3 | 4 | -3 | 4 |
| Propagation condition | 1~3 | - | AWGN |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

##### A.6.7.13.X2.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

-------------------------------------------------END OF CHANGE-------------------------------------------------