3GPP TSG-RAN WG4 Meeting # 104-e                                                              R4-2213572

# Electronic Meeting, August 15 – August 26, 2022

Source:	Sony

Title:	Views on RAN4 action on UL Segmented Transmission for UL synchronization for IoT NTN

Agenda item:	13.2.1

Document for:	Approval

# Background

In [1], an LS from RAN1 is sent to RAN4 with the aspects of UL segmented transmission and leaves it up to RAN4 to specify it for UL synchronization for IoT NTN:

- UE applies segmented UL transmission according to duration configuration by the network

- Different values (e.g., TA) for pre-compensation may be used per segment, where UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment

RAN1 respectfully requests RAN4 to prioritize the UL Segmented Transmission for UL synchronization for IoT NTN work by considering the above.

In this contribution, we share our views on the necessary changes in the RAN4 specifications to enable the UL segmented transmission for IoT NTN in Rel-17.

# Discussion

For IoT NTN, due to the tight link budget (in particular for uplink), it is expected that the UE needs to transmit uplink with multiple repetitions over a long PUSCH transmission, where a number of PUSCH repetitions is defined as a UL transmission segment. According to RAN1 LS [1], different TA values can be applied for different segments, and the motivation is to allow the UE to adjust pre-compensation values during ongoing repetition to avoid UL sync loss caused by large TA and Doppler drift.

However, the legacy TN eMTC and NB-IoT UE shall not change TA during ongoing UL transmissions according to TS 36.133 in section 7.20.2/ 7.24.2 as below:

Therefore, without clarification in TS 36.133, Rel-17 IoT NTN UE may follow the behavior of legacy TN NB-IoT or eMTC, which disables the feature introduced by RAN1. Thus, a clarification on the IoT NTN behavior in TS36.133 is needed.

Proposal 1: RAN4 shall clarify the IoT NTN behavior in TS 36.133 for Rel-17 to allow UEs to adjust TA during the ongoing repetition.

Meanwhile, it is worth clarifying that the TA value can be adjusted per segment but is not mandatory if the variation is small enough. In addition, UE shall keep a constant TA value within each segment. Therefore, the RAN4 specification shall also specify this UE behavior accordingly.

Proposal 2: RAN4 shall specify the IoT NTN behavior in TS 36.133 to ensure the UE keeps a constant TA value within each segment

In line with the above two proposals and in line with the agreements in RAN1 and the LS from RAN1, the following text proposal for TS36.133 is made:

For satellite access, when a repetition is configured on the uplink for which R > 1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary, as defined above.

The text proposal above can be either added in the end of section 7.20.2/ 7.24.2 (for NB-IoT and eMTC respectively) in 36.133 or in a potential new section in 36.133 for the IoT NTN UE.

In addition, for NTN eMTC PUCCH/PUSCH with frequency hopping enabled, the UE can adjust the uplink transmit timing and transmit frequency when hopping to a new narrowband if the frequency hopping interval is less than or equal to the configured transmission segment duration, which may also need RAN4 to clarify the corresponding UE behavior in 36.113.

Observation 1: for eMTC PUCCH/PUSCH with frequency hopping enabled, the UE can adjust the uplink transmit timing and transmit frequency when hopping to a new narrowband if the frequency hopping interval is less than or equal to the configured transmission segment duration, which may also need RAN4 to clarify the corresponding UE behaviour in 36.113.

Proposal 3: RAN4 shall further study if there is any specification impact when UE performs frequency hopping for IoT NTN.

# Conclusion

In this contribution, we have made the following observations and proposals regarding the UL segmented transmission for IoT NTN in Rel-17 from the RAN4 aspect:

Observation 1: for eMTC PUCCH/PUSCH with frequency hopping enabled, the UE can adjust the uplink transmit timing and transmit frequency when hopping to a new narrowband if the frequency hopping interval is less than or equal to the configured transmission segment duration, which may also need RAN4 to clarify the corresponding UE behavior in 36.113.

Proposal 1: RAN4 shall clarify the IoT NTN behavior in TS 36.133 for Rel-17 to allow UEs to adjust TA during the ongoing repetition.

Proposal 2: RAN4 shall specify the IoT NTN behavior in TS 36.133 to ensure the UE keeps a constant TA value within each segment

Proposal 3: RAN4 shall further study if there is any specification impact when UE performs frequency hopping for IoT NTN.

# References

- R1-2205642	LS on UL Segmented Transmission for UL synchronization for IoT NTN    RAN1