3GPP TSG-RAN WG4 Meeting#104-e                              R4-2213717

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	9.5.5.2

Source: 	ZTE Corporation

Title: 	Discussion on FR1 NR repeater test conformance testing

Document for:	Approval

Introduction

In last RAN4 meeting, Core requirement of NR repeater has been completed. Based on the approved Rel-17 RAN4 led repeater WID RP-212129, the conformance testing for both NR repeater should be completed in Sep, 2022. In this contribution, we provide some further views on conformance testing requirement for FR1 NR repeater.

Discussion

- 2.1. Test setup for FR1 NR repeater

For FR1 TDD set-up, there is one remaining issue as following left for further discussions. From our understanding, the DL information at the repeater output could be estimated by DL PSS/SSS/DMRS signals and UL timing information at the repeater output could be estimated by the PRACH or SRS signals.

If the group delay of repeater is marginal, then DL and UL timing information could be also informed by the cable. Or if the group delay of repeater is relatively large, the vendor could declare the group delay, then DL and UL timing information could be informed by the cable and declared group delay.

Observation 1: there might be two options to obtain the DL/UL timing information:

Option 1: the DL information at the repeater output could be estimated by DL PSS/SSS/DMRS signals and UL timing information at the repeater output could be estimated by the PRACH or SRS signals.

Option 2: If the group delay of repeater is marginal, then DL and UL timing information could be also informed by the cable. Or if the group delay of repeater is relatively large, the vendor could declare the group delay, then DL and UL timing information could be informed by the cable and declared group delay.

Sub-topic#1-2 FR1 TDD set-up

Recommended Agreement:

Only one link direction at a time is being tested

UL/DL timing could be provided through DL signals or cable to repeater under the test

- Other methods not precluded;

- FFS whether and if needed how to specify UL/DL timing into specification

- 2.2. Measurement uncertainty

In the last RAN4 meeting, we agreed to use R4-2210014 as a baseline and discuss further, the following potential issue which may require adjusting the MU values have been raised:

- Input signal uncertainty and it effect on the output power.

- ALCR BW’s need clarification on which BW is intended.

- Confirm UL and DL values are the same

The following table is used as the starting point for further discussion in next meeting,

For BW mentioned in the following table, the BW should be targeted for nominal repeater channel bandwidth since we don’t have carrier bandwidth definition for NR repeater.

Proposal 1: BW should be replaced by the nominal repeater channel bandwidth for ACLR requirements.

| Subclause | Maximum Test System Uncertainty and Range over which Test System Uncertainty applies |
| --- | --- |
| 6.2 Output power | ±0.7 dB, f ≤ 3 GHz ±1.0 dB, 3 GHz < f ≤ 6 GHz (Note) |
| 6.3 Frequency stability | ±12 Hz Measurement results of 500 Hz |
| 6.4 Out of band gain | ±0,5 dB, f ≤ 3.0 GHz ±0,8 dB, 3.0 GHz < f ≤ 6 GHz Calibration of test set-up shall be made without DUT in order to achieve the accuracy |
| 6.5.2 ACLR | ACLR/ CACLR nominal repeater channel bandwidth ≤ 20MHz: ±0.8 dB nominal repeater channel bandwidth > 20MHz: ±1.2 dB Absolute power ±2.0 dB, f ≤ 3 GHz Absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) CACLR nominal repeater channel bandwidth ≤ 20MHz: ±0.8 dB nominal repeater channel bandwidth > 20MHz: ±1.2 dB CACLR absolute power ±2.0 dB , f ≤ 3 GHz CACLR absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) |
| 6.5.3 Operating band unwanted emission (except 9.1.5.4) | ±1.5 dB, f ≤ 3 GHz ±1.8 dB, 3 GHz < f ≤ 6 GHz (Note) and The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station according toTS36.141-1 [11] |
| 6.5.4.2.1 General Spurious emissions | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB 19 GHz < f ≤ 26 GHz: ±4.5 dB |
| 6.5.4.2.2 Additional spurious emissions requirements | ±2.0 dB for > -60 dBm, f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 4.2 GHz ±3.0 dB, 4.2 GHz < f ≤ 6 GHz ±3.0 dB for ≤ -60 dBm, f ≤ 3 GHz ±3.5 dB, 3 GHz < f ≤ 4.2 GHz ±4.0 dB, 4.2 GHz < f ≤ 6 GHz |
| 6.5.4.2.3 Co-location with base stations and repeater type 1-C Nodes | ±3.0 dB |
| 6.6 Error vector magnitude | 1% signal analyser, 2% stimulus signal Requirement limit shifted by RSS requirement and stimulus signal EVM. Analyser error added to requirement limit. |
| 6.7 Input intermodulation | ±1.3 dB, f ≤ 3 GHz ±1.4 dB, 3 GHz < f ≤ 6 GHz |
| 6.8 Output intermodulation | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests in 6.6.3 (ACLR), 6.6.4 (OBUE) and 6.6.5 (spurious emissions) which have to be carried out in the presence of the interferer. ±1.0 dB |
| 6.9 Adjacent channel rejection ratio | 0,7 dB |
| 6.10 Transmit ON/OF power | Transmit OFF power ±2.0 dB , f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) |

- 2.3. Test procedure

Regarding the test procedures for FR1 NR repeater, there are two remaining issues left for further discussion in [7].

In the following section, for EVM conformance testing under ALC condition, from our understanding, it’s reasonable to have test under the ALC and non-ALC condition to ensure the EVM performance otherwise some clipping or other incorrect approach handling might cause signal quality within channel degradation.

Proposal 2: to use option 3 for the EVM conformance testing.

For RX IM requirement for repeater, currently we think that option 1 is sufficient for test its function if RX IM falling into the channel unless there is other technical reasons to need multiple testing.

Proposal 3: to use option 1 for the Rx IM conformance testing.

Conclusions

In this contribution, we provided some initial views on conformance testing requirement for FR1 NR repeater. and proposals are made as following:

Observation 1: there might be two options to obtain the DL/UL timing information:

Option 1: the DL information at the repeater output could be estimated by DL PSS/SSS/DMRS signals and UL timing information at the repeater output could be estimated by the PRACH or SRS signals.

Option 2: If the group delay of repeater is marginal, then DL and UL timing information could be also informed by the cable. Or if the group delay of repeater is relatively large, the vendor could declare the group delay, then DL and UL timing information could be informed by the cable and declared group delay.

Proposal 1: BW should be replaced by the nominal repeater channel bandwidth for ACLR requirements.

Proposal 2: to use option 3 for the EVM conformance testing.

Proposal 3: to use option 1 for the Rx IM conformance testing.

References

- TS 38.106 17.0.0

- TS 36.143 17.0.0

- R4-2210631,WF on NR FR1/FR2 repeater measurement system set-up, ZTE, Revised to R4-2211223

- R4-2211223,WF on NR FR1/FR2 repeater measurement system set-up, ZTE, approved

- R4-2211150,WF on NR FR1/FR2 repeater MU and TT, Huawei, approved.

- R4-2211151, WF on NR FR1/FR2 repeater declarations, Nokia

- R4-2211152,WF on NR FR1/FR2 repeater test procedures and test issues, Ericsson

- R4-2211153, WF on NR FR1/FR2 repeater test specs draft rules and TP splits, CATT, approved.

- R4-2210845,Spec skeleton for TS 38.115-1, CATT, approved.

- R4-2210846,Spec skeleton for TS 38.115-2 v.0.0.1, ZTE Corporation. Approved.