3GPP TSG-RAN WG4 Meeting # 104	R4-2213782

# Electronic Meeting, Aug 15th –Aug 26rd,2022

Title: 	Discussion on remain issues for demodulation requirements for inter cell MMSE-IRC receiver

Source: 	Huawei, HiSilicon

Agenda item:	9.10.1.1

Document for:	Discussion

Background

There are only two open issues for PDSCH performance requirements for MMSE-IRC receiver. In this paper we provide our views on these open issues.

Discussions

The only left open issue to be discussed is PDCCH configuration. The options are listed as follows:

- Way Forward - Option 1: All the REs in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports. - Option 2: Assume PDCCH transmission from interference cells, and for each cell, OCNG signal is transmitted on each RE that is not occupied by the PDCCH of this cell. - Option 3: Assume PDCCH transmission from interference cells which is non-overlapping with serving cell.

Based on our understanding, our purpose is to verify the PDSCH requirements, therefore we should focus on interference signals in PDSCH region.  But we have assumed that SSB, TRS and CSI which are not overlapped with PDSCH signals in serving cell are transmitted in interference cells, therefore, we don't see the reason that PDCCH is not transmitted. Moreover, PDCCH is used for scheduling the PDSCH, it is not practical to only transmit PDSCH without PDCCH scheduling. Same time, we should also guarantee the PDCCH performance (BLER<1%) when UE decoding PDSCH, the simplest way is transmitting the PDCCHs from different cells in different PDCCH candidate positions/ CCE indexs to avoid overlapping. I.e. PDCCH of serving cell is transmitted in CCE#0-7, PDCCH of interference cell 1 is transmitted in CCE#8-11 and PDCCH of interference cell 2 is transmitted in CCE#12-15 as shown in Figure 2-1. For other configurations, propose to reuse the existing NR common test parameter in Table 5.2-1 in TS 38.101-4.

Figure 2-1: Proposed PDCCH transmission pattern

Table 2-1 shows the summary of PDCCH configurations

Table 2-1: Summary of PDCCH configurations

| PDCCH parameters | Serving cell | Interference cell 1 | Interference cell 2 |  |
| --- | --- | --- | --- | --- |
| Number of PRBs in CORESET | 48 for FDD;102 for TDD |  |  |  |
| Symbols with PDCCH | 2 |  |  |  |
| Number of PDCCH candidates and aggregation levels | 1/AL8 | 1/AL4 |  | 1/AL4 |
| CCE-to-REG mapping type | Non interleaved |  |  |  |
| DCI format | DCI format 1_1 |  |  |  |
| PDCCH transmission | CCE #0-7 (PDCCH candidate #0) | CCE #8-15(PDCCH candidate #1) | CCE #12:15(PDCCH candidate #2) |  |

Proposal 1: Use Table 2-1 as PDCCH configurations. I.e. Option 3

SNR deriving rule

RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes X dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within X dB. - Option 1: X = 2dB - Option 2: X = 2.5dB

We suggest option 2 to follow the legacy procedure.

Proposal 2: RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes X dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within X dB. X=2.5

Conclusion

In this paper, we provide our views on inter cell MMSE-IRC receiver. The proposals are:

Proposal 1: Use Table 2-1 as PDCCH configurations. I.e. Option 3

Proposal 2: RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes X dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within X dB. X=2.5

Reference

[1]   WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC. Ericsson