3GPP TSG-RAN WG4 Meeting # 104e	R4-2213814

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Discussions on remain issues on FR2-2 PUCCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.14.8.3.2

Document for:	Discussion

Background

As per [1], RAN 4 left some open issues on FR2-2 PUCCH demodulation requirements, in this paper, we provide our views on these open issues

Discussions

General test parameters

Antenna configuration

Same as FR2-1 PUCCH cases, we suggest to use 1T2R.

Propagation conditions

FR2-2 is applicable for static environment, we suggest to use TDLA10-200.

SCS/Bandwidth

Same as PUSCH/PDSCH requirements, we propose to consider 120kHz and 480kHz SCS. As for bandwidth, we suggest to consider minimum bandwidth for each SCS considering testability.

Proposal 1:  Consider following assumptions for FR2-2 PUCCH requirements definition:

- 1T2R

- TDLA10-200

- 120kHz/100MHz and 480kHz/400MHz

Proposed cased for PF0

For FR2-2 PF0 requirements definition, we reached following agreements in last meeting:

- For Format 0: - 1RB /1 symbol - FFS for [10/16] RB/1 symbol - For Format 0: - Intra-slot frequency hopping: enabled if number of OFDM symbols higher than 1 - Test metric: ACK missed detection rate < 1%

RAN 1 introduced multi-RB for PF0/1/4, we think it should be verified. To minimum the test number, we suggest to configure 1RB for 1 OFDM symbol and 16RBs for 2 OFDM symbol.

The proposed cases are captured in Table 2-1:

Table 2-1: Proposed cases for FR2-2 PF0

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA10-200 Low | 1 | 120 | 100 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |
|  |  |  |  | 480 | 400 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |

Proposal 2: Use Table 2-1 for FR2-2 PF0 simulation assumptions.

Proposed cases for PF1

For FR2-2 PF1 requirements definition, we reached following agreements in last meeting:

- For format 1: - 1 RB/14 symbols - [10/16] RB/14 symbols - For format 1: - Intra-slot frequency hopping: enabled - Test metric: ACK missed detection rate < 1% and NACK to ACK probability < 0.1%

The number of RBs of PF1 can be flexibly configured from 0 to 16 in FR2-2. Considering 1RB has been agreed, we propose to additionally consider 16RBs.

The proposed cases are captured in Table 2-2:

Table 2-2: Proposed cases for FR2-2 PF1

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA10-200 Low | 2 | 120 | 100 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<0.1% |
|  |  |  |  | 480 | 400 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<1% |

Proposal 3: Use Table 2-2 for FR2-2 PF1 simulation assumptions.

Proposed cases for PF2

For FR2-2 PF2 requirements definition, we reached following agreements in last meeting:

- For format 2: Reusing FR2-1 - For format 2: - Intra-slot frequency hopping: enabled - Test metric: ACK missed detection rate < 1%, UCI BLER < 1%

Based on the agreements, the proposed cases are captured in Table 2-3:

Table 2-3: Proposed cases for FR2-2 PF2

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA10-200 Low | 120 | 100 | 1 | 4 | 4 | Enabled | Prob(DTXACK)<1% Prob(ACK miss)<1% |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | Prob(DTXACK)<1% Prob(ACK miss)<1% |
|  |  |  | 480 | 400 | 1 | 4 | 4 | Enabled | Prob(DTXACK)<1% Prob(ACK miss)<1% |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | Prob(DTXACK)<1% Prob(ACK miss)<1% |

Proposal 4: Use Table 2-3 for FR2-2 PF2 simulation assumptions.

Proposed cased for PF3

For FR2-2 PF2 requirements definition, we reached following agreements in last meeting:

- For format 3: Reusing FR2-1 - For format 3: - Intra-slot frequency hopping: enabled - Test metric: UCI BLER < 1%

In Rel-15, RAN 4 has defined two DMRS configurations: Additional DMRS configuration and No additional DMRS. Additional DMRS configuration always apply for high Doppler scenario. As we don’t consider high Doppler spread, we suggest to only use no additional DMRS.

The proposed cases are captured in Table 2-3:

Table 2-4: Proposed cases for FR2-2 PF3

| Number of TX antennas | Number of demodulation branches | Modulation order | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | QPSK | TDLA10-200 Low | 120 | 100 | 14 | 1 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  | 480 | 400 | 14 | 1 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |

Proposal 5: Use Table 2-4 for FR2-2 PF3 simulation assumptions.

Proposed cased for PF4

For FR2-2 PF4 requirements definition, we reached following agreements last meeting:

- For format 4: - 1 RB/14 symbols - [10/16] RB/14 symbols

Same views as PF3, we propose to only consider no additional DMRS configuration.

The proposed cases are captured in Table 2-4:

Table 2-5: Proposed cases for FR2-2 PF4

| Number of TX antennas | Number of demodulation branches | Modulation order | Propagation conditions and correlation matrix | Length of the orthogonal cover code | Index of the orthogonal cover code | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | QPSK | TDLA10-200 Low | n2 | n0 | 120 | 100 | 14 | 1 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  |  |  |  |  | 14 | 16 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 480 | 400 | 14 | 1 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  |  |  |  |  | 14 | 16 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |

Proposal 6: Use Table 2-5 for FR2-2 PF4 simulation assumptions.

Conclusion

In this paper we provide our views on open issues for FR2-2 PUCCH performance requirements definition. The proposals are:

Proposal 1:  Consider following assumptions for FR2-2 PUCCH requirements definition:

- 1T2R

- TDLA10-200

- 120kHz/100MHz and 480kHz/400MHz

Proposal 2: Use Table 2-1 for FR2-2 PF0 simulation assumptions.

Proposal 3: Use Table 2-2 for FR2-2 PF1 simulation assumptions.

Proposal 4: Use Table 2-3 for FR2-2 PF2 simulation assumptions.

Proposal 5: Use Table 2-4 for FR2-2 PF3 simulation assumptions.

Proposal 6: Use Table 2-5 for FR2-2 PF4 simulation assumptions.

Reference

[1]   R4-2210664 WF on general and BS aspects for FR2-2 demodulation requirements. Intel Corporation