3GPP TSG-RAN WG4 Meeting # 104e	R4-2213815

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Simulation results on FR2-2 PUCCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.14.8.3.2

Document for:	Discussion

Background

As per [1], RAN 4 agreed to introduce FR2-2 PUCCH performance requirements. In this paper we provide our initial simulation results based on our proposed simulation assumptions.

Simulation results

Simulation results for PF0

Table 2-1: Simulation assumptions for FR2-2 PF0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |

Table 2-2: Simulation results for FR2-2 PF0

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric | Target SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA10-200 Low | 1 | 120 | 100 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | 6.2 |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | -10.0 |
|  |  |  |  | 480 | 400 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | 6.0 |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | -10.7 |

Simulation results for PF1

Table 2-3: Simulation assumptions for FR2-2 PF1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 and 16 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

Table 2-4: Simulation results for FR2-2 PF1

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric | Target SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA10-200 Low | 2 | 120 | 100 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% | -3.8 |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% | -5.1 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% | -15.5 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<1% | -16.8 |
|  |  |  |  | 480 | 400 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% | -4.1 |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% | -5.9 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% | -14.7 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<1% | -16.4 |

Simulation results for PF2

Table 2-5: Simulation assumptions for FR2-2 PF2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs-1) |
| Number of PRBs | 4 and 9 |
| Number of symbols | 1 and 2 |
| The number of UCI information bits | 4 and 22 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |

Table 2-6: Simulation results for FR2-2 PF2

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Test metric | Target SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA10-200 Low | 120 | 100 | 1 | 4 | 4 | Enabled | Prob(DTXACK)<1% Prob(ACK miss)<1% | 3.2 |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | Prob(UCI BLER)<1% | -0.1 |
|  |  |  | 480 | 400 | 1 | 4 | 4 | Enabled | Prob(DTXACK)<1% Prob(ACK miss)<1% | 1.4 |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | Prob(UCI BLER)<1% | -1.1 |

Simulation results for PF3

Table 2-7: Simulation assumptions for FR2-2 PF3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |

Table 2-8: Simulation results for FR2-2 PF3

| Number of TX antennas | Number of demodulation branches | Modulation order | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric | Target SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | QPSK | TDLA10-200 Low | 120 | 100 | 14 | 1 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% | -2.1 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% | -1.0 |
|  |  |  |  | 480 | 400 | 14 | 1 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% | -2.2 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% | -1.0 |

Simulation results for PF4

Table 2-9: Simulation assumptions for FR2-2 PF4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hoppingstartingPRB | 0 |
| Number of PRBs | 1 and 16 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

Table 2-10: Simulation results for FR2-2 PF4

| Number of TX antennas | Number of demodulation branches | Modulation order | Propagation conditions and correlation matrix | Length of the orthogonal cover code | Index of the orthogonal cover code | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric | Target SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | QPSK | TDLA10-200 Low | n2 | n0 | 120 | 100 | 14 | 1 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% | 1.4 |
|  |  |  |  |  |  |  |  | 14 | 16 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% | -11.3 |
|  |  |  |  |  |  | 480 | 400 | 14 | 1 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% | 1.6 |
|  |  |  |  |  |  |  |  | 14 | 16 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% | -11.0 |

Conclusion

In this paper we provide our initial simulation results for FR2-2 PUCCH.

Reference

[1]   R4-2210664 WF on general and BS aspects for FR2-2 demodulation requirements. Intel Corporation