3GPP TSG-RAN WG4 Meeting #104-e	R4-2213857

# Electronic Meeting, 15th – 26th Aug, 2022

Title: 	Discussion and simulation results on satellite NTN demod PUCCH

Source: 	Huawei, HiSilicon

Agenda item:	9.11.7.2.2

Document for:	Discussion

Background

During RAN4#103-e meeting, WF [1] on NTN SAN demodulation requirements was approved. In this contribution, we discuss and provide our initial simulation results about the NTN PUCCH demodulation requirements.

Discussion

Scope of PUCCH requirements

Agreements - PUCCH format 2/3/4: additional note should be added that the UCI information does not contain CSI part 1 and CSI part 2. - PUCCH format 0: select 2 symbol case from legacy BS PUCCH format 0 requirements. - PUCCH format 1: Same as the legacy BS PUCCH format 1 - PUCCH format 2: select both two cases with different UCI information bits from legacy BS requirements - PUCCH format 3: only select 1 PRB with 14 symbol case from legacy BS requirements - PUCCH format 4: Same as the legacy BS PUCCH format 4 - For PUCCH format 3/4, FFS DMRS configuration: - Option 1: DM-RS 1+0 - Option 2: Both DM-RS 1+0 and 1+1 - PUCCH format 1(multi-slot): Same as the legacy BS PUCCH format 1 (multi-slot)

The remained issue is whether additional DMRS is configured for PUCCH format 3/4. To reduce the test effort, we prefer to only define DMRS 1+0 that is typical configuration. However, considering that there is also manufacture declaration introduced for the latest specification for the NTN SAN conformance testing, we are OK to use the same method as legacy BS to define both DM-RS 1+0 and 1+1 requirements and BS decide whether to perform testing via the manufacture declaration, i.e. supporting additional DM-RS for PUCCH format 3/4 or not.

Use the same method as legacy BS to define both DM-RS 1+0 and 1+1 requirements and BS decide whether to perform testing via the manufacture declaration, i.e. supporting additional DM-RS for PUCCH format 3/4 or not.

Simulation

PUCCH format 0

Here we provide the initial simulation results for PUCCH format 0 for alignment, as shown in Table 3.1-2, based on simulation assumption as shown in Table 3.1-1.

Table 3.1-1 Simulation assumption for PUCCH format 0

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Channel model | NTN-TDLA100-200 |
| Number of OFDM symbols | 2 |
| Test metric | ACK missed detection probability <1% |

Table 3.1-2 Ideal simulation results for PUCCH format 0

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x1 | 6.92 |
| 2 | 15 | 5 | 1x2 | 0.72 |
| 3 | 30 | 10 | 1x1 | 8.58 |
| 4 | 30 | 10 | 1x2 | 1.42 |

PUCCH format 1

Here we provide the initial simulation results for PUCCH format 1 for alignment, as shown in Table 3.2-2, based on simulation assumption as shown in Table 3.2-1.

Table 3.2-1 Simulation assumption for PUCCH format 1

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 3.2-2 Ideal simulation results for PUCCH format 1

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ACK missed detection | NACK to ACK |
| 1 | 15 | 5 | 1x1 | -1.19 | -0.29 |
| 2 | 15 | 5 | 1x2 | -7.21 | -5.77 |
| 3 | 30 | 10 | 1x1 | 0.44 | 3.65 |
| 4 | 30 | 10 | 1x2 | -6.25 | -3.39 |

PUCCH format 2

ACK missed detection

Here we provide the initial simulation results for PUCCH format 2 for ACK missed detection for alignment, as shown in Table 3.3.1-2, based on simulation assumption as shown in Table 3.3.1-1.

Table 3.3.1-1 Simulation assumption for PUCCH format 2 for ACK missed detection

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | ACK missed detection probability <1% |

Table 3.3.1-2 Ideal simulation results for PUCCH format 2 for ACK missed detection

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x1 | 9.64 |
| 2 | 15 | 5 | 1x2 | 1.38 |
| 3 | 30 | 10 | 1x1 | 8.88 |
| 4 | 30 | 10 | 1x2 | 1.21 |

UCI BLER

Here we provide the initial simulation results for PUCCH format 2 for UCI BLER for alignment, as shown in Table 3.3.2-2, based on simulation assumption as shown in Table 3.3.2-1.

Table 3.3.2-1 Simulation assumption for PUCCH format 2 for UCI BLER

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | UCI block error probability<1% |

Table 3.3.2-2 Ideal simulation results for PUCCH format 2 for UCI BLER

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x1 | 3.94 |
| 2 | 15 | 5 | 1x2 | -2.00 |
| 3 | 30 | 10 | 1x1 | 3.93 |
| 4 | 30 | 10 | 1x2 | -1.98 |

PUCCH format 3

Here we provide the initial simulation results for PUCCH format 3 for alignment, as shown in Table 3.4-2, based on simulation assumption as shown in Table 3.4-1.

Table 3.4-1 Simulation assumption for PUCCH format 3

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DM-RS |
| Channel model | NTN-TDLA100-200 |
| Test metric | UCI block error probability<1% |

Table 3.4-2 Ideal simulation results for PUCCH format 3

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x1 | 3.95 |
| 2 | 15 | 5 | 1x2 | -2.28 |
| 4 | 30 | 10 | 1x1 | 7.54 |
| 5 | 30 | 10 | 1x2 | 0.00 |

PUCCH format 4

Here we provide the initial simulation results for PUCCH format 4 for alignment, as shown in Table 3.5-2, based on simulation assumption as shown in Table 3.5-1.

Table 3.5-1 Simulation assumption for PUCCH format 4

| Parameter | Value |
| --- | --- |
| PUCCH format | 4 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DMRS |
| Channel model | NTN-TDLA100-200 |
| Test metric | UCI block error probability<1% |

Table 3.5-2 Ideal simulation results for PUCCH format 4

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x1 | 21.25 |
| 2 | 15 | 5 | 1x2 | 1.94 |
| 3 | 30 | 10 | 1x1 | 16.79 |
| 4 | 30 | 10 | 1x2 | 2.91 |

PUCCH format 1 (multi-slot)

Here we provide the initial simulation results for PUCCH format 1 (multi-slot) for alignment, as shown in Table 3.6-2, based on simulation assumption as shown in Table 3.6-1.

Table 3.6-1 Simulation assumption for PUCCH format 1 (multi-slot)

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 3.6-2 Ideal simulation results for PUCCH format 1 (multi-slot)

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ACK missed detection | NACK to ACK |
| 1 | 15 | 5 | 1x1 | -4.15 | -2.27 |
| 2 | 15 | 5 | 1x2 | -10.26 | -9.08 |
| 3 | 30 | 10 | 1x1 | -3.57 | -0.20 |
| 4 | 30 | 10 | 1x2 | -9.88 | -7.18 |

Conclusion

In this contribution, we discuss and provide our initial simulation results on NTN PUCCH demodulation requirements.

Reference

R4-2210662, WF on NTN SAN demodulation requirements, RAN4#103-e, Huawei, HiSilicon