3GPP TSG-RAN WG4 Meeting #104-e	R4-2214394

Electronic Meeting, 15 ‒ 26 August 2022

Title: 	WF on RedCap UE demodulation and CQI reporting requirements

Agenda Item:	9.18.6

Source: 	Ericsson

Document for:	Approval

# Topic #1: General topics for UE demodulation and CSI reporting requirements

## Sub-topic 1-1: Specification structure

Agreement: Introduce UE demodulation and CSI reporting requirements for 2Rx RedCap UE supporting HD-FDD in FDD bands as well as 1Rx UE

- The requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements.

  - Interested companies can evaluate if the same requirements can be applied for both 2Rx HD-FDD and 2Rx FD-FDD.

Agreement: Section names for RedCap UE demodulation and CSI reporting requirements

- If new section is added for RedCap performance requirements, put ‘for RedCap’ (not ‘for RedCap UE’) for the section name

Agreement: Applicable FR2 bands for RedCap UE

- In FR2 bands, specify RedCap UE demodulation and CSI reporting requirements for FR2-1 only

# Topic #2: UE demodulation requirements

## Sub-topic 2-1: PDSCH demodulation requirements

Agreement: Define 256QAM demodulation requirements for 1Rx RedCap UE in FR1 or not.

- Define 256QAM demodulation requirements for 1Rx RedCap UE in FR1

  - Set MCS20

# Topic #3: CSI reporting requirements

Agreement: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD

- Configure the following parameters for CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD:

  - CSI-RS periodicity and offset: 10/1

  - CSI-Report periodicity and offset: 10/9

  - CQI/RI/PMI delay: [14ms]

  - Interested companies are encouraged to evaluate the performance difference between CQI delay 14ms and 10ms in RAN4#104-bis-e. If significant performance degradation is observed compared with CQI delay 10ms, RAN4 will revisit the CQI/RI/PMI delay

    - If CQI delay 10ms is applied, consider CSI periodicity and offset is set to 10/5.

Agreement: Lower test points for CQI reporting test in fading condition for 2Rx (FR1 FDD and TDD)

- Set SNR=6/7dB for lower test points for CQI reporting test in fading condition for 2Rx

Way forward: Static channel matrix used for 1Rx UE and SNR test point offset for CQI reporting tests

- Interested companies are encouraged to evaluate the options for the static channel matrix and SNR test point offset.

  - Option 1: Set the static channel matrix in the frequency domain as H1RX=1j. Set SNR test point X=[0]dB lower than 2Rx test case.

  - Option 2: Keep the previous agreement on the static channel matrix in the frequency domain, that is, H1RX=11. Set SNR test point X=3dB lower than 2Rx test case.

Agreement: Mapping of CQI index to information bit payload

- For RedCap CSI reporting test using CQI table 1, apply the following configuration for mapping of CQI index to information bit payload

  - FR1

    - Reuse value 0 for the overhead parameter for TBS determination

    - Set 52RB for 10MHz/15kHz and 51RB for 20MHz/30kHz

| TBS Scheme |  |  |  | TBS.1-1 | TBS.1-2 | TBS.1-X1 | TBS.1-X2 | TBS.1-X3 | TBS.1-X4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 66 | 66 | 52 | 52 | 51 | 51 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Number of DMRS Res (Note 1) |  |  |  | 24 | 24 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 6 | 6 | 0 | 0 | 0 | 0 |
| Available RE-s |  |  |  | 7590 | 7590 | 6240 | 6240 | 6120 | 6120 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A | N/A | N/A |
| 1 | 0.2344 | 0 | QPSK | 1800 | 3624 | 1480 | 2976 | 1480 | 2856 |
| 2 | 0.2344 | 0 |  | 1800 | 3624 | 1480 | 2976 | 1480 | 2856 |
| 3 | 0.3770 | 2 |  | 2856 | 5640 | 2408 | 4744 | 2408 | 4616 |
| 4 | 0.6016 | 4 |  | 4480 | 8968 | 3752 | 7424 | 3752 | 7296 |
| 5 | 0.8770 | 6 |  | 6528 | 13064 | 5504 | 11016 | 5376 | 10760 |
| 6 | 1.1758 | 8 |  | 8712 | 17928 | 7296 | 14600 | 7168 | 14344 |
| 7 | 1.4766 | 11 | 16QAM | 11016 | 22032 | 9224 | 18432 | 8968 | 17928 |
| 8 | 1.9141 | 13 |  | 14344 | 28680 | 12040 | 24072 | 11784 | 23568 |
| 9 | 2.4063 | 15 |  | 17928 | 35856 | 15112 | 30216 | 14600 | 29192 |
| 10 | 2.7305 | 18 | 64QAM | 20496 | 40976 | 16896 | 33816 | 16896 | 33816 |
| 11 | 3.3223 | 20 |  | 25104 | 50184 | 20496 | 40976 | 20496 | 40976 |
| 12 | 3.9023 | 22 |  | 29192 | 58384 | 24576 | 49176 | 24072 | 48168 |
| 13 | 4.5234 | 24 |  | 33816 | 67584 | 28168 | 56368 | 27656 | 55304 |
| 14 | 5.1152 | 26 |  | 38936 | 77896 | 31752 | 63528 | 31240 | 62504 |
| 15 | 5.5547 | 28 |  | 42016 | 83976 | 34816 | 69672 | 33816 | 67584 |
| Note 1: Number of DMRS Res includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-1 of TS 38.214 [12] Note 5: TBS.1-3, TBS.1-4, TBS.1-5 and TBS.1-6 are applicable to RedCap UE. |  |  |  |  |  |  |  |  |  |

  - FR2

    - Reuse existing test for CQI reporting under static condition in clause 8.2.2.2.1 and corresponding CSI RMC for RedCap UE in FR2-1

Agreement: PMI feedback scheduling pattern (aperiodic CSI reporting) for both FD-FDD and HD-FDD

- Configure the following parameters for PMI reporting tests for both FD-FDD and HD-FDD:

  - CSI request: 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0.

    - Reuse the FRC from Rel-15 PMI test (R.PDSCH 1-6.1 FDD)

  - Aperiodic Report Slot Offset: 3 slots

  - CQI/RI/PMI delay: 6ms

Agreement: Whether to define RI reporting requirements for RedCap 2Rx UEs

- Define RI reporting requirements for RedCap 2Rx UEs

  - Apply Test 2 only

- Replace fading CQI test for 2 Rx UE (high SNR point) by RI test case (Test 2)

# Others

- Interested companies are encouraged to provide both the ideal and impairment simulation results for RedCap UE demodulation and CSI reporting requirements in RAN4#104-bis-e.