3GPP TSG-WG RAN4 Meeting #104-e	Revised : R4-2214880

# Online, 15th-26th August 2022

Title:	Text Proposals  for  TR 38.892 for APT 600MHz NR band

Source:	Spark NZ

Document for:	Approval

Agenda item:	10.19.1

# 1	Introduction

A study item to determine the feasibility of some band plan options in the extended 600 Mhz NR band was approved in RAN#96e meeting [7].

A skeleton TR 38.892  has been provided to this meeting R4-2214879.

This contribution proposes text proposals for various sections of the skeleton TR as per below:

# 2	Text for Section 2 - References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

.

[2]	ETSI EN 300 422-1 (V1.4.2) (2011-08): "Electromagnetic compatibility and Radio spectrum Matters (ERM); Wireless microphones in the 25 MHz to 3 GHz frequency range; Part 1: Technical characteristics and methods of measurement".

[3]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[4]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[5] 		3GPP TR 36 820  , “ LTE for 700 MHz digital dividend’.

[6]		3GPP TR 36 755, “ US 600 Mhz band for LTE”

[7] 		RP 221778 , WID on APT 600 Mhz band

[8]		3GPP  TR 38 860, “ Study on Extended 600 MHz NR band “.

# 3.0 Text for Section 3 - Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

BW	Bandwidth

f	Separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax 	The largest value of f used for defining the requirement

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

f_offset 	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax 	The maximum value of f_offset used for defining the requirement

NDL 	Downlink EARFCN

NOffs-DL 	Offset used for calculating downlink EARFCN

NOffs-UL 	Offset used for calculating uplink EARFCN

NUL 	Uplink EARFCN

PREFSENS	Reference Sensitivity power level

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

BS	Base Station

CBRS	Citizens Broadband Radio Service

CIM	Counter-InterModulation

CW	Continuous Wave

DL	Downlink

DTV	Digital Television

EARFCN 	E-UTRA Absolute Radio Frequency Channel Number

eCFR	electronic Code of Federal Regulation

EIRP	Effective Isotropic Radiated Power

ERP 	Effective Radiated Power

ETSI	European Telecommunications Standards Institute

E-UTRA	Evolved UTRA

FCC	Federal Communications Commission

FDD	Frequency Division Duplex

HAAT	Height Above Average Terrain

LA	Local Area

LO 	Local Oscillator

MOP	Maximum Output Power

MR	Medium Range

MSR	Multi-Standard Radio

NF	Noise Figure

NRAO	National Radio Astronomy Observatory

NSF	National Science Foundation

PA 	Power Amplifier

PRB	Physical Resource Block

RAS	Radio Astronomy Service

REFSENS	Reference Sensitivity

RF	Radio Frequency

RX	Receiver

SEM	Spectrum Emission Mask

TX	Transmitter

UE	User Equipment

UHF	Ultra High Frequency

UL	Uplink

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

VLBA	Very Long Baseline Array

WA	Wide Area

WMTS	Wireless Medical Telemetry Service

# 4.0	Text for Section 4 - Background

Editor’s note: The text in this section is largely from the WID [7]

The 470-694 MHz frequency range is allocated to the broadcasting service and mobile service on a co-primary basis in ITU Region 3. The frequency band 470-698 MHz, or parts thereof, was identified by WRC-15 in 7 countries in Region 3 through new footnote No. 5.296A for use by those administrations as listed wishing to implement terrestrial IMT systems. In addition, there is interest from other significant markets to do the same. Elsewhere, USA, Mexico and several other countries in ITU Region 2 also identified this band for IMT through footnotes 5.295 and 5.308A. It is noted that resolves 2 of revised Resolution 224 (Rev.WRC-19) to encourage administrations to take into account results of the existing relevant ITU Radio communication Sector studies, when implementing IMT applications/systems in the frequency bands 694-862 MHz in Region 1, in the frequency band 470-806 MHz in Region 2, in the frequency band 790-862 MHz in Region 3, in the frequency band 470-698 MHz, or portions thereof, for those administrations mentioned in No. 5.296A, and in the frequency band 698-790 MHz, or portions thereof, for those administrations mentioned in No. 5.313A.

Spectrum below 1 GHz is expectedly well suited for mobile broadband applications.  In particular, the unique propagation characteristics of the bands below 1 GHz allow for wider area coverage, which in turn requires fewer infrastructures and facilitates service delivery to rural or sparsely populated areas. In this regard, the 700MHz ecosystem is growing swiftly: there are over 34 commercial networks deployments. The APT700 band plan coming out from Region 3 played a huge role in its success globally. Outside of APAC, countries in Region 2 have adopted or plan to adopt the APT700 band plan (3GPP band 28) for LTE system deployments. The lower duplexer of APT700 plan has also been adopted for Region 1 since the conclusion of WRC-15.

As the utilisation of the 700MHz spectrum increases over time, it is desirable to look at additional spectrum that could be considered as a companion besides 3GPP Band 28. Therefore, the use of parts of the 600MHz band for the mobile broadband service would provide a vital means of delivering high quality, wide area broadband services including in rural areas and deep inside buildings. The timely availability of frequency arrangements is essential for the development of IMT specifications and standards and the early consideration by Administrations in the footnotes referred to above of suitable frequency arrangements.

The APT region is very diverse and consists of highly developed and developing countries and some with extremely large and rural population base. The sub 1 GHz bands is well suited for the later.

During the last year or so, 3GPP RAN 4 has completed a study item on the feasibility of various duplex filter options for use in this band. The results of this study are documented in TR 38.860 [8] This study was sent to the AWG in an LS RP-212629 in Sep 2021 with a request to provide guidance on a preferred band plan and information on regulatory aspects for the normative work to begin.

The AWG 28 meeting has considered the request of the 3GPP and has provided a response to this LS. In this response the LS has indicated a preference for option B1 (full band)- seer Fig 4-1 below, and has also requested for the work to begin immediately with a view to completion by Dec 2022. Additionally, the answers to the regulatory questions sought by the 3GPP have now been provided via a reply LS RP 221045.

To facilitate the development of additional spectrum in the APT region it is proposed to develop the following band option:

Figure 4-1: APT 600 Mhz band- option B1

https://gsacom.com/paper/gsa-apt700-global-status-regulatory-deployments-devices/