3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215572

Electronic Meeting, 10 – 19 October 2022

Agenda Item:	4.3.4.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposal on suitability of OTA measurement systems on BS conformance testing for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The revised work item for extending current NR operation to 71 GHz was approved at TSG RAN#97-e [1]. One of the objectives of this work item is to define BS conformance testing requirements for the band(s) in the frequency range between 52.6GHz and 71GHz. This topic was discussed at TSG RAN4#103-e and the WF was agreed [2-3].

This contribution discusses the suitability of OTA measurement systems on BS conformance testing for extending current NR operation to 71 GHz based on the agreed WF in [2].

# 2.	Discussion

The following points relating to OTA measurement systems were agreed in [2]:

- General measurement environment/chamber

The suitability of each OTA chamber (Far-field anechoic chamber, CATR, Near-field chamber, PWS, etc.) for each test in the frequency range between 52.6 GHz and 71 GHz should be studied and confirmed by test equipment vendors, or the list of OTA chamber should be updated for each BS type 2-O test in the frequency range between 52.6 GHz and 71 GHz.

It is FFS is any update needed or not in TS 38.141-2 Annex and/or TR 37.941.

- Measurement uncertainty and calibration

  - The measurement capabilities resulting from different test equipment implementations should be studied and provided by test equipment vendors to decide the achievable and satisfactory measurement performance for BS type 2-O testing in the frequency range between 52.6 GHz and 71 GHz.

For BS type 2-O testing, the suitable OTA measurement system set-ups for each transmitter or receiver requirement are provided as informative contents in TS 38.141-2 [4] annexes E1 and E2, respectively. They are summarized in table 1 below.

Table 1: Suitable OTA measurement system set-up for BS type 2-O testing

| Requirements | Indoor Anechoic Chamber | Compact Antenna Test Range | Near Field Test Range | Plane Wave Synthesizer |
| --- | --- | --- | --- | --- |
| Radiated transmit power, OTA output power dynamics, OTA transmitted signal quality, OTA occupied bandwidth, and OTA transmit ON/OFF power | Suitable | Suitable | Suitable | Suitable |
| OTA base station output power, OTA ACLR, OTA operating band unwanted emissions | Suitable | Suitable | Suitable | Suitable |
| OTA spurious emissions | Suitable | Suitable |  |  |
| OTA transmitter intermodulation | Suitable | Suitable |  | Suitable |
| OTA sensitivity and OTA reference sensitivity level | Suitable | Suitable |  | Suitable |
| OTA dynamic range | Suitable | Suitable |  | Suitable |
| OTA adjacent channel selectivity, general OTA blocking, and OTA narrowband blocking | Suitable | Suitable |  | Suitable |
| General OTA out-of-band blocking | Suitable | Suitable |  | Suitable |
| OTA co-location blocking | Suitable | Suitable |  | Suitable |
| OTA receiver spurious emissions | Suitable | Suitable |  |  |
| OTA receiver intermodulation | Suitable | Suitable |  |  |
| OTA in-channel selectivity | Suitable | Suitable |  | Suitable |

There is ongoing study on the required adjustments (in terms of additional components and calibration as well as updates of measurement uncertainties) for using Indoor Anechoic Chamber and Compact Antenna Test Range for BS type 2-O testing in the frequency range between 52.6 GHz and 71 GHz [5-7]. On the other hand, there has been no discussion so far for the use of Near Field Test Range and Plane Wave Synthesizer. Therefore, the suitability of using these OTA measurement system set-ups for BS type 2-O testing in the frequency range between 52.6GHz and 71GHz should be studied and confirmed by TE vendors, or the list of OTA measurement system set-ups should be updated for each BS type 2-O testing in the frequency range between 52.6GHz and 71GHz.

Proposal: The suitability of using Near Field Test Range and Plane Wave Synthesizer for BS type 2-O testing in the frequency range between 52.6GHz and 71GHz should be studied and confirmed by TE vendors, or the list of OTA measurement system set-ups should be updated for each BS type 2-O testing in the frequency range between 52.6GHz and 71GHz.

# 3.	Conclusion

This contribution has discussed the suitability of OTA measurement systems on BS conformance testing for extending current NR operation to 71 GHz based on the agreed WF, with the proposal as follows.

Proposal: The suitability of using Near Field Test Range and Plane Wave Synthesizer for BS type 2-O testing in the frequency range between 52.6GHz and 71GHz should be studied and confirmed by TE vendors, or the list of OTA measurement system set-ups should be updated for each BS type 2-O testing in the frequency range between 52.6GHz and 71GHz.

# References

[1]	RP-222656, “Revised WID: Extending current NR operation to 71 GHz”, Qualcomm.

[2]	R4-2214373, “WF on FR2-2 BS test environments”, Ericsson.

[3]	R4-2214374, “WF on FR2-2 BS conformance testing”, Nokia, Nokia Shanghai Bell.

[4]	3GPP TS 38.141-2 v17.6.0: “NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing”.

[5]	R4-2212465, “On general aspects relevant for conformance testing up to 71 GHz”, Ericsson.

[6]	R4-2213925, “about FR2-2 BS conformance test system, Tx test link budget”, Keysight Technologies.

[7]	R4-2213927, “about FR2-2 BS conformance test system, Rx test link budget”, Keysight Technologies.