3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215578

Electronic Meeting, 10 – 19 October 2022

Agenda Item:	6.7.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on UE RF requirements for FR2-1 UL 256QAM

Document for:	Approval

# 1.	Introduction

The revised work item on NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved at TSG RAN#96 [1]. One of the objectives of this work item is to investigate and enable UL 256QAM for FR2-1.

One of the main tasks to investigate and enable UL 256QAM for FR2-1 is to specify the UE RF requirements. This topic was discussed at TSG RAN4#104-e and the WF was agreed [2]. This contribution provides proposals on UE RF requirements for FR2-1 UL 256QAM according to the agreed WF.

# 2.	Discussion

2.1.	EVM test

It was agreed in the WF [2].

- Option 1: Configuring PTRS for 256QAM EVM testing to compensate CPE.

- The DMRS based channel estimate in the PTRS-ready EVM calculator shall utilize CPE-corrected DMRS symbols.

- The PTRS extraction and correction stage in the PTRS-ready EVM calculator is the final refinement of the received signal.

- FFS on the compensation for Inter Carrier Interference (ICI).

ICI compensation will increase UE implementation complexity and power usage to run the algorithm, and the achievable performance for ICI compensation will depend on the algorithm. It would not be straightforward to agree on the detailed algorithm in TSG RAN4 according to experience. Therefore, it is proposed to consider ICI compensation only if sufficient performance improvement is shown by proponent with explanation of the underlying algorithm.

2.2	PTRS configuration

It was agreed in the WF [2].

- Option 1: PTRS configuration shall be aligned with the UE’s recommended PTRS configuration.

- Option 2: Use a fixed PTRS configuration for all devices

Using different PTRS configurations will result in different EVM measurements in the EVM test for the same device under test. This means that if option 1 is adopted, different EVM requirements will need to be specified for each of the possible PTRS configurations to properly benchmark the EVM performance of the device against other devices using the same PTRS configuration in real-life operation. Moreover, it will increase the TE complexity to support each of the possible PTRS configurations for the EVM and test. Therefore, it is proposed to adopt option 2 to use a fixed PTRS configuration for all devices for the EVM test.

2.3	EVM budget in MPR simulation

It was agreed in the WF [2].

- Option 3: Discuss it after EVM is defined.

One of the main discussion points at TSG RAN4#104-e on the EVM budget was the phase noise assumption. The phase noise for mm-wave frequencies was studied in the NR study phase and corresponding findings were recorded in TR 38.803 [4]. Therefore, it is proposed to use the findings recorded in TR 38.803 on phase noise for mm-wave frequencies as a basis for the phase noise assumption on the EVM budget.

2.4	minimum EIRP

It was agreed in the WF [2].

- Option 3: Discuss it after EVM and operating SNR are defined.

The following observations were made from the simulation results provided in [3]:

Observation 1: For the simulation results at 29 GHz (n257):

-	If MCS index is 21, 256QAM goes over 64QAM in around 24-25 dB for all simulated EVMs.

-	If MCS index is 23, 256QAM goes over 64QAM in around 28 dB for (transmit and receive) EVM of 3%, around 32 dB for (transmit and receive) EVM of 3.5%, and loses for (transmit and receive) EVM of 4%.

Observation 2: For the simulation results at 39 GHz (n260):

-	Only with (transmit and receive) EVM of 0%, MCS21 for 256QAM seems to win 64QAM in around 36 dB.

Therefore, it is proposed to use EVM of 3.5% (current EVM requirements in FR1 for 256QAM) and operating SNR of 32 dB as a basis at 29 GHz (n257).

# 4.	Conclusion

This contribution has provided proposals on UE RF requirements for FR2-1 UL 256QAM according to the agreed WF, they are summarized as follows:

Proposal 1: To consider ICI compensation only if sufficient performance improvement is shown by proponent with explanation of the underlying algorithm.

Proposal 2: To adopt option 2 to use a fixed PTRS configuration for all devices for the EVM test.

Proposal 3: To use the findings recorded in TR 38.803 on phase noise for mm-wave frequencies as a basis for the phase noise assumption on the EVM budget.

Proposal 4: To use EVM of 3.5% (current EVM requirements in FR1 for 256QAM) and operating SNR of 32 dB as a basis at 29 GHz (n257).

# References

[1]	RP-221862, “Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi.

[2]	R4-2214453, “WF on UL 256QAM”, Xiaomi.

[3]	R4-2215577, “Link level simulation results for FR2-1 UL 256QAM”, Nokia, Nokia Shanghai Bell.

[4]	3GPP TR 38.803 v14.3.0: “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.