3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215579

Electronic Meeting, 10 – 19 October 2022

Agenda Item:	6.8.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on consideration of UE architecture for FR2-1 multi-Rx chain DL reception

Document for:	Approval

# 1.	Introduction

The revised work item on Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception was approved at TSG RAN#96 [1]. One of the objectives of this work item is to:

•	Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs

The subject was discussed at TSG RAN4#104-e and the WF was agreed [2]. This contribution provides proposals on consideration of UE architecture for FR2-1 multi-Rx chain DL reception.

# 2.	Discussion

The following points relating to UE architecture assumption were included in [2]:

•	FFS whether the concept of panel should not be explicitly used in core requirements and test configurations.

•	FFS whether the single panel should be excluded.

For spherical coverage requirements in the current specification TS 38.101-2 [3], single-Rx chain architecture has been assumed as shown in figure 1 (left) below where the UE may have multiple panels and the Rx chain may be switched among them to operate a single panel (or a subset of the panels) at the time. This corresponds to single AoA testing for spherical coverage requirements. On the other hand, the objectives of this work item are to address multi-Rx chains with multiple AoAs. A UE with multi-Rx chains is shown in figure 1 (right) below where both Rx chains may be simultaneously connected to any of the UE panels, i.e., two or more panels may be active at the same time.

Figure 1: Single-Rx chain UE (left) vs multi-Rx chain UE (right).

It is also worth noting that in the current specifications for spherical coverage requirements only single beam per panel operation has been assumed. Nonetheless, nowadays UE architectures could also implement multiple beams per panel, i.e., more than one Rx chain may be connected to a single panel. As an example, depending on the UE front-end architecture, a single panel may support:

- Only one Rx chain hence form a dual-polarized beam with maximum gain, e.g., 12 dB gain for a 1x4 linear array, as shown in figure 2 (right) below.

- All two Rx chains on the same panel hence form two dual-polarized beams with lower gain, e.g., 9 dB gain with two dual-polarized 1x2 linear arrays, as shown in figure 2 (left) below.

A UE may exhibit a combination of single-beam and multi-beam panels distributed across the peripheral of the device.

Figure 2: Architecture view: multi-beam operation on one panel (left) versus single-beam operation on one panel (right).

Figure 3: UE Rx antenna patterns view: multi-beam operation on the one panel (left and middle) versus single-beam operation on one panel (right).

In comparison of the two implementations shown in figures 2 and 3, single-beam operation offers higher beamforming gain while multi-beam operation may provide lower beamforming gain (due to splitting the array into two sub-arrays). Furthermore, inter-beam-interference may arise when two beams are received simultaneously, and the achievable spherical coverage may vary depending on the single-beam vs multi-beam operation on one panel.

Since requirements in RAN4 specifications should in general be implementation agnostic, it is proposed to consider both single-beam and multi-beam operations on one or more panels in defining the RF requirements for FR2-1 UE multi-Rx chain DL reception. Consequently, it is proposed that the concept of panel should not be explicitly used in core requirements and test configurations, and the single panel with single-beam operation or multi-beam operation should not be excluded for FR2-1 UE multi-Rx chain DL reception.

# 3.	Conclusion

This contribution has provided proposals on consideration of UE architecture for FR2-1 multi-Rx chain DL reception. They are summarized as follows:

Proposal 1: Consider both single-beam and multi-beam operations on one or more panels in defining the RF requirements and test for FR2-1 UE multi-Rx chain DL reception.

Proposal 2: The concept of panel should not be explicitly used in core requirements and test configurations for FR2-1 UE multi-Rx chain DL reception.

Proposal 3: The single panel with single-beam operation or multi-beam operation should not be excluded for FR2-1 UE multi-Rx chain DL reception.

# References

[1]	RP-221753, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated.

[2]	R4-2214457, “WF on multi-Rx chain DL reception”, Sony.

[3]	3GPP TS 38.101-2 v17.6.0: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.