3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216767

Electronic meeting, October 10th – 19th 2022

Agenda Item:	7.5.6

Source:	Ericsson

Title:	Discussions on NTN IoT RRM requirements

Document for:	Discussion, Decision

# 1	Introduction

During RAN4#104-e, RAN4 had initial discussions to identify the RRM impact of introducing requirements for IoT NTN and good progress wade made. The summary of the discussions was summarized in [1].

In this contribution we further discuss and provide our view on the open issues identified at last meeting and other issues necessary to complete the work item.

# 2	Discussion

## 2.1	General

## 2.2	IDLE mode Mobility

Following agreement was reached regarding availability of valid information for the neighbour/target cell [1]:

#### Issue 1-3-2: information for the neighbor/target cell - Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbour/target cell is made available to the UE.

As this agreement affects RAN2, it is suggested to inform RAN2 to ensure that valid information about the neighbour/target cells are made available at the UE for performing the IDLE/INACTIVE mode procedures.

- RAN4 to inform RAN2 about RAN4 agreement that RAN4 assumes valid information about neighbour cells are available at the UE to perform the neighbour cell measurements.

The open issue related to the exact measurement capability of NGSO satellites is shown below:

#### __Issue 1-3-4: measurement capability on number of NGSO satellites - FFS For both NB and M1 in NGSO, the number of target satellites UE needs to monitor [per carrier] is 2 including serving LEO satellite. - FFS whether to introduce UE capability for the number of target satellites the UE can monitor per carrier including serving LEO satellite, which can be up to 4

The measurement capability in Rel-17 NTN WI is that the NTN UE monitors 4 satellites including LEO. However, we understand that the IoT UEs are typically low complexity UEs compared to legacy UEs and hence may not have the same capability to monitor the same number of satellites as normal NR NTN UE. Therefore, we can compromise to the option that UE monitors 2 satellites including a LEO satellite.

- For both NB-IoT and eMTC UEs in NGSO, the number of target satellites UE needs to monitor per carrier is 2 including serving LEO satellite.

#### __Issue 1-3-5: For NGSO, Doppler shift impact in Multiple NGSO satellites - RAN4 shall investigate Doppler shift impact on the overlapping receiving window from multiple intra-frequency and multiple inter-frequency satellites on IoT NTN - Except NB-IoT in Connected mode - FFS the following proposals - the scaling factor for measurement of multiple LEO satellites should also apply - For eMTC over NTN, RAN4 not to define scheduling restriction due to RRM measurement

Doppler frequency in IoT NTN may be rather large depending on the satellite deployment scenario. The Doppler frequencies corresponding to different satellite deployment scenario are listed below in table 1 from TR 38.811 v15.4.0. The maximal Doppler shift is 48KHz in LEO case. The Doppler frequency is therefore much more critical issue compared to legacy TN system.

Table 1: Summary of Doppler shift and shift variation for different altitudes Frequency (GHz); Max Doppler; Relative Doppler; Max Doppler shift variation. 2; +/- 48 kHz; 0.0024 %; - 544 Hz/s; LEO at 600 km altitude. 20; +/- 480 kHz; 0.0024 %; -5.44 kHz/s. 30; +/- 720 kHz; 0.0024 %; -8.16 kHz/s. 2; +/- 40 kHz; 0.002 %; -180 Hz/s; LEO at 1500 km altitude. 20; +/- 400 kHz; 0.002 %; -1.8 kHz/s. 30; +/- 600 kHz; 0.002 %; -2.7 kHz/s. 2; +/- 15 kHz; 0.00075 %; -6 Hz/s; MEO at 10000 km altitude. 20; +/- 150 kHz; 0.00075 %; -60 Hz/s. 30; +/- 225 kHz; 0.00075 %; -90 Hz/s.

Meanwhile, as per IoT NTN characteristics, e.g. 100/200KHz channel raster and 200KHz BW for NB-IoT, if two satellites face opposite Doppler shifts, i.e. +48KHz and -48KHz, the relative frequency offset between the two satellites due to Doppler shift is 96KHz in worst case. It occupies almost half of NB-IoT’s BW. To measure the two satellites, assuming their transmitting frequency is same, e.g. intra-frequency, the 200+96=296KHz reception window is unacceptable to all UEs and out-of-band emission requirements, it may need RF retuning action depending on IoT device’s implementation.

Figure 1 Doppler shift

One of obvious impact to RRM requirements is intra-frequency measurements. For neighbor cells which have strong Doppler shift to serving satellite, the intra-frequency measurements may need more time. The relaxation of measurement time can refer to measurement samples in inter-frequency measurements.

- Intra-frequency measurements (idle and connected modes) requirements shall be relaxed for neighbor cells which have strong Doppler shift to serving satellite.

- Intra-frequency measurements (idle and connected modes) requirements on neighbor cells which have strong Doppler shift to serving satellite can refer to measurement samples inter-frequency measurements.

DRX/eDRX applicability

Impact on long DRX cycles and eDRX was discussed at last meeting with following outcome [1]:

#### __Issue 1-3-6: DRX/eDRX, HD-FDD applicability - FFS the applicability of DRX/eDRX cycle length and PTW length for LEO due to change in satellite coverage (t-Service). - The existing TN requirements can be used as a starting point.

It shall be noted that the coverage of a cell served by a LEO satellite can vary significantly compared to the coverage of a cell served by a GEO satellite. The information about the serving cell coverage is provided to the UE and UE knows based on this information when the coverage will be impacted (e.g. when in time the cell will disappear). This follows the Rel-17 NTN UE behavior. One important difference between Rel-17 NTN work and the Rel-18 LTE IoT NTN work is that the latter supports also operation with long DRX cycles, more specifically extended DRX cycles with PTW. Taking into consideration that the measurement delays are typically longer for IoT compared to legacy UEs, it is important to allow the IoT UEs sufficient time to perform or complete a measurement (e.g. cell detection or cell measurement) before the coverage of that cell is going to be impacted. Therefore the UE shall not apply the eDRX cycles or long DRX cycles when the coverage of the satellites is about to disappear e.g. when the current time is close to or equal to ‘t-Service’. This may also apply to the serving cell evaluation which would require the UE to evaluate the serving cell using shorter eDRX cycles.

Likewise, then the coverage of the serving cell is resumed after ‘t-ServiceStart’, the UE may need some additional time to reacquire the synchronization. Unlike the the TN operation where the UE can wake up before the ON duration/active time to perform T/F tracking, the NTN IoT UE can only perform the sync after ‘t-ServiceStart’. Therefore it is important to assume shorter DRX cycle to speed up the evaluation after ‘t-ServiceStart’. For this reason, the UE can also be allowed to drop the paging during the [2] DRX cycles immediately after ‘t-ServiceStart’.

- If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_cycle, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s starting from at least [1] eDRX cycle before ‘t-Service-r17’.

- If the UE is not configured with eDRX_IDLE cycle and configured with DRX cycle ≥ [1.28] s then the UE shall meet the requirements defined for DRX cycle of [640] ms during at least [2] configured DRX cycles immediately after ‘t-ServiceStart-r17’ [2].

- If the UE is configured with eDRX_IDLE cycle then the UE shall meet the requirements defined for eDRX_IDLE cycle of 5.12 s during [2] eDRX_IDLE cycles immediately after ‘t-ServiceStart-r17’.

- The UE is allowed to drop paging during [2] DRX cycles immediately after ‘t-ServiceStart-r17’.

New UE capability

RAN4 discussed whether to introduce new UE capabilities for enhanced RRM requirements in IDLE/INACTIVE, relaxed cell reselection on GEO and parallel measurements on multiple LEO satellites. These are capabilities that were introduced in Rel-17 NR NTN.

#### __Issue 1-5: UE capability introduced in R17 NR NTN - FFS whether to introduce the following UE capability, for LTE IoT NTN in Rel-18 - Enhanced RRM requirements for measurements in IDLE and INACTIVE modes - Relaxed cell reselection on GEO - Parallel measurements on multiple LEO satellites

Our general view is that the IoT NTN UE requirements can be based on the minimum UE capabilities which do no have the capabilities to perform the above tasks. However, it is still beneficial to introduce capabilities, following the Rel-17 NTN approach, for more advanced UEs similar to cat-1bis in LTE. This means, in practice the UEs which do have the additional capabilities to perform enhanced RRM requirements or parallel measurements can meet the requirements associated with those. Other UEs can meet the basic requirements.

- New UE capabilities (similar to Rel-17 NTN) for IoT NTN for following:

- Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

- Relaxed cell reselection on GEO

- Parallel measurements on multiple LEO satellites

Discontinuous coverage

Regarding the discussions on discontinuous coverage, it was agreed that the UE requirements are defined based on the assumption that UE is able to predict the coverage of the cell and that the UE in IDLE or PSM mode is not required to perform any cell search while in out of coverage in discontinuous coverage. However, whether and how to define requirements for these cases remains open. Our view is to follow the Rel-17 NTN approach for defining the requirements for discontinuous coverage, where the requirements are defined based on current UE time (T1) and ‘t-Service-r17’ and ‘t-Service-start-r17 ’.

Rationally, the UE shall initiate cell (re)selection procedure at least when ‘t-Service-start-r17’. At that moment and during a time period, e.g. Tdelta in below Figure 2, UE shall identify and measurement on the new serving cell as soon as possible. Another issue is paging reception could not be kept in Tdelta and cause dropping paging.

Figure 2 Discontinuous coverage

The requirements are introduced based on ‘t-Service’ and ‘t-Service-start-r17’in our companion contribution containing the change request [3].

Regarding the discussions on discontinuous coverage, it was agreed that the UE requirements are defined based on the assumption that UE is able to predict the coverage of the cell and that the UE in IDLE or PSM mode is not required to perform any cell search while in out of coverage in discontinuous coverage. However, whether and how to define requirements for these cases remains open. Our view is to follow the Rel-17 NTN approach for defining the requirements for discontinuous coverage, where the requirements are defined based on current UE time (T1) and ‘t-Service and ‘t-Service-start ’. The requirements are introduced based on ‘t-Service’ in our companion contribution containing the change request.

- Discontinuous coverage requirements are defined based on at least ‘t-Service-r17’ indicating satellite coverage (cell stop time) and ‘t-Service-start-r17 ’.

Cell re-selection in NGSO

Based on the last meeting discussions, following issues were identified with regard to how to define the UE requirements [1]:

#### __Issue 2-2-1b: NGSO, cell re-selection - RAN4 to discuss which eDRX/DRX cycles to exclude due to the impact of Tservice (cell stop time) for serving cell and neighbour cell measurements in LEO. - FFS whether to scaled up the existing TN delay requirements by KSatellite, where KSatellite is the number NGSO satellites. - FFS: In NGSO, the cat-M/NB UE in IDLE mode is allowed to meet the relaxed neighbour cell measurement requirements provided that the UE has met the relaxed monitoring criterion and the serving cell is not going to stop serving the area in the next L1 number of DRX cycles; L1 is FFS. - FFS: In NGSO, the cat-M/NB UE in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the relaxed monitoring criterion and the serving cell is not going to stop serving the area in the next L2 number of DRX cycles; L2 is FFS.

The first issue is related to whether to exclude certain eDRX/DRX cycles due to impact of Tservice. This issue is overlapping with the topic on DRX/eDRX applicability in section 2.2 above. Therefore we propose to consider those proposal.

The second issue is related to whether to scale the existing TN delay requirements with a satellite specific scaling factor KSatellite. It shall be noted that IoT NTN UEs already support relaxed measurements, e.g. UE can perform relaxed serving cell and relaxed neighbour cell measurements based on fulfillment of relaxation criteria. Therefore there is no need to scale the existing TN requirements with a satellite specific factor.

Observation 1	IoT UEs in LTE already support relaxed serving and neighbour cell measurements.

- No need to scale existing TN requirements with KSatellite.

Third issue is related to DRX/eDRX discussion which are discussed in section 2.2 above. For the reasons explained above, i.e. due to that the coverage of a NGSO cell is impacted because of satellite movement, UE shall meet the relaxed measurement requirements only when the received information (based on t-service) indicates the UE will be in-coverage at least in the next L1 number of DRX cycles. Otherwise, if the received information indicate that the current cell will disappear within next L1 number of DRX cycles then the UE shall not enter relaxed mode. For NB-IoT UE is configured with DRX cycles, L1 can be set to 4 DRX cycles since serving cell evaluations are based on 4 DRX cycles in enhanced coverage. For NB-IoT UE configured  with eDRX cycles, L1 can be set to at least one eDRX cycle. For similar reason, L1 is set to 4 (assuming DRX cycles of 2.56 sec) and L1 is set to 1 eDRX cycle for eMTC UE.

- In NGSO, the cat-M/NB UE configured with ‘t-Service’ in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the existing relaxation conditions and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [4] DRX cycles before ‘t-Service-r17’ if not configured with eDRX_IDLE cycle.

- In NGSO, the cat-M/NB UE configured with ‘t-Service’ in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the existing relaxation conditions and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [1] DRX cycles before ‘t-Service-r17’ if configured with eDRX_IDLE cycle.

Relaxed cell measurements in IDLE mode

In eMTC TN requirements, the UE is allowed to meet relaxed serving cell measurement requirements which is derived based on a formula that depends on the configured DRX or extended DRX (eDRX) cycle, PTW length, and a signaled parameter num-DRX-CyclesRelaxed by the network as shown Table 1 and 2 below [2]:

Table 1: The relaxation factor N for a UE not configured with eDRX IDLE cycle

| DRX cycle length [s] | Value |
| --- | --- |
| 0.32 | Min(n , 32) |
| 0.64 | Min(n , 16) |
| 1.28 | Min(n , 8) |
| 2.56 | Min(n , 4) |
| NOTE: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |

Table 2: The relaxation factor N for a UE configured with eDRX IDLE cycle

| DRX cycle length [s] | Value |  |  |
| --- | --- | --- | --- |
|  | 1.28 ≤ PTW length [s] < 2.56 | 2.56 ≤ PTW length [s] < 5.12 | 5.12 ≤ PTW length [s] |
| 0.32 | Min(n , 2) | Min(n , 4) | Min(n , 8) |
| 0.64 | 1 | Min(n , 2) | Min(n , 4) |
| 1.28 | N/A | 1 | Min(n , 2) |
| 2.56 | N/A | N/A | 1 |
| NOTE: n is signalled by the network by using num-DRX-CyclesRelaxed defined in TS 36.331 [2]. |  |  |  |

The fixed or static measurement relaxation factors were designed for a fixed terrestrial network where the base stations are stationary, i.e. the coverage of cell is rather static. However, the fixed relaxation factors may not work well for a non-terrestrial network where the satellites serving the cells can be moving and thus the cell coverage can be dynamic. Moreover, the LEO and GEO satellites also differ in terms of their speed, their altitudes, their orbits etc. The coverage of the NTN cells served by LEO satellites may be quite dynamic compared to the coverage of a cell sered by a GEO satellite. This means that the coverage of the cell may disappear for some time, or degraded for some time as the satellite moves away wrt the UE location which is relatively fixed. Therefore, the existing formula/requirements for deriving the relaxation factor can be reused for cat-M when served by a GEO satellite. However, the level of relaxation shall be reduced (e.g. by factor 2) when served by a LEO satellite.

- In NGSO, the serving cell measurement relaxation factor is reused from corresponding TN requirements for eMTC and NB-IOT in IDLE mode when cell is served by a GEO satellite.

- In NGSO, the serving cell measurement relaxation factor is reduced by factor N from corresponding TN requirements for eMTC and NB-IOT in IDLE mode when cell is served by a LEO satellite compared to corresponding GEO satellite, where N=[2].

Moreover, as an additional safety-mechanism to avoid that a UE served by a LEO satellite mistakenly enters into a relaxed mode and to prepare for a potential cell change, UE shall meet the relaxed requirements if it has found at least one additional satellite on the serving carrier including the serving satellite.

- The eMTC and NB-IoT UE is allowed to meet the relaxed neighbour cell requirements provided that it has found more than [1] satellite including the serving satellite.

Cell re-selection in enhanced coverage, NGSO

RAN4 discussed whether the existing enhanced coverage requirements (e.g. intra/inter frequency) can be reused for NB-IoT and eMTC without following outcome [1]:

#### __Issue 2-2-4: NGSO, cell Re-selection in Enhanced Coverage - For Enhanced Coverage intra-/inter-frequency measurement, the existing TN requirement on Tmeasure, Tevaluate can be the baseline. - The exact values are FFS. - FFS the cell detection time (Tdetect).

Our view is that the current enhanced coverage delay requirements can be reused since no major impact was identified based on the link budget calculations. For similar reason, also the channel quality reporting defined for 1.4 MHz BW requirements and WUS reception requirements from TN can be reused for eMTC and NB-IoT NTN.

- For Enhanced coverage, the current intra- and inter-frequency measurement requirements (Tdetect, Tmeasure, Tevaluate) from TN can be reused for IoT NTN for eMTC and NB-IoT.

- For eMTC in LEO, the channel quality reporting requirements (defined for 1.4 MHz) from TN in IDLE mode are reused.

- For eMTC and NB-IOT, the WUS reception requirements from TN requirements are reused.

Transmission using preconfigured uplink resources (PUR)

The eMTC and NB-IoT UE can be configured with PUR resources along with a TA command. The UE has to update the uplink timing for transmitting on PUR according to TS 36.211 v17.2.0 i.e. transmission of uplink radio frame number  from the UE starts TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs seconds before start of the corresponding downlink radio frame at the UE. The parameters (NTA,adjcommon  and NTA,adjUE) are directly or indirectly received by the UE from the network i.e. in SIB31 which contains satellite assistance information (SAI) for the serving cell. Upon expiry of the SAI the TA should be considered invalid by the UE and UE should not transmit on PUR. The SAI expires upon expiry of T317 which is started upon the reception of SAI.

In addition, the UE should only perform the PUR provided that the serving cell is not going to stop serving the area it is currently covering until the next PUR occasion. Otherwise, the UE may assume the TA as invalid and not carry out the PUR transmissions e.g. TA is invalid when the current time is close to, equal to or greater than ‘t-Service’.

- RAN4 shall not reuse the legacy RSRP-based TA validation for PUR in IoT NTN.

- The UE update the uplink timing for transmitting on PUR using the configured TA command according to TS 36.211 v17.2.0 i.e. transmission of uplink radio frame number  from the UE starts TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs

- The UE assumes TA is valid provided that the following conditions are met, otherwise the TA is considered invalid:

      - Satellite assistance information (SAI) is valid i.e. T317 has not expired and

      - Current time of the UE is at least DT seconds earlier than t-Service, where DT is the configured PUR periodicity.

## 2.3	CONNECTED mode Mobility

RRC re-establishment, RRC connection release with redirection and random access

For the CONNECTED mode mobility procedures RRC re-establishment, RRC connection release with redirection and random access, our view is that the existing requirements from TN can be reused. We have not identified as NTN specific impact, and this is also in line with the Rel-17 NTN requirements.

- Existing TN RRC connection release with redirection requirements are reused for NTN IoT.

- Existing TN random access requirements are reused for NTN IoT.RAN4 shall not reuse the legacy RSRP-based TA validation for PUR in IoT NTN.

- Existing TN RRC re-establishment requirements are reused for NTN IoT.

In RRC_CONNECTED state, the network maintains the connection between the serving cell and the UE, e.g. by collecting measurements, and providing the UE with updated configuration when needed.

In NTN scenario, the network may lose the ability to maintain the connection between the serving cell and the UE, e.g. due to bad coverage or a failed handover. Under such circumstances, the UE may be trapped in a non-reachable state since the network cannot contact the UE to restore the communication link. In these cases, it may be advantageous for the UE to reset the RRC connection and restart the communication.

Timing based CHO in NTN facilitates mobility to avoid interruption due to satellites’ s expiry or missing.  Since no connected mode mobility in IoT, instead RRC re-establishment undertakes cell change function in connected mode, it’s feasible to take RRC re-establishment before expiry of serving cell coverage (Tservice).

- RRC re-establishment shall be started before or at the least at the time instance of expiry of serving cell coverage (‘t-Service’).

The legacy RRC connection redirection to non-anchor carrier for NB-IoT is defined under the conditions that the anchor and non-anchor carriers are within 20 MHz and are served by the same BS or by the co-located BSs. In the legacy RRC connection redirection to non-anchor carrier delay, the UE switches to the non-anchor carrier as indicated in the IE DL-Bitmap-NB to acquire the UL grant from the target non-anchor carrier and use the UL grant to send NPUSCH in the target non-anchor carrier. In our view the same principle can be reused for RRC connection redirection to non-anchor carrier for satellite access. Furthermore, the requirements can be applied only when the anchor carrier and the target non-anchor carrier are served by the same satellite access nodes.

- The legacy RRC redirection to non-anchor carrier delay requirements for NB-IoT can be reused for RRC redirection to non-anchor carrier delay requirements for NB-IoT with satellite access.

- The RRC redirection to non-anchor carrier delay requirements for NB-IoT with satellite are applicable provided that the anchor and the target non-anchor carriers are served by the same satellite access node and frequencies of the anchor and the target non-anchor carriers are within 20 MHz.

The legacy RRC connection release with redirection procedure for UE category M1 is defined for CE Mode A. It requires the UE to identify the target cell and acquire the SI (if not provided by the serving cell) in the RRC release message before sending the RA to the target cell. In our view the same principle can be reused for redirection procedure for UE category M1 for satellite access. The requirements should apply only for UE category M1 supporting FDD or HD-FDD operation. Furthermore, same requirements can be applied regardless of whether the serving and the target cells are served by the same or by different satellite access nodes.

- The legacy RRC connection release with redirection delay requirements for UE category M1 in CE Mode A can be reused for RRC connection release with redirection delay requirements for UE category M1 in CE Mode A with satellite access.

- The same RRC connection release with redirection delay requirements can be applied regardless of whether the serving and the target cells are served by the same or by different satellite access nodes

Handover

Whether to support conditional handover for category M1 UEs remain open as follows [1]:

#### __Issue 3-4: M1, CHO requirements - Option 1: define CHO requirements for M1 in NTN - FFS no need to consider time or location based CHO - Option 2: CHO requirements for M1 should be introduced first in the Terrestrial IoT

It is recalled that requirements for conditional handover is currently missing for eMTC in TN networks. Whether to define CHO requirements for eMTC in NTN remains open. Conditional handover can be based on RSRP, time and location information. Given that IoT NTN can be operated by different types of satellites that have different characteristics, timing and/or location-based CHO handover can be beneficial.

- RAN4 to define the CHO requirements base eMTC in Rel-18 IoT NTN.

## 2.4	Timing and RLM requirements

Timing requirements

#### __Issue 4-1-1: UE transmit timing (Te) requirement - The reference time point for Te_NTN - Option 1: The reference point for Te_NTN should be the downlink timing of the reference cell minus (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Ts - Option 2: The reference point for Te should be the downlink timing of the reference cell minus the UE UL timing. FFS the UE UL timing. - Whether to relaxed Te by considering GNSS estimation accuracy - Option 1: no - Option 2: yes - FSS whether and how to relax the GNSS estimation accuracy #### __Issue 4-1-2: Gradual timing adjustment - FFS For gradual timing adjustment, the reference timing shall be (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Ts before the downlink timing of the reference cell. Clarify the adjustment with “apart from a change of NTA,UE-specific and NTA,common” - FFS the gradual timing adjustment figures (max step size, max, min rate etc) - FFS the impact of “segmented UL transmission

Regarding the first issue on reference point for Te_NTN, option 1 is our preferred option as it is more concrete and more specific reference time is defined.

- The reference point for Te_NTN should be the downlink timing of the reference cell minus (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Ts.

Regarding the open issue related to GNSS estimation accuracy, it is noted that the Te was relaxed due to UE and satellites positioning error in Rel-17 NR NTN. It is therefore reasonable to reuse the same agreement for IoT NTN.

- The agreement from Rel-17 NR NTN on relaxed Te due to GNSS estimation accuracy is reused.

RLM requirements

At last meeting it was agreed to use the existing RLM requirements for eMTC and NB-IoT assuming that the cell is managed by a GEO satellite. But for a cell managed by a LEO satellite, following open issues remain [1]:

#### __Issue 4-4: RLM - For NB NGSO, FFS the impact of satellite assistance information validity and t-Service - For M1 NGSO, FFS the impact of satellite assistance information validity and t-Service - FFS the following proposal - Option 1: The UE in DRX shall evaluate the RLM according non-DRX requirements provided that the following conditions are met, otherwise the UE is allowed to evaluate following DRX requirements: - Satellite assistance information (SAI) is valid i.e. T317 has not expired and - Current time of the UE is at least DT seconds earlier than t-Service.

As described in section 2.2, the coverage of a cell served by a NTN satellite can be impacted (disappear) based on type of satellite (e.g. LEO). The coverage information of the serving cell is already provided to the UE as agreed in Rel-17 NTN work. Our view is that that the UE shall evaluate the RLM according to non-DRX requirements when the when the coverage of the satellites is about to disappear e.g. when the current time is close to or equal to ‘t-Service’. Similar to the discussions on neighbour cell measurements in section 2.2, the UE configured with DRX can continue to evaluate RLM based on DRX requirements if at least T is left until the expiry of the current cell. Otherwise, the UE shall evaluate following the non-DRX requirements. T can be set to the minimum evaluation period expressed in term of DRX cycles which is 5 for eMTC and 10 for NB-IoT assuming longest DRX cycles.

This allows the UE to complete an ongoing RLM evaluation or new evaluations before the UE is going to out of coverage wrt to that serving cell.

- If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and DRX_cycle, then at least during [5] DRX cycles before t-Service-r17, the UE shall meet the non-DRX requirements.

- If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_cycle, then at least during [1] eDRX cycle before t-Service-r17, the UE shall meet the non-DRX requirements.

## 2.4	CONNECTED mode measurement requirements

Measurement requirements

The CONNECTED mode measurement requirements were discussed with following outcome [1]:

- FFS the following proposals - For M1 in GEO, the existing M1 TN intra frequency measurement requirements apply, as in 8.13.2.1 for CE mode A and 8.13.3.1 for CE mode B - For M1 in GEO, the existing M1 TN inter frequency requirements apply, as in 8.13.2.6 for CE mode A and 8.13.3.5 for CE mode B. - For M1 in NGSO, the delay requirements are scaled up by the number NGSO satellites.

Similar to the earlier discussions and proposals for the CONNECTED mode mobility procedures above, our view is that the existing intra-frequency and inter-frequency measurements requirements in CE mode A and B can be reused for eMTC when the measured is managed by a GEO satellite. For NGSO, there is no need to scale the existing TN requirements with the number of satellites since IoT requirements are already relaxed compared to corresponding requirements with 2 Rx. Therefore there is no need to scale the existing TN requirements with a satellite specific factor.

- For M1 in GEO, the existing M1 TN intra frequency measurement requirements apply,   as in 8.13.2.1 for CE mode A and 8.13.3.1 for CE mode B

- For M1 in GEO, the existing M1 TN inter frequency requirements apply, as in 8.13.2.6 for CE mode A and 8.13.3.5 for CE mode B.

- For M1 in NGSO, no need to scale the delay requirements with the number NGSO satellites.

Regarding the requirements on channel quality reporting, our corresponding proposal from IDLE mode is applicable also for CONNECTED mode. This means the corresponding TN requirements can be reused for CONNECTED.

- For eMTC in LEO, the channel quality reporting requirements (defined for 1.4 MHz) from TN in CONNECTED mode are reused.

# Conclusion

In this contribution we have discussed and provided our view on the RRM issues identified in the way forward document at last meeting. More specifically, we have discussed RRM requirements for the different types of mobility procedures in RRC IDLE and CONNECTED mode and also provided views on additional topics. Based on the discussions, we have made following proposals and observations:

- RAN4 to inform RAN2 about RAN4 agreement that RAN4 assumes valid information about neighbour cells are available at the UE to perform the neighbour cell measurements.

- For both NB-IoT and eMTC UEs in NGSO, the number of target satellites UE needs to monitor per carrier is 2 including serving LEO satellite.

- Intra-frequency measurements (idle and connected modes) requirements shall be relaxed for neighbor cells which have strong Doppler shift to serving satellite.

- Intra-frequency measurements (idle and connected modes) requirements on neighbor cells which have strong Doppler shift to serving satellite can refer to measurement samples inter-frequency measurements.

- If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_cycle, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s starting from at least [1] eDRX cycle before ‘t-Service-r17’.

- If the UE is not configured with eDRX_IDLE cycle and configured with DRX cycle ≥ [1.28] s then the UE shall meet the requirements defined for DRX cycle of [640] ms during at least [2] configured DRX cycles immediately after ‘t-ServiceStart-r17’ [2].

- If the UE is configured with eDRX_IDLE cycle then the UE shall meet the requirements defined for eDRX_IDLE cycle of 5.12 s during [2] eDRX_IDLE cycles immediately after ‘t-ServiceStart-r17’.

- The UE is allowed to drop paging during [2] DRX cycles immediately after ‘t-ServiceStart-r17’.

- New UE capabilities (similar to Rel-17 NTN) for IoT NTN for following:

- Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

- Relaxed cell reselection on GEO

- Parallel measurements on multiple LEO satellites

- Discontinuous coverage requirements are defined based on at least ‘t-Service-r17’ indicating satellite coverage (cell stop time) and ‘t-Service-start-r17 ’.

Observation 1	IoT UEs in LTE already support relaxed serving and neighbour cell measurements.

- No need to scale existing TN requirements with KSatellite.

- In NGSO, the cat-M/NB UE configured with ‘t-Service’ in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the existing relaxation conditions and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [4] DRX cycles before ‘t-Service-r17’ if not configured with eDRX_IDLE cycle.

- In NGSO, the cat-M/NB UE configured with ‘t-Service’ in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the existing relaxation conditions and the serving cell is not going to stop serving the area, where the UE is located, at least during the last [1] DRX cycles before ‘t-Service-r17’ if configured with eDRX_IDLE cycle.

- In NGSO, the serving cell measurement relaxation factor is reused from corresponding TN requirements for eMTC and NB-IOT in IDLE mode when cell is served by a GEO satellite.

- In NGSO, the serving cell measurement relaxation factor is reduced by factor N from corresponding TN requirements for eMTC and NB-IOT in IDLE mode when cell is served by a LEO satellite compared to corresponding GEO satellite, where N=[2].

- The eMTC and NB-IoT UE is allowed to meet the relaxed neighbour cell requirements provided that it has found more than [1] satellite including the serving satellite.

- For Enhanced coverage, the current intra- and inter-frequency measurement requirements (Tdetect, Tmeasure, Tevaluate) from TN can be reused for IoT NTN for eMTC and NB-IoT.

- For eMTC in LEO, the channel quality reporting requirements (defined for 1.4 MHz) from TN in IDLE mode are reused.

- For eMTC and NB-IOT, the WUS reception requirements from TN requirements are reused.

- RAN4 shall not reuse the legacy RSRP-based TA validation for PUR in IoT NTN.

- The UE update the uplink timing for transmitting on PUR using the configured TA command according to TS 36.211 v17.2.0 i.e. transmission of uplink radio frame number  from the UE starts TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs

- The UE assumes TA is valid provided that the following conditions are met, otherwise the TA is considered invalid:

                - Satellite assistance information (SAI) is valid i.e. T317 has not expired and

                - Current time of the UE is at least DT seconds earlier than t-Service, where DT is the configured PUR periodicity.

- Existing TN RRC connection release with redirection requirements are reused for NTN IoT.

- Existing TN random access requirements are reused for NTN IoT.RAN4 shall not reuse the legacy RSRP-based TA validation for PUR in IoT NTN.

- Existing TN RRC re-establishment requirements are reused for NTN IoT.

- RRC re-establishment shall be started before or at the least at the time instance of expiry of serving cell coverage (‘t-Service’).

- The legacy RRC redirection to non-anchor carrier delay requirements for NB-IoT can be reused for RRC redirection to non-anchor carrier delay requirements for NB-IoT with satellite access.

- The RRC redirection to non-anchor carrier delay requirements for NB-IoT with satellite are applicable provided that the anchor and the target non-anchor carriers are served by the same satellite access node and frequencies of the anchor and the target non-anchor carriers are within 20 MHz.

- The legacy RRC connection release with redirection delay requirements for UE category M1 in CE Mode A can be reused for RRC connection release with redirection delay requirements for UE category M1 in CE Mode A with satellite access.

- The same RRC connection release with redirection delay requirements can be applied regardless of whether the serving and the target cells are served by the same or by different satellite access nodes

- RAN4 to define the CHO requirements base eMTC in Rel-18 IoT NTN.

- The reference point for Te_NTN should be the downlink timing of the reference cell minus (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Ts.

- The agreement from Rel-17 NR NTN on relaxed Te due to GNSS estimation accuracy is reused.

- If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and DRX_cycle, then at least during [5] DRX cycles before t-Service-r17, the UE shall meet the non-DRX requirements.

- If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_cycle, then at least during [1] eDRX cycle before t-Service-r17, the UE shall meet the non-DRX requirements.

- For M1 in GEO, the existing M1 TN intra frequency measurement requirements apply,   as in 8.13.2.1 for CE mode A and 8.13.3.1 for CE mode B

- For M1 in GEO, the existing M1 TN inter frequency requirements apply, as in 8.13.2.6 for CE mode A and 8.13.3.5 for CE mode B.

- For M1 in NGSO, no need to scale the delay requirements with the number NGSO satellites.

- For eMTC in LEO, the channel quality reporting requirements (defined for 1.4 MHz) from TN in CONNECTED mode are reused.

# References

R4-2214350, " WF on LTE IoT NTN RRM requirements ", Moderator (MediaTek inc.)

TS 36.133 V16.7.0

R4-2216768, “IDLE mode requirements for IoT NTN (cat-M)”, Ericsson