3GPP TSG-RAN WG4 Meeting #105	R4-2218700

Toulouse, France, 14th – 18th November 2022

Agenda item:	6.1.3.3

Source: 		NTT DOCOMO, INC.

Title: 	TP for TS 38.115-2 OTA Out of band gain requirements

Document for:	Approval

- Introduction

In RAN4#104bis-e, MU/TT values for FR2 repeaters were discussed and agreed with []. In this contribution, we provide our views and text proposal on fixing of TT values for OOB Gain for FR2 repeater.

- Discussion

In RAN4#104bis-e, remaining MU/TT values for FR2 NR repeater were discussed and agreed with []. The agreements are as follows:

RAN4#104bis-e [1] - Take below values as baseline with [ ], further feedback from TE vendors are encouraged. Subclause; Maximum Test System Uncertainty and Range over which Test System Uncertainty applies. 7.3 OTA Frequency stability; ±[12] Hz Measurement results of [5000] Hz. 6.4 OTA Out of band gain; ±[2.1] dB, 24.25GHz < f ≦ 29.5GHz ±[2.4] dB, 37GHz < f ≦ 43,5GHz ±[2.6] dB, 43.5GHz < f ≦ 48.2GHz. 7.7 OTA Input intermodulation; ±[2.0] dB, f ≤ 3.0 GHz ±[2.6] dB, 3.0 GHz < f ≤ 4.2 GHz ±[3.2] dB, 4.2 GHz < f ≤ 6.0 GHz. 7.8 OTA Adjacent channel rejection ratio; ±[2.7] dB (24.25 – 29.5 GHz) ±[2.7] dB (37 – 43.5 GHz) ±[2.9] dB (43.5 GHz < f ≤ 48.2 GHz).

Above requirements are basically proposed in [2] and [3]. In terms of OTA OOB gain, the MU/TT values for OTA Output power (TRP) were taken as baseline. In previous meeting, further feedback from TE vendors were encouraged so the values are with []. Regarding time schedule, TS 38.115-2 is to be approved in RAN#98(2022/12). Therefore, RAN4#105 is the last meeting allocated for Rel-17 NR repeaters and we have to conclude test requirements for NR repeaters.

Observation 1: Regarding time schedule, RAN4 have to conclude test requirements for Rel-17 NR repeater in this meeting.

If any critical concerns on MU/TT values from TE vendors and/or manufacturers would not be raised in this meeting, RAN4 can conclude these values for TS38.115-2. The text proposals are shown in Annex A in this paper.

Proposal 1: RAN4 approve the attached TP to TS 38.115-2 related to OTA OOB Gain if any concerns would not be raised.

Regarding Annex C in TS 38.115-2, Table number is described as “Table B.1-1” but it would be better to replace to “Table C.1-1” since the clause where this table exists is Annex C (editorial correction). This editorial correction is also included in attached TP.

- Conclusion

In this contribution, we provide views on TT values for OTA OOB Gain. The following an observation and a proposal are obtained. TP for TS 38.115-2 are shown in Annex A. It also contains editorial correction.

Observation 1: RAN4 have to conclude test requirements for Rel-17 NR repeater in this meeting.

Proposal 1: RAN4 approve the attached TP to TS 38.115-2 related to OTA OOB Gain if any concerns would not be raised.

References

- R4-2217477, “Email discussion summary for [104-bis-e][303] NR_Repeater_RFConformance_Part2”, Moderator(CATT), RAN4 #104bis-e, October 2022

- R4-2215389, “Discussion of remaining issues for FR2 MU”, CATT, RAN4#104bis-e, October 2022

- R4-2215791, “Discussion on test tolerance values of OOB Gain for FR2”, NTT DOCOMO, RAN4 #104bis-e, October 2022

- 3GPP TS 38.115-2, v 0.2.0

Annex A: TP to TS 38.115-2

------------------------------ Modified section ------------------------------

4.1.2.2	Radiated characteristics measurements

The maximum OTA Test System uncertainty for radiated characteristics measurements are given in tables 4.1.2.2-1. Details for derivation of OTA Test System uncertainty are given in corresponding clauses in TR 37.941 [x].

Table 4.1.2.2-1: Maximum Test System uncertainty for radiated characteristics tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Radiated transmit power (EIRP) | Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
|  | Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.5 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.3 OTA repeater output power (TRP) | ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 43.5 GHz) ±2.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.4 OTA frequency stability | ±[12] Hz Measurement results of [5000] Hz |  |
| 6.5 OTA out of band gain | ±2.1 dB, 24.25GHz < f ≦ 29.5GHz ±2.4 dB, 37GHz < f ≦ 43,5GHz ±2.6 dB, 43.5GHz < f ≦ 48.2GHz |  |
| 6.6.2 OTA ACLR | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 43.5 GHz) ±2.8 dB (43.5 GHz < f ≤ 48.2 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.6.3 OTA operating band unwanted emission | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.6.4 OTA spurious emissions | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |  |
| 6.7 OTA EVM | [1.25%] signal analyser [2%] stimulus signal |  |
| 6.8 OTA input intermodulation | ±[2.0] dB, f ≤ 3.0 GHz ±[2.6] dB, 3.0 GHz < f ≤ 4.2 GHz ±[3.2] dB, 4.2 GHz < f ≤ 6.0 GHz |  |
| 6.9 OTA ACRR | ±[2.7] dB (24.25 – 29.5 GHz) ±[2.7] dB (37 – 43.5 GHz) ±[2.9] dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.1 OTA transmitter OFF power | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.2 OTA transient period | N/A |  |

------------------------------ Omitted unchanged sections ------------------------------

6.4	OTA out of band gain

6.4.1	Definition and applicability

Out of band gain refers to the gain of the repeater outside the passband.

6.4.2	Minimum Requirement

The minimum requirement is in TS 38.106 [2], clause 7.4.2.

6.4.3	Test purpose

The test purpose is to verify that out of band gain is within the limit specified by the minimum requirement.

6.4.4	Method of test

6.4.4.1	Initial conditions

Test environment:

-	Normal, see annex B.2,

6.4.4.2	Procedure

For normal test environment conditions in OTA domain, the test procedure is as follows:

1)	Place the repeater at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the repeater with the test system.

3)	Orient the positioner (and repeater and test signal generator) in order that the direction to be tested aligns with the test antenna and the correct angle of arrival for the input signal is achieved..

4)	Set the CW generator power at the RIB as shown in annex D.1.1 with a power equivalent to Pin,p,EIRP, in the correct direction in respect to the repeater.

5)	Orient the positioner (and repeater and test signal source) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I) whilst maintaining the correct direction of arrival for the test signal.

6)	Measure the radiated power for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I), whilst maintaining the input signal in the correct direction with respect to the repeater.

8)	Calculate TRP using the EIRP measurements. The out of band gain at the frequency under test is given by the difference in dB between the measured output TRP and the EIRP of the input signal.

9) Repeat steps 2-8, shifting the offset frequency of the CW from the edge of the passband from 200kHz to 10MHz in steps of 200kHz for each passband and, for the case of multi-band connectors each operating band

6.4.5	Test Requirements

The gain outside the passband shall not exceed the maximum level specified in table 6.4.5-1, where:

-	f_offset_CW is the offset between the outer channel edge frequency of the outer channel in the passband and a CW signal.

Table 6.4.5-1: Out of band gain limits for bands above 24.25 GHz and below 29.5 GHz

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0.1*Minimum {400MHz, passband BW}  f_offset_CW < 150 MHz | 70.1 dB |
| 150 MHz  f_offset_CW < 400 MHz | 57.1 dB |
| 400 MHz  f_offset_CW < f_offset_max | 37.1 dB |

Table 6.4.5-2: Out of band gain limits for bands above 37 GHz and below 43.5 GHz

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0.1*Minimum {400MHz, passband BW}  f_offset_CW < 150 MHz | 70.4 dB |
| 150 MHz  f_offset_CW < 400 MHz | 57.4 dB |
| 400 MHz  f_offset_CW < f_offset_max | 37.4 dB |

Table 6.4.5-3: Out of band gain limits for bands above 43.5 GHz and below 48.2 GHz

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0.1*Minimum {400MHz, passband BW}  f_offset_CW < 150 MHz | 70.6 dB |
| 150 MHz  f_offset_CW < 400 MHz | 57.6 dB |
| 400 MHz  f_offset_CW < f_offset_max | 37.6 dB |

------------------------------ Omitted unchanged sections ------------------------------

C.1	Measurement of radiated characteristics

Table C.1-1: Derivation of test requirements (radiated tests)

| Test | Minimum requirement in TS 38.106 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power (EIRP) | See TS 38.106 [2], clause 7.2 | Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 43.5GHz 2.2 dB, 43.5GHz < f ≦ 48.2GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.5 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3 OTA repeater output power (TRP) |  | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43,5GHz 2.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4 OTA frequency stability | See TS 38.106 [2], clause 7.3 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.5 OTA out of band gain | See TS 38.106 [2], clause 7.4 | ±2.1 dB, 24.25GHz < f ≦ 29.5GHz ±2.4 dB, 37GHz < f ≦ 43,5GHz ±2.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.6.2 OTA ACLR | See TS 38.106 [2], clause 7.5.2 | Relative: 2.3 dB, 24.25GHz < f ≦ 29.5GHz 2.6 dB, 37GHz < f ≦43.5GHz 2.8 dB, 43.5GHz < f ≦ 48.2GHz Absolute: 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 43.5GHz < f ≦48.2GHz | Formula: Relative limit - TT Absolute limit +TT |
| 6.6.3 OTA operating band unwanted emission | See TS 38.106 [2], clause 7.5.3 | 0 MHz f < 0.1*BWcontiguous 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 43.5GHz < f ≦48.2GHz 0.1*BWcontiguous f < fmax 0 dB | Formula: Minimum Requirement + TT |
| 6.6.4 OTA spurious emissions | See TS 38.106 [2], clause 7.5.4 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7 OTA EVM | See TS 38.106 [2], clause 7.6 | 1.4%, Up to 16QAM 1.5%, 64QAM NOTE 2 1.8%, 256QAM Note 3 | Test requirement limit shifted by RSS of minimum requirement and stimulus signal EVM. Analyser error added to requirement limit. Formula: EVM limit + TT |
| 6.7 OTA input intermodulation | See TS 38.106 [2], clause 7.7 | ±[2.0] dB, f ≤ 3.0 GHz ±[2.6] dB, 3.0 GHz < f ≤ 4.2 GHz ±[3.2] dB, 4.2 GHz < f ≤ 6.0 GHz |  |
| 6.9 OTA ACRR | See TS 38.106 [2], clause 7.8 | ±[2.7] dB (24.25 – 29.5 GHz) ±[2.7] dB (37 – 43.5 GHz) ±[2.9] dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.1 OTA transmitter OFF power | See TS 38.106 [2], clause 7.9.2 | 2.9 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Minimum Requirement + TT |
| 6.10.2 OTA transient period | See TS 38.106 [2], clause 7.9.3 | N/A |  |
| NOTE 1: TT values are applicable for normal condition unless otherwise stated. NOTE 2: Support of 64QAM is based on the declaration NOTE 3: Support of 256QAM is based on the declaration. |  |  |  |

------------------------------ End of text proposal ------------------------------