3GPP TSG-RAN WG4 #105			    R4-2218725

Toulouse France, November 14th – 18th, 2022

Agenda item:	8.18.2.1

Source: 	Qualcomm Incorporated

Title: 	Preliminary results on SBFD adjacent channel coexistence

Document for:	Discussion

# Introduction

In RAN4#104-bis-e, further agreements on the SBFD adjacent channel coexistence studies were reached and a WF was approved for future meetings [1]. In [2] we provided preliminary results for SBFD coexistence, in this paper, we present preliminary simulation results for the agreed parameters and scenarios in [1], and further discuss the impact of SBFD deployments on gNB RF requirements considering self-interference, inter-subband CLI, and other coexistence aspects.

# Scenarios and simulation assumptions

## Coexistence scenarios

As agreed in RAN4#104-e and RAN4#104-bis-e [1,2], different deployment scenarios and transmission configurations for the victim and the aggressor networks have been agreed as shown below, where it was agreed to prioritize case 1 and 2 for the DUD and DU subband configurations.

Agreed scenarios from RAN4#104-e and RAN4#104-bis-e: FR; Scenario No.; Deployment Scenario1 (Aggressor -> Victim); Priority. FR1 (4GHz); 1; Urban Macro -> Urban Macro; High. 2; Indoor -> Indoor; Low. FR2 (30GHz); 3; Urban Macro -> Urban Macro; High. 4; Urban Micro -> Urban Micro; Low. 5; Indoor -> Indoor; Lw. Note 1: The Urban Macro is agreed as baseline scenario for SBFD co-ex study with high priority in RAN4#104-e, while it does not preclude other scenarios.. Victim; Aggressor; Figures; Aggressor baseline; Priority. NR TDD DL; SBFD (DUD); Case 1; NR TDD DL; High. SBFD (DU); Case 2; NR TDD DL; High. Case 3; NR TDD DL; Low. NR TDD UL; SBFD(DUD); Case 4; NR TDD UL; Low. SBFD(DU); Case 5; NR TDD UL; Low. Case 6; NR TDD UL; Low. SBFD (DUD); NR TDD DL; Case 1; No system in adjacent channel; High. SBFD (DU); NR TDD DL; Case 2; High. Case 3; Low. SBFD(DUD); NR TDD UL; Case 4; Low. SBFD(DU); NR TDD UL; Case 5; Low. Case 6; Low.

Since it has been agreed to consider ACLR flat modelling, RAN4 need to further discuss the differences between DUD and DU configuration in terms of Tx leakage modelling and the anticipated impact  on the adjacent channel coexistence work. In this paper, we focus on the DU SBFD configuration case, where we believe with the current agreements in RAN4, DUD and DU configurations might lead to similar results in terms of system performance. 
Observation: With current agreements on ACLR modelling, RAN4 need to discuss expected differences between DUD and DU configurations in terms of Tx leakage modelling and the anticipated impact  on the adjacent channel coexistence work.

## Deployment scenarios

For urban macro scenarios, a pictorial representation with 100% grid shift and ISD=500m is shown in Figure 1, where the victim and aggressor networks are depicted in blue and red, respectively. As agreed in [1], we present results for the UE uniform dropping case, where UEs are randomly dropped within the serving area of each cell. Figure 1 presents the distributions of the distance between the aggressor gNBs/UEs and the victim UEs. As it can be observed, the probability of having small inter-UE distances is low for both FR1 and FR2 in UMa scenarios. Thus, it is expected that the contribution of the inter-UE CLI will not be as impactful as the inter-gNB CLI for UMa scenarios.

Figure 1 UMa deployment with 100% grid shift and inter-UE distance distributions

Observation: The probability of having small  inter-UE distances is low in UMa scenarios, which will lead to negligible contribution of the inter-UE CLI for SBFD deployments.

## Simulation parameters

In this section we provide the list of simulations assumptions based on the agreements in [1] and TR 38.828. Table 3 lists our preliminary simulation parameters that were used to reproduce the results presented in Section 4.

Table 1 List of simulation parameters

| General | Frequency | FR1: 4GHz as exemplary frequency FR2: 30GHz as exemplary frequency |  |
| --- | --- | --- | --- |
|  | Channel BW | FR1: 100MHz FR2: 200MHz |  |
|  | SCS | 15KHz |  |
|  | SBFD configuration | DU configuration with 80MHz downlink and 20MHz for uplink as explained in figure 2. For FR2, each DL sub-band BW is 160MHz while 40MHz is assumed for UL. |  |
|  | ISD | 500m for FR1 and 200m for FR2 |  |
|  | UE-gNB minimum distance | 35m for UMa and 0m for InH |  |
|  | Path-loss model | Based on 38.828 Section 5.2.2.1.1 Macro (Aggressor) → Macro (Victim) - Macro-to-UE: UMa see TR 38.803 - Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.803 - UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 + penetration loss see TR 38.803 |  |
|  | Grid shift consideration | 100 % |  |
|  | Evaluation metrics | Follow the TR 38.828’s evaluation criteria to check the 50% and 5% throughput loss compared to the baseline scenario defined. |  |
|  | Handover margin | 3 dB |  |
| UE parameters | UMa UE distribution | Indoor/Outdoor UE ratio | Re-use TR 38.828. For Macro-to-Macro assume 20% indoor and 80% outdoor ratio. For FR2, no indoor UEs are assumed. |
|  |  | UE distribution mechanism | Evenly randomize the UEs within each cell |
|  |  | Number of UEs | Equal to the number of sub-bands, i.e., 2 UEs for {DU} subband config, 3 UEs for {DUD} config. |
|  | Antenna parameters | FR1: Re-use TR 38.828 assumptions (FR1 max Tx 23dBm, min Tx -40 dBm with 0dBi omni directional antenna) FR2: Option 2: FR2 max Tx 23dBm (peak eirp 43dBm) with (M, N, P)=(1, 4, 2), 2 panels antennas and element gain as 1.5 dBi |  |
|  | Power control | Same as PC model in 38.803 and 38.901 with UL SNR target = 15dB and minimum conducted power = -40 dBm |  |
|  | UE antenna height | 1.5m |  |
|  | Traffic model | Full buffer to consider worst case scenario for network loading |  |
|  | Noise figure | FR1: 9 dB and FR2: 10 dB |  |
|  | ACLR/ACS | FR1: ACLR 30 dB and ACS 33 dB FR2: ACLR 17 dB and ACS 23 dB 1 step ACLR model is assumed. |  |
| gNB parameters | Antenna parameters | Utilize an extra panel for subband UL operation to provide enough spatial isolation. Accordingly, the TRP and element number for DL and UL in SBFD BS will be the same as TDD BS configuration. Antenna modeling follows that of TR 38.828 For Legacy TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ For SBFD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ |  |
|  | gNB antenna height | 25m for UMa and ceiling mounter AP is considered at 3m for InH |  |
|  | Mechanical down tilt | 6 degrees mechanical down tilt. |  |
|  | gNB Tx power | FR1: 49 dBm for both legacy TDD and SBFD FR2: 43 dBm for both legacy TDD and SBFD |  |
|  | Noise figure | FR1: 5 dB and FR2: 10 dB |  |
|  | ACLR/ACS | FR1: ACLR 45 dB and ACS 46 dB FR2: ACLR 28 dB and ACS 23.5 dB Flat ACLR modeling is assumed. |  |

## Self-interference mitigation capability

To enable proper reception of the uplink signal at the SBFD gNB receiver, gNB should mitigate the direct self-interference ‘leakage’ and any significant clutter reflections. To quantify the gNB self-interference capability, we modelled the self-interference as flat as agreed in RAN4 [1,2], with the mitigation components shown in the table below.

Table 2 Self-interference mitigation capabilities

|  |  | FR1 Wide-Area BS | FR2 Wide-Area BS |
| --- | --- | --- | --- |
| Component capability | Spatial isolation | 85 dB | 85 dB |
|  | Frequency isolation | 45 dB | 28 dB |
|  | Digital IC | 15dB | 15 dB |
| Overall RSIC capability |  | 145 dB | 128 dB |

# Preliminary simulation results

Throughout this section our preliminary simulation results for urban macro scenario are presented. We follow the simulation methodology agreed in [1], where the RF parameters are determined based on the degradation caused by the adjacent channel interference (ACI), following similar coexistence simulation work as in [3] and [4].

## FR1 results

#### NR TDD DL as victim

Figure 2 presents the SINR (with ACI) and interference power distribution (with ACI) for FR1 UMa deployment when the victim is operating in a TDD DL slot. As observed, the performance impact of SBFD operation in the adjacent channel is nearly negligible when compared to the legacy NR TDD DL. It is apparent that for Macro-Macro scenarios (impact on UE), the adjacent interference is dominated by legacy DL interference from co-channel and aggressor gNBs. In addition, no performance degradation relating to ACLR/ACS due to inter-UE CLI is observed in SBFD adjacent channel operation. Additionally, Figure 2 presents the distribution of the adjacent interference power for the three network operation modes. We can observe that the SBFD network is dominated by the legacy interference from aggressor gNBs towards the victim UEs, while the inter-UE CLI component is marginal. We can also observe that although the legacy network is transmitting over full 100MHz bandwidth, it provides quite close statistics compared to the SBFD gNB that is transmitting DL sub-band of 80MHz. We observe a gap of approximately 1 dB between legacy and SBFD curves. Note that this gap is dependent on how the ACLR is modelled as highlighted in section 3.6.1, where we considered a flat ACLR model for the gNB of value 45 dB.

Observation: For FR1 macro-macro scenarios (impact on UE), the adjacent interference is dominated by legacy DL interference from legacy gNBs. No performance degradation due to inter-UE CLI is observed in adjacent channel.

Figure 2 SINR and interference power distributions for UMa FR1 deployment when victim is TDD DL

#### SBFD as victim

##### SBFD DL results

Figure 3 presents the SINR and interference power distribution for FR1 UMa deployment when the victim is operating in a SBFD DL slot. We compare multiple scenarios here, namely, single operator (i.e., no adjacent channel operation), multiple operator (adjacent channel operator is either TDD DL, TDD UL, or SBFD). Similar to the TDD DL victim case, the system is dominated by co-channel interference from other gNBs in the victim network. As a result, no degradation is observed between the SBFD and TDD DL single operator case. When there is operation in the adjacent channel, approximately 1 dB degradation in the SINR is observed for different types of aggressor networks. To further clarify, we plot the distribution of the interference power to decouple the different interference components. First, we can observe that the co-channel interference is the dominant compared to the inter-subband inter-UE CLI. In addition, we can see the impact of the SBFD subband configuration between the TDD DL and SBFD deployment (i.e., around 1 dB for PSD integration). Secondly, we observe that in terms of interference statistics, SBFD deployment in the adjacent channel will result in same aggregate adjacent interference as TDD DL, since the former is dominant by the inter-gNB CLI.

Figure 3 SINR and interference power distributions for UMa FR1 deployment when victim is SBFD DL

Observation: For FR1 and SBFD DL as a victim, no SINR degradation is observed compared to legacy TDD DL network.

##### SBFDL UL results

Figure 4 presents the SINR and interference power distribution for FR1 UMa deployment when the victim is operating in a SBFD UL slot, similar to the previous results, we include an TDD UL single operator network for comparison. In addition, we consider as agreed in RAN4#104-bis-e [1] to model the co-site inter-sector inter-subband interference similar to the self-interference, thus providing sufficient isolation between the sectors within a given cell. As expected, due to the inter-subband inter-gNB CLI in SBFD deployments, SINR deterioration is observed for the single operator case, which hinders achieving the SNR target which equals 15 dB. This is also clear when comparing the interference distribution between the legacy inter-UE interference in synchronized networks to the inter-subband inter-gNB CLI. Additionally, we observe that when there an aggressor network, performance varies depending on the transmission direction (i.e., served slot). For TDD UL, no observed degradation when compared to single operator case, whereas when the aggressor is DL, additional degradation is observed due to the additional adjacent inter-gNB CLI.

Observation:  For FR1 and SBFD UL as a victim, the aggregate inter-gNB CLI dominates the aggregate legacy co-channel interference. Similar behaviour is also observed for the aggregate adjacent interference.

Figure 4 SINR and interference power distributions for UMa FR1 deployment when victim is SBFD UL

## FR2 results

### NR TDD DL as victim

Similar to FR1, Figure 6 present the SINR (with ACI) where the trends and observations for FR1 are consistent for FR2. In details, for Macro-Macro scenarios (impact on UE), the adjacent interference is dominated by legacy DL interference from aggressor gNBs and no performance degradation relating to ACLR/ACS due to inter-UE CLI is observed in SBFD adjacent channel operation.

Figure 6 SINR distributions for UMa FR2 deployment when victim is SBFD DL

Observation: For FR2, no SINR degradation is observed when the victim network is SBFD DL compared to legacy TDD DL network.

### SBFD As victim

Figure 7 presents the SINR distribution for FR2 UMa deployment when the victim is operating in a SBFD DL slot. The trends and observations for FR1 are similar for FR2. For Macro-Macro scenarios (impact on UE), the adjacent interference is dominated by legacy DL interference from aggressor gNBs and no performance degradation relating to ACLR/ACS due to inter-UE CLI is observed in SBFD adjacent channel operation.

Observation: For FR2 and SBFD DL as a victim, no SINR degradation is observed compared to legacy TDD DL network.

Figure 7 SINR distributions for UMa FR2 deployment when victim is SBFD DL

Finally, Figure 8 presents the aggregate interference distributions when SBFD UL is victim. As we see the aggregate interference is dominated by the inter-subband inter-gNB CLI. For legacy TDD UL networks, we only see the impact of the inter-UE legacy co-channel interference. Accordingly, advanced solutions are envisaged to ensure that the impact of inter-subband inter-gNB CLI is properly mitigated.

Observation: When SBFD is a victim, to protect the SBFD UL slots, advanced solutions are envisaged to ensure that the impact of inter-subband inter-gNB CLI is properly mitigated.

Figure 8 Aggregate interference distributions for UMa FR2 deployment when victim is SBFD DL

# Conclusion

Throughout this contribution, we provided our views simulation scenarios for the SBFD adjacent coexistence work within RAN4, preliminary list of simulation assumptions, and simulation results investigating the impact of SBFD. In summary we have made the following observations:

Observation: With current agreements on ACLR modelling, RAN4 need to discuss expected differences between DUD and DU configurations in terms of Tx leakage modelling on the adjacent sub-bands.

Observation: The probability of having small  inter-UE distances is low in UMa scenarios, which will lead to negligible contribution of the inter-UE CLI for SBFD deployments.

Observation: For FR1 macro-macro scenarios (impact on UE), the adjacent interference is dominated by legacy DL interference from legacy gNBs. No performance degradation due to inter-UE CLI is observed in adjacent channel.

Observation: For FR1 and SBFD DL as a victim, no SINR degradation is observed compared to legacy TDD DL network.

Observation: For FR1 and SBFD UL as a victim, the aggregate inter-gNB CLI dominates the aggregate legacy co-channel interference. Similar behaviour is also observed for the aggregate adjacent interference.

Observation: For FR2 and SBFD DL as a victim, no SINR degradation is observed compared to legacy TDD DL network.

Observation: When SBFD is a victim, to protect the SBFD UL slots, advanced solutions are envisaged to ensure that the impact of inter-subband inter-gNB CLI is properly mitigated.

# References

- R4-2217466, “WF for adjacent channel co-existence evaluation of SBFD operation”, Samsung, RAN4#104-bis-e.

- R4-2215345, “SBFD adjacent channel coexistence evaluation”, Qualcomm.

- TR 38.828, “Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR; (Release 16)”, 3GPP

- TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”, 3GPP