3GPP TSG-RAN WG4 Meeting #105              	R4-2219017

Toulouse, France, 14th Nov-18th Nov, 2022

Agenda item:	6.2.6.2.1

Source:	Samsung

Title:	Simulation results on PUSCH demodulation requirement for Rel-17 NTN

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, the updated test setup for PUSCH demodulation requirement with NTN was agreed. The related agreement was captured into the WF [1].

In this contribution, the ideal and impairment results are provided for requirement derivation.

# 2	Initial simulation results

In this section, based on the agreed simulation assumption in the last meeting, the simulation results for different are provided for simulation alignment purpose.

Table 1: ideal and impairment simulation results for NTN normal PUSCH with CP-OFDM, NTN-TDLA100-200

| Case | Tx | Rx | Channel | SCS (kHz) | CBW (MHz) | RB | MCS | Mapping type | DMRS | SNR (ideal) | SNR (Impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | NTN-TDLA100-200 Low | 15 | 5 | 25 | 4 | A | 1+1(2,11) | 1.6 | 3.6 |
| 1 |  | 1 |  | 15 | 5 | 25 |  | B | 1+1(0,10) | 1.6 | 3.6 |
| 2 |  | 2 |  | 15 | 5 | 25 |  | A | 1+1(2,11) | -2.4 | -0.4 |
| 3 |  | 2 |  | 15 | 5 | 25 |  | B | 1+1(0,10) | -2.4 | -0.4 |
| 4 |  | 1 |  | 30 | 10 | 24 |  | A | 1+1(2,11) | 1.5 | 3.5 |
| 5 |  | 1 |  | 30 | 10 | 24 |  | B | 1+1(0,10) | 1.5 | 3.5 |
| 6 |  | 2 |  | 30 | 10 | 24 |  | A | 1+1(2,11) | -2.5 | -0.5 |
| 7 |  | 2 |  | 30 | 10 | 24 |  | B | 1+1(0,10) | -2.5 | -0.5 |

Table 2: ideal and impairment simulation results for NTN normal PUSCH with CP-OFDM, NTN-TDLC5-200

| Case | Tx | Rx | Channel | SCS (kHz) | CBW (MHz) | RB | MCS | Mapping type | DMRS | SNR (ideal) | SNR (Impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | NTN-TDLA100-200 Low | 15 | 5 | 25 | 4 | A | 1+1(2,11) | 0.1 | 2.1 |
| 1 |  | 1 |  | 15 | 5 | 25 |  | B | 1+1(0,10) | 0.1 | 2.1 |
| 2 |  | 2 |  | 15 | 5 | 25 |  | A | 1+1(2,11) | -3.1 | -1.1 |
| 3 |  | 2 |  | 15 | 5 | 25 |  | B | 1+1(0,10) | -3.1 | -1.1 |
| 4 |  | 1 |  | 30 | 10 | 24 |  | A | 1+1(2,11) | -0.1 | 1.9 |
| 5 |  | 1 |  | 30 | 10 | 24 |  | B | 1+1(0,10) | -0.2 | 1.8 |
| 6 |  | 2 |  | 30 | 10 | 24 |  | A | 1+1(2,11) | -3.4 | -1.4 |
| 7 |  | 2 |  | 30 | 10 | 24 |  | B | 1+1(0,10) | -3.4 | -1.4 |

Table 3: ideal and impairment simulation results for NTN normal PUSCH with DFT-s-OFDM, NTN-TDLA100-200

| Case | Tx | Rx | Channel | SCS (kHz) | CBW (MHz) | RB | MCS | Mapping type | DMRS | SNR (ideal) | SNR (Impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | NTN-TDLA100-200 Low | 15 | 5 | 25 | 4 | A | 1+1(2,11) | 2.0 | 4.0 |
| 1 |  | 1 |  | 15 | 5 | 25 |  | B | 1+1(0,10) | 2.0 | 4.0 |
| 2 |  | 2 |  | 15 | 5 | 25 |  | A | 1+1(2,11) | -2.3 | -0.3 |
| 3 |  | 2 |  | 15 | 5 | 25 |  | B | 1+1(0,10) | -2.2 | -0.2 |
| 4 |  | 1 |  | 30 | 10 | 24 |  | A | 1+1(2,11) | 1.9 | 3.9 |
| 5 |  | 1 |  | 30 | 10 | 24 |  | B | 1+1(0,10) | 1.9 | 3.9 |
| 6 |  | 2 |  | 30 | 10 | 24 |  | A | 1+1(2,11) | -2.3 | -0.3 |
| 7 |  | 2 |  | 30 | 10 | 24 |  | B | 1+1(0,10) | -2.3 | -0.3 |

Table 4: ideal and impairment simulation results for NTN normal PUSCH with DFT-s-OFDM, NTN-TDLC5-200

| Case | Tx | Rx | Channel | SCS (kHz) | CBW (MHz) | RB | MCS | Mapping type | DMRS | SNR (ideal) | SNR (Impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | NTN-TDLA100-200 Low | 15 | 5 | 25 | 4 | A | 1+1(2,11) | 0.1 | 2.1 |
| 1 |  | 1 |  | 15 | 5 | 25 |  | B | 1+1(0,10) | 0.1 | 2.1 |
| 2 |  | 2 |  | 15 | 5 | 25 |  | A | 1+1(2,11) | -3.1 | -1.1 |
| 3 |  | 2 |  | 15 | 5 | 25 |  | B | 1+1(0,10) | -3.1 | -1.1 |
| 4 |  | 1 |  | 30 | 10 | 24 |  | A | 1+1(2,11) | -0.2 | 1.8 |
| 5 |  | 1 |  | 30 | 10 | 24 |  | B | 1+1(0,10) | -0.2 | 1.8 |
| 6 |  | 2 |  | 30 | 10 | 24 |  | A | 1+1(2,11) | -3.4 | -1.4 |
| 7 |  | 2 |  | 30 | 10 | 24 |  | B | 1+1(0,10) | -3.5 | -1.5 |

Table 5: ideal and impairment simulation results for NTN UL timing adjustment

| Case | Tx | Rx | Channel | SCS (kHz) | CBW (MHz) | RB | MCS | Mapping type | DMRS | SNR (ideal) | SNR (Impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | NTN-TDLA100-200 Low | 15 | 5 | 0~11 for moving UE 12~23 for stationary UE | 4 | A | 1+1(2,11) | 2.4 | 4.4 |
| 1 |  | 1 |  | 15 | 5 |  |  | B | 1+1(0,10) | 2.4 | 4.4 |
| 2 |  | 2 |  | 15 | 5 |  |  | A | 1+1(2,11) | -2.1 | -0.1 |
| 3 |  | 2 |  | 15 | 5 |  |  | B | 1+1(0,10) | -2.1 | -0.1 |
| 4 |  | 1 |  | 30 | 10 |  |  | A | 1+1(2,11) | 1.7 | 3.7 |
| 5 |  | 1 |  | 30 | 10 |  |  | B | 1+1(0,10) | 1.7 | 3.7 |
| 6 |  | 2 |  | 30 | 10 |  |  | A | 1+1(2,11) | -2.3 | -0.3 |
| 7 |  | 2 |  | 30 | 10 |  |  | B | 1+1(0,10) | -2.3 | -0.3 |

Table 6: ideal and impairment simulation results for NTN PUSCH repetition type A

| Case | Tx | Rx | Channel | SCS (kHz) | CBW (MHz) | RB | MCS | Mapping type | DMRS | Aggregation | SNR (ideal) | SNR (Impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | NTN-TDLA100-200 Low | 15 | 5 | 25 | 4 | A | 1+1(2,11) | n2 | -6.7 | -4.7 |
| 1 |  | 1 |  | 15 | 5 | 25 |  | B | 1+1(0,10) | n2 | -6.7 | -4.7 |
| 2 |  | 2 |  | 15 | 5 | 25 |  | A | 1+1(2,11) | n2 | -10.4 | -8.4 |
| 3 |  | 2 |  | 15 | 5 | 25 |  | B | 1+1(0,10) | n2 | -10.4 | -8.4 |
| 4 |  | 1 |  | 30 | 10 | 24 |  | A | 1+1(2,11) | n2 | -6.9 | -4.9 |
| 5 |  | 1 |  | 30 | 10 | 24 |  | B | 1+1(0,10) | n2 | -6.9 | -4.9 |
| 6 |  | 2 |  | 30 | 10 | 24 |  | A | 1+1(2,11) | n2 | -10.5 | -8.5 |
| 7 |  | 2 |  | 30 | 10 | 24 |  | B | 1+1(0,10) | n2 | -10.4 | -8.4 |

# 3	Conclusion

In this contribution, the ideal and impairment simulation results for NTN PUSCH requirement are provided requirement derivation.

# References

R4-2217348, WF on NTN SAN demodulation performance requirements