3GPP TSG-RAN WG4 Meeting #105              	R4-2219018

Toulouse, France, 14th Nov-18th Nov, 2022

Agenda item:	6.2.6.2.2

Source:	Samsung

Title:	Discussion and simulation results on PUCCH demodulation requirement for Rel-17 NTN

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, the detail test setup for PUCCH demodulation requirement with NTN was agreed. The related agreement was captured into the WF [1].

In this contribution, the view on the remaining issue for SAN PUCCH requirement is provided. Meanwhile, the ideal and impairment results are provided for requirement derivation.

# 2	Discussion

In this section, the view on remaining issue about test setup for PUCCH requirement is provided.

Antenna configuration for PUCCH requirements

- Option 1: Consider SAN PUCCH format 0 and format 2 requirements for only 2Rx configuration with NLOS channel. Corresponding manufacture declarations and applicability rules should be further discussed. - Option 2: Introduce LOS channel for SAN PUCCH format 0 and 2 demodulation requirements with 1Rx configuration. - Option 3: Discuss a general rule that if the final derived simulation result is larger than a certain value, such as [10dB], then the corresponding cases can be not defined. - Option 4: Consider both 1Rx and 2Rx for SAN PUCCH format 0 and 2 requirements with NLOS channel with the same test applicability rule for different antenna configurations as for PUSCH. - Option 5: Option 1 + Option 2, i.e. Consider 1Rx SAN PUCCH format 0 and 2 requirements with LOS channel and 2Rx SAN PUCCH format 0 and 2 requirements with NLOS channel, with the same test applicability rule for different antenna configurations as for PUSCH.

In the previous meeting, NLOS channel with NTN-TDLA was considered for PUCCH requirement for both 1 Rx and 2Rx for PUCCH requirement. Based on the simulation results summary provided, large performance gap is existed among companies, especially for 1T1R configuration with TDLA channel, where the achieved SNR is round 10 dB. Based on it, companies proposed to change the channel model for format 0 and format 2 with 1T1R configuration.

Regarding the additional channel model for PUCCH requirement, we don think it is necessary to define requirement with LOS channel as TDLC, since the test coverage of different channel condition has already been covered in PUSCH. Meanwhile, from receiver perspective, there is no different foreseen. Therefore, we prefer to consider only 2Rx requirement for format 0 and format 2. The test applicability rule can be considered as

- Unless otherwise stated, PUCCH requirement test with format 0 and 2 with 2Rx configuration with NLOS channel shall apply only if the BS supports it.

Considering RAN4 has already agreed to introduce both 1Rx and 2Rx for PUCCH format 1, 3 and 4 requirements with test applicability rule, we are also fine to define requirement with 1Rx and 2Rx for PUCCH format 0 and 2, with applying the same test applicability rule

Proposal 1: RAN4 consider SAN PUCCH format 0 and format 2 requirements for only 2Rx configuration with NLOS channel with test applicability rule as

- Unless otherwise stated, PUCCH requirement test with format 0 and 2 with 2Rx configuration with NLOS channel shall apply only if the BS supports it.

or define 1Rx and 2Rx for SAN PUCCH format 0 and 2 requirements with NLOS channel with the same test applicability rule for different antenna configurations as for PUSCH

# 3	Simulation results

In this section, based on the agreed simulation assumption in the last meeting, the simulation results for different are provided for simulation alignment purpose

Table 1. Ideal and impairment simulation results for PUCCH format 0

| Case | Tx/Rx | Format | SCS | BW | Payload | RB | Number of symbols | Channel | Metric | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 0 | 15KHz | 5MHz | 1 | 1 | 2 | NTN-TDLA100-200 | ACK missed <1% | 7.79 | 9.79 |
| 2 | 1T2R | 0 | 15KHz | 5MHz | 1 | 1 | 2 | NTN-TDLA100-200 | ACK missed <1% | 1.92 | 3.92 |
| 3 | 1T1R | 0 | 30KHz | 10MHz | 1 | 1 | 2 | NTN-TDLA100-200 | ACK missed <1% | 9.52 | 11.52 |
| 4 | 1T2R | 0 | 30KHz | 10MHz | 1 | 1 | 2 | NTN-TDLA100-200 | ACK missed <1% | 2.63 | 4.63 |

Table 2. Ideal and impairment simulation results for PUCCH format 1

| Case | Tx/Rx | Format | SCS | BW | Payload | RB | Number of symbols | Channel | Metric | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 1 | 15KHz | 5MHz | 2 | 1 | 14 | NTN-TDLA100-200 | ACK missed <1% | -0.61 | 1.39 |
|  |  |  |  |  |  |  |  | NTN-TDLA100-200 | NACK to ACK < 0.1% | 0.75 | 2.75 |
| 2 | 1T2R | 1 | 15KHz | 5MHz | 2 | 1 | 14 | NTN-TDLA100-200 | ACK missed <1% | -6.69 | -4.69 |
|  |  |  |  |  |  |  |  |  | NACK to ACK< 0.1% | -5.24 | -3.24 |
| 3 | 1T1R | 1 | 30KHz | 10MHz | 2 | 1 | 14 | NTN-TDLA100-200 | ACK missed <1% | 0.27 | 2.3 |
|  |  |  |  |  |  |  |  | NTN-TDLA100-200 | NACK to ACK< 0.1% | 1.42 | 3.4 |
| 4 | 1T2R | 1 | 30KHz | 10MHz | 2 | 1 | 14 | NTN-TDLA100-200 | ACK missed <1% | -6.06 | -4.0 |
|  |  |  |  |  |  |  |  | NTN-TDLA100-200 | NACK to ACK< 0.1% | -4.82 | -2.8 |

Table 3. Ideal and impairment simulation results for PUCCH format 2

| Case | Tx/Rx | Format | SCS | BW | Payload | RB | Number of symbols | First symbol | Channel | Metric | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 2 | 15KHz | 5MHz | 4 | 4 | 1 | 13 | NTN-TDLA100-200 | ACK missed <1% | 12.31 | 14.31 |
| 2 | 1T2R | 2 | 15KHz | 5MHz | 4 | 4 | 1 | 13 | NTN-TDLA100-200 | ACK missed <1% | 3.69 | 5.69 |
| 3 | 1T1R | 2 | 30KHz | 10MHz | 4 | 4 | 1 | 13 | NTN-TDLA100-200 | ACK missed <1% | 10.31 | 12.31 |
| 4 | 1T2R | 2 | 30KHz | 10MHz | 4 | 4 | 1 | 13 | NTN-TDLA100-200 | ACK missed <1% | 3.2 | 5.2 |
| 5 | 1T1R | 2 | 15KHz | 5MHz | 22 | 9 | 2 | 12 | NTN-TDLA100-200 | UCI BLER<1% | 4.40 | 6.40 |
| 6 | 1T2R | 2 | 15KHz | 5MHz | 22 | 9 | 2 | 12 | NTN-TDLA100-200 | UCI BLER<1% | -1.29 | 0.71 |
| 7 | 1T1R | 2 | 30KHz | 10MHz | 22 | 9 | 2 | 12 | NTN-TDLA100-200 | UCI BLER<1% | 3.97 | 5.97 |
| 8 | 1T2R | 2 | 30KHz | 10MHz | 22 | 9 | 2 | 12 | NTN-TDLA100-200 | UCI BLER<1% | -1.49 | 0.51 |
| 9 | 1T1R | 2 | 15KHz | 5MHz | 4 | 4 | 1 | 13 | NTN-TDLC5-200 | ACK missed <1% | 3.72 | 5.72 |
| 10 | 1T2R | 2 | 15KHz | 5MHz | 4 | 4 | 1 | 13 | NTN-TDLC5-200 | ACK missed <1% | -1.27 | 0.73 |
| 11 | 1T1R | 2 | 30KHz | 10MHz | 4 | 4 | 1 | 13 | NTN-TDLC5-200 | ACK missed <1% | 3.72 | 5.72 |
| 12 | 1T2R | 2 | 30KHz | 10MHz | 4 | 4 | 1 | 13 | NTN-TDLC5-200 | ACK missed <1% | -1.35 | 0.65 |
| 13 | 1T1R | 2 | 15KHz | 5MHz | 22 | 9 | 2 | 12 | NTN-TDLC5-200 | UCI BLER<1% | 2.03 | 4.03 |
| 14 | 1T2R | 2 | 15KHz | 5MHz | 22 | 9 | 2 | 12 | NTN-TDLC5-200 | UCI BLER<1% | -3.27 | -1.27 |
| 15 | 1T1R | 2 | 30KHz | 10MHz | 22 | 9 | 2 | 12 | NTN-TDLC5-200 | UCI BLER<1% | 1.81 | 3.81 |
| 16 | 1T2R | 2 | 30KHz | 10MHz | 22 | 9 | 2 | 12 | NTN-TDLC5-200 | UCI BLER<1% | -3.37 | -1.37 |

Table 4. Ideal and impairment simulation results for PUCCH format 3

| Case | Tx/Rx | Format | SCS | BW | Payload | RB | Number of symbols | DMRS | Channel | Metric | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 3 | 15KHz | 5MHz | 16 | 1 | 14 | Without DMRS | NTN-TDLA100-200 | UCI BLER<1% | 4.44 | 6.44 |
| 2 | 1T2R | 3 | 15KHz | 5MHz | 16 | 1 | 14 | With out DMRS | NTN-TDLA100-200 | UCI BLER<1% | -1.91 | 0.09 |
| 3 | 1T1R | 3 | 30KHz | 10MHz | 16 | 1 | 14 | With out DMRS | NTN-TDLA100-200 | UCI BLER<1% | 5.17 | 7.17 |
| 4 | 1T2R | 3 | 30KHz | 10MHz | 16 | 1 | 14 | With out DMRS | NTN-TDLA100-200 | UCI BLER<1% | -1.19 | 0.81 |
| 5 | 1T1R | 3 | 15KHz | 5MHz | 16 | 1 | 14 | With DMRS | NTN-TDLA100-200 | UCI BLER<1% | 3.82 | 5.82 |
| 6 | 1T2R | 3 | 15KHz | 5MHz | 16 | 1 | 14 | With DMRS | NTN-TDLA100-200 | UCI BLER<1% | -2.27 | -0.27 |
| 7 | 1T1R | 3 | 30KHz | 10MHz | 16 | 1 | 14 | With DMRS | NTN-TDLA100-200 | UCI BLER<1% | 5.13 | 7.13 |
| 8 | 1T2R | 3 | 30KHz | 10MHz | 16 | 1 | 14 | With DMRS | NTN-TDLA100-200 | UCI BLER<1% | -1.38 | 0.62 |

Table 5. Ideal and impairment simulation results for PUCCH format 4

| Case | Tx/Rx | Format | SCS | BW | Payload | RB | Number of symbols | DMRS | Channel | Metric | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 4 | 15KHz | 5MHz | 22 | 1 | 14 | Without DMRS | NTN-TDLA100-200 | UCI BLER <1% | 6.99 | 8.99 |
| 2 | 1T2R | 4 | 15KHz | 5MHz | 22 | 1 | 14 | Without DMRS | NTN-TDLA100-200 | UCI BLER <1% | 0.01 | 2.01 |
| 3 | 1T1R | 4 | 30KHz | 10MHz | 22 | 1 | 14 | Without DMRS | NTN-TDLA100-200 | UCI BLER <1% | 7.59 | 9.59 |
| 4 | 1T2R | 4 | 30KHz | 10MHz | 22 | 1 | 14 | Without DMRS | NTN-TDLA100-200 | UCI BLER <1% | 0.71 | 2.71 |
| 5 | 1T1R | 4 | 15KHz | 5MHz | 22 | 1 | 14 | With DMRS | NTN-TDLA100-200 | UCI BLER <1% | 6.28 | 8.28 |
| 6 | 1T2R | 4 | 15KHz | 5MHz | 22 | 1 | 14 | With DMRS | NTN-TDLA100-200 | UCI BLER <1% | -0.21 | 1.79 |
| 7 | 1T1R | 4 | 30KHz | 10MHz | 22 | 1 | 14 | With DMRS | NTN-TDLA100-200 | UCI BLER <1% | 7.60 | 9.60 |
| 8 | 1T2R | 4 | 30KHz | 10MHz | 22 | 1 | 14 | With DMRS | NTN-TDLA100-200 | UCI BLER <1% | 0.66 | 2.66 |

Table 6. Ideal and impairment simulation results for PUCCH format 1 with multi-slot

| Case | Tx/Rx | Format | Number of slots for PUCCH repetition | SCS | BW | Payload | RB | Number of symbols | Channel | Metric | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 1 | 2 | 15KHz | 5MHz | 2 | 1 | 14 | NTN-TDLA100-200 | ACK missed <1% | -3.69 | -1.69 |
|  |  |  |  |  |  |  |  |  |  | NACK to ACK< 0.1% | -1.87 | 0.13 |
| 2 | 1T2R | 1 | 2 | 15KHz | 5MHz | 2 | 1 | 14 | NTN-TDLA100-200 | ACK missed <1% | -9.68 | -7.68 |
|  |  |  |  |  |  |  |  |  |  | NACK to ACK< 0.1% | -8.07 | -6.07 |
| 3 | 1T1R | 1 | 2 | 30KHz | 10MHz | 2 | 1 | 14 | NTN-TDLA100-200 | ACK missed <1% | -2.93 | -0.93 |
|  |  |  |  |  |  |  |  |  |  | NACK to ACK<0.1% | -1.44 | 0.56 |
| 4 | 1T2R | 1 | 2 | 30KHz | 10MHz | 2 | 1 | 14 | NTN-TDLA100-200 | ACK missed <1% | -9.29 | -7.29 |
|  |  |  |  |  |  |  |  |  |  | NACK to ACK<0.1% | -7.95 | -5.95 |

# 4	Conclusion

In this contribution, the ideal and impairment simulation results of SAN PUCCH are provided for requirement derivation.

Proposal 1: RAN4 consider SAN PUCCH format 0 and format 2 requirements for only 2Rx configuration with NLOS channel with test applicability rule as

- Unless otherwise stated, PUCCH requirement test with format 0 and 2 with 2Rx configuration with NLOS channel shall apply only if the BS supports it.

or define 1Rx and 2Rx for SAN PUCCH format 0 and 2 requirements with NLOS channel with the same test applicability rule for different antenna configurations as for PUSCH.

# References

R4-2217348, WF on NTN SAN demodulation requirement