3GPP TSG-RAN WG4 Meeting # 105	R4-2219135

# Toulouse, France, November 14 – November 18, 2022

Agenda item:	8.13.2

Source: 	Qualcomm Incorporated

Title: 	Discussions on the ATG UE RF requirements

Document for:	Approval

# Introduction

In RAN4#104-bis-e, the ATG UE RF requirements were discussed and the WF was approved in [1]. In this meeting, we provide our views on the remaining issues.

# Discussion

## UE Doppler pre-compensation

On the UE Doppler pre-compensation discussion, it was agreed in RAN4#104-bis-e as following:

Agreement: - Focus on the frequency error requirements considering the Doppler pre-compensation function - Further check whether and how the Doppler pre-compensation functionality specified for NTN works for ATG with the understanding that there is no RAN1/2 impact expected.

To apply the NTN Doppler pre-compensation functionality for ATG UE, the necessary information is the location of ATG BS since the ATG BS is the stationary. With the ATG BS location, ATG UE could do the Doppler pre-compensation based on the aircraft location and speed from GNSS on the aircraft. Based on the previous discussions, the common understanding is ATG BS location should be broadcasted in SIB19 which was specified for NTN. In TS 38.331, the IE of EphemerisInfo can be used to inform the location/position vector of BS in ATG.

Observation 1: The IE of EphemerisInfo in SIB19 which was specified for NTN can be used to inform the location vector of BS in ATG.

Since there will be no new specific bands for ATG, the SIB19 would be broadcasted in legacy TN bands. And the ATG UE should be able to decode SIB19 on ATG bands (TN bands).

Proposal 1: RAN4 to confirm the ATG BS location is broadcasted in SIB19 and the ATG UE should be able to decode SIB19 which is sent on ATG bands (TN bands) to support UE Doppler pre-compensation.

## Power class for ATG UE

In RAN4#104-bis-e, the power class for ATG UE was not decided and there are following WF for ATG terminal’s maximum output power:

Firstly, it’s proposed to define a range of ATG terminal’s maximum output power considering UL budget, UL traffic mode and regulation. Upper boundary: FFS (e.g. fixed value or infinite) Lower boundary: FFS (e.g. 23dBm, 26dBm, 29dBm…..) Secondly, it’s proposed to consider the following options in this R18 WI: Option 1: Specify one value for ATG terminal’s maximum output power from the range. Option 2: specify a set of ATG terminal’s maximum output power from the range. Option 3: specify a range of ATG terminal’s maximum output power Thirdly, it’s proposed to consider the following options for indicating the ATG terminal’s maximum output power Option 1: manufacturer declaration. Option 2: ATG terminal indicate the maximum output power using UE capability.

Specify one or a set of values for ATG UE maximum output power (MoP) is the typical way RAN4 defining the UE MoP requirements. Several UE RF requirements including MPR, A-MPR, emission etc., are related to the maximum output power. If RAN4 is to define a range of ATG UE MoP, it is not clear how to define the other UE RF requirements mentioned above which are closely related to MoP. In addition, it might lead to the issue for regulatory certification if ATG UE indicate the MoP by declaration. With that, we have the following proposal:

Proposal 2: RAN4 to consider specifying one or a set of values for ATG UE maximum output power which is indicated by ATG UE capability.

## Maximum input level

In RAN4#104-bis-e, the following was agreed regarding maximum input level

Further analyse and assume BS output power, minimum distance between BS and UE, couple loss and coexistence output. Then come back to this Maximum input level.

For TN, the BS height of 35m was assumed and -25dBm was defined with the assumptions of min. CL is 70dB. As agreed in the co-existence study discussion, 3000m is the lower boundary for the aircraft altitude. Therefore, the min CL is around 108dB with 3000m distance under FSPL. Assuming the similar transmit power as TN BS in ATG DL, the maximum input level can be relaxed ~30dB.

Proposal 3: RAN4 to consider 30dB relaxation as the starting point for the maximum input level compared with TN requirements.

# Conclusion

In this paper, we provided our views on remaining issues for ATG UE RF requirements. We have the following observation and proposals:

Observation 1: The IE of EphemerisInfo in SIB19 which was specified for NTN can be used to inform the location vector of BS in ATG.

Proposal 1: RAN4 to confirm the ATG BS location is broadcasted in SIB19 and the ATG UE should be able to decode SIB19 which is sent on ATG bands (TN bands) to support UE Doppler pre-compensation.

Proposal 2: RAN4 to consider specifying one or a set of values for ATG UE maximum output power which is indicated by ATG UE capability.

Proposal 3: RAN4 to consider 30dB relaxation as the starting point for the maximum input level compared with TN requirements.

# References

- R4-2217738, “WF on ATG terminal RF requirement”, Huawei, HiSilicon, RAN4#104-bis-e.