3GPP TSG-RAN WG4 Meeting #105	R4-2220573

Toulouse, France, 14 ‒ 18 Nov 2022

Title: 	WF on NTN UE RF requirements

Agenda Item:	     8.23.5

Source: 	ZTE

Document for:	Approval

Agreement

Issue 1-2: The scope of NTN UE in Ka-band from 3GPP perspective

Agreement:

- Define the requirements based on the assumption that the dish antenna is used for GEO and phase antenna for LEO

  - FFS whether to restrict the assumption to Rel-18 or not.

Sub-topic 3 Implementation assumption for NTN VSAT UE

Issue 3-1: IF conversion

Agreement:

- IF conversion is assumed as the baseline for movable NTN UE

Sub-topic 4 RF requirements for NTN UE in Ka-band

Agreement:

- For NTN UE with parabolic antenna, to define the conducted requirement and further discuss how to define radiated requirement for it (e.g. EIRP limits, etc)

Issue 4-2: For NTN UE with phase antenna array, radiated requirement only to be defined?

Agreement:

- For NTN UE with phase antenna array if defined, only radiated requirements are to be specified.