# 3GPP TSG-RAN WG4 Meeting # 106                                   R4-2301053

# Athens, Greece, February 27 – March 3, 2023

# Title: 	Discussion on 4Tx BS demodulation

# Source: 	ZTE Corporation

# Agenda item:	9.6.6.2

# Document for:	Approval

# 1 Introduction

The objectives for 4Tx of further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1) are listed in the following [1]:

Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices [RAN4]

- Investigate framework and architecture Example bands:

  - TDD bands: n41, n77/n78

  - FDD bands: n1

    - Note 1: the total number of example bands should be limited to 3. n77/n78 are considered as one band during the study.

    - Note 2: other bands to be introduced in the release independent way later on from Rel-18

    - Note 3: specifying requirements for TDD bands has first priority

- Specify the UE RF requirements to support 4Tx

  - First priority: 4x4 UL MIMO

  - Second priority: investigate and if necessary specify TxD requirement to support the same power class in UL MIMO and single antenna port

  - PA configuration assumption:

    - First priority: 4x23dBm

    - Second priority: 2x23dBm + 2x26dBm, 4x26dBm

  - UE power class

    - First priority: PC 1.5

    - Second priority: PC2/PC3, and/or new power class if needed

    - Note 1: PC1.5 is only applicable for TDD bands

  - Note: detailed combinations for 2nd priority PA/UE power class assumptions are to be revisited in RAN#97

- NOTE1: Requirements are specified with phase approach. Objectives with 1st priority are considered first.

- NOTE2: It is assumed the devices are equipped with 4Rx antennas for band n1 requirement definition.

In this contribution, we give some initial discussions on performance part related to 4Tx.

# 2	Discussion

In NR Rel-17, the performance requirements of 1Tx and 2Tx for PUSCH have been defined. We need to define the demodulation requirements for 4Tx.

Proposal 1: To update Table 8.2.1.1-1 in 38.104 v18.0.0 for PUSCH performance requirement as Table 2-1

Table 2-1 Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1}, {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

With regard to waveform, we can find that only 1Tx transmission requirements are defined DFT-s-OFDM waveform. The demodulation performance of 4Tx is not considered for DFT-OFDM waveform.

Proposal 2: Not to consider 4Tx PUSCH demodulation requirements for DFT-s-OFDM waveform.

With regard to Minimum requirements for PUSCH with 30% of maximum throughput, we can find that only 1Tx transmission requirements are defined.

Proposal 3: Not to consider 4Tx PUSCH demodulation requirements for Minimum requirements for PUSCH with 30% of maximum throughput.

With regard to requirements for PUSCH, we can find that only 2 MCS are defined for 2Tx transmission requirements. Therefore, a natural choice is to define two MCS for 4Tx as it does for 2Tx.

With regard to channel model for 4Tx requirement, TDLB100-400 Low and TDLC300-100 Low is used for QPSK R=193/1024 and 16QAM R=658/1024 as it does for 1Tx and 2Tx.

Proposal 4: To consider QPSK R=193/1024 and 16QAM R=658/1024 for 4Tx PUSCH demodulation requirements and the channel models are TDLB100-400 Low and TDLC300-100 Low.

4Tx requires a new test case, so we list the FRCs related to 4Tx requirements for reference in Annex. Table 2-2 to table 2-8 list the target SNR for 70% max throughput based on the initial simulation results with the parameters and channel model mentioned above. Figure 2-1 to figure 2-7 show the throughput for corresponding simulation cases.

Table 2-2 5MHz BW 15kHz SCS type A mapping 4 Tx 4Rx 4layer

| MCS | FRC | SNR |
| --- | --- | --- |
| QPSK R=193/1024 | G-FR1-A3-39 | 0.6703 |
| 16QAM R=658/1024 | G-FR1-A4-29 | 20.8204 |

Figure 2-1 Throughput for 5MHz BW 15kHz SCS type A mapping 4 Tx 4Rx 4layer

Table 2-3 10MHz BW 15kHz SCS type A mapping 4 Tx 4Rx 4layer

| MCS | FRC | SNR |
| --- | --- | --- |
| QPSK R=193/1024 | G-FR1-A3-40 | 0.8071 |
| 16QAM R=658/1024 | G-FR1-A4-30 | 21.2190 |

Figure 2-12 Throughput for 10MHz BW 15kHz SCS type A mapping 4 Tx 4Rx 4layer

Table 2-4 20MHz BW 15kHz SCS type A mapping 4 Tx 4Rx 4layer

| MCS | FRC | SNR |
| --- | --- | --- |
| QPSK R=193/1024 | G-FR1-A3-41 | 0.9765 |
| 16QAM R=658/1024 | G-FR1-A4-31 | 21.2538 |

Figure 2-3 Throughput for 20MHz BW 15kHz SCS type A mapping 4 Tx 4Rx 4layer

Table 2-5 10MHz BW 30kHz SCS type A mapping 4 Tx 4Rx 4layer

| MCS | FRC | SNR |
| --- | --- | --- |
| QPSK R=193/1024 | G-FR1-A3-42 | 0.5247 |
| 16QAM R=658/1024 | G-FR1-A4-32 | 23.0575 |

Figure 2-4 Throughput for 10MHz BW 30kHz SCS type A mapping 4 Tx 4Rx 4layer

Table 2-6 20MHz BW 30kHz SCS type A mapping 4 Tx 4Rx 4layer

| MCS | FRC | SNR |
| --- | --- | --- |
| QPSK R=193/1024 | G-FR1-A3-43 | 0.6999 |
| 16QAM R=658/1024 | G-FR1-A4-33 | 23.1079 |

Figure 2-5 Throughput for 20MHz BW 30kHz SCS type A mapping 4 Tx 4Rx 4layer

Table 2-7 40MHz BW 30kHz SCS type A mapping 4 Tx 4Rx 4layer

| MCS | FRC | SNR |
| --- | --- | --- |
| QPSK R=193/1024 | G-FR1-A3-44 | 0.9456 |
| 16QAM R=658/1024 | G-FR1-A4-34 | 23.2998 |

Figure 2-6 Throughput for 40MHz BW 30kHz SCS type A mapping 4 Tx 4Rx 4layer

Table 2-8 100MHz BW 30kHz SCS type A mapping 4 Tx 4Rx 4layer

| MCS | FRC | SNR |
| --- | --- | --- |
| QPSK R=193/1024 | G-FR1-A3-45 | 0.9609 |
| 16QAM R=658/1024 | G-FR1-A4-35 | 24.9617 |

Figure 2-7 Throughput for 100MHz BW 30kHz SCS type A mapping 4 Tx 4Rx 4layer

# 3 Conclusion

In this contribution, we give some discussions for:

Proposal 1: To update Table 8.2.1.1-1 in 38.104 v18.0.0 for PUSCH performance requirement as Table 2-1

Table 2-1 Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1}, {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Proposal 2: Not to consider 4Tx PUSCH demodulation requirements for DFT-s-OFDM waveform.

Proposal 3: Not to consider 4Tx PUSCH demodulation requirements for Minimum requirements for PUSCH with 30% of maximum throughput.

Proposal 4: To consider QPSK R=193/1024 and 16QAM R=658/1024 for 4Tx PUSCH demodulation requirements and the channel models are TDLB100-400 Low and TDLC300-100 Low.

# 4 Reference

[1]  RP-223478 Revised WID Rel-18 NR_ENDC_ RF_FR1_enh2, Huawei, HiSilicon, NTT DOCOMO

# 5 Annex

Table A.3-16: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 4 transmission layer (QPSK, R=193/1024)

| Reference channel | G-FR1-A3-39 | G-FR1-A3-40 | G-FR1-A3-41 | G-FR1-A3-42 | G-FR1-A3-43 | G-FR1-A3-44 | G-FR1-A3-45 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 |
| Payload size (bits) | 5384 | 11280 | 23048 | 5256 | 11016 | 23048 | 59496 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 2 | 3 | 7 | 2 | 3 | 7 | 16 |
| Code block size including CRC (bits) (Note 2) | 2728 | 3792 | 3320 | 2664 | 3704 | 3320 | 3744 |
| Total number of bits per slot | 28800 | 59904 | 122112 | 27648 | 58752 | 122112 | 314496 |
| Total symbols per slot | 3600 | 7488 | 15264 | 3456 | 7344 | 15264 | 39312 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0= 2 and l =11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |  |  |

Table A.4-9: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 4 transmission layer (16QAM, R=658/1024)

| Reference channel | G-FR1-A4-29 | G-FR1-A4-30 | G-FR1-A4-31 | G-FR1-A4-32(Note 3) | G-FR1-A4-33 | G-FR1-A4-34 | G-FR1-A4-35 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 |
| Payload size (bits) | 36896 | 77896 | 155776 | 35856 | 75792 | 155776 | 401640 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 5 | 10 | 19 | 5 | 9 | 19 | 48 |
| Code block size including CRC (bits) (Note 2) | 7408 | 7816 | 8224 | 7200 | 8448 | 8224 | 8392 |
| Total number of bits per slot | 57600 | 119808 | 244224 | 55296 | 117504 | 244224 | 628992 |
| Total symbols per slot | 3600 | 7488 | 15264 | 3456 | 7344 | 15264 | 39312 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0= 2 and l=11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. NOTE 3: The calculation of the “Total number of bits per slot” and “Total symbols per slot” fields include the REs taken up by CSI part 1 and CSI part 2, if present. |  |  |  |  |  |  |  |