3GPP TSG-RAN WG4 Meeting #106	R4-2302067

Athens, Greece, 27 February – 03 March, 2023

Agenda item:	12.3

Source:            	Google Inc.

Title:                	Discussion on Intra-Band EN-DC band combinations

Document for:	Approval

# Introduction

In RAN4#105 meeting, the inconsistency of two configurations Case 3 and Case 4 for intra-band EN-DC band combinations have been discussed. Case 3 is the intra-band contiguous DL EN-DC configuration with intra-band non-contiguous UL EN-DC configuration, and Case 4 is the intra-band non-contiguous DL EN-DC configuration, where LTE and NR adjacent DL carriers are contiguous, with intra-band contiguous and non-contiguous UL EN-DC configuration.

- Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)41AB DC_(n)41CA DC_(n)41DA | DC_41A_n41A |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |

- Case 4: LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, it will have two kinds of UL ENDC configurations:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

The WF has been approved by RAN4 to drop Option 2b and use only Option 2c as the solution to check with RAN2 whether intrabandENDC-support as “both” can be applicable or not.

### Issue 1-1-1: Which solution to be adopted for Case 3

- Option 2b: In Rel-16 and Rel-17, report an additional band combination DC_48A_n48A in both UL and DL to support the Case 3 configurations DL DC_(n)48CA with UL DC_48A_n48A and DL DC_(n)48DA with UL DC_48A_n48A, i.e. - UE indicate “contiguous” capability for DL DC_(n)48CA with UL DC_(n)48AA - UE additionally indicate “non-contiguous” capability for DC_48A_n48A in both UL and DL - Option 2c: In Rel-16 and Rel-17, UE shall also support non-contiguous operation in the DL (DC_48C_n48A), then the network can configure DL_(n)48CA with the middle LTE cell DL-only and the UL with a gap (non-contiguous)

Agreement:

- Drop Option 2b.

- Check with RAN2 whether the interpretation for “both” can be non-contiguous in UL and contiguous in DL.

For Case 4, the agreement has been made in the WF to use Option 2b as the solution, where the intra-band EN-DC sub-blocks is limited to two and one of them consists of a a contiguous EN-DC configuration in Table 5.3B.0-1 in 38.101-3.

### Issue 1-2-1: Which solution to be adopted for Case 4

- Option 2b: Rel-16 and 17 combinations of contiguous and non-contiguous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous EN-DC configuration in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contiguous and non-contiguous EN-DC in the UL, i.e. - UE indicate “both” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A - Option 2c: In Rel-16 and Rel-17, report an additional band combination DC_48A_n48A in both UL and DL to support the Case 4 configuration DL DC_48A_(n)48AA with UL DC_48A_n48A and DC_(n)48AA with UL DC_48A_n48A, i.e. - UE indicate “contiguous” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA - UE additionally indicate “non-contiguous” capability for DC_48A_n48A in both UL and DL - Option 3: New signalling - A solution is necessary in RAN2 to address the ambiguity issue for configurations on some intra-band EN-DC band combinations with more than 2 carriers from Rel-15.

Agreement:

- Option 2b: Rel-16 and 17 combinations of contiguous and non-contiguous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous EN-DC configuration in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contiguous and non-contiguous EN-DC in the UL, i.e.

  - UE indicate “both” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A and DC_48A-48A_n48A with UL DC_48A_n48A

  - The Case 4 is limited to one NR sub-block (band entry) with one or more E-UTRA sub-blocks, the intraBandENDC-Support still indicting the relation between any one of the E-UTRA sub-blocks (band entries) and the single NR sub-block

- Check with RAN2 about option 2b and new signalling is not precluded for case 4.

However, in our opinion, there are still some unclear part in the agreement for case 3 and case 4, which may need to be elaborated. Hence, in this paper, we would like to share our view in the following.

# Discussion

Solution for Case 3

In RAN4#105, the solution for case 3 has been agreed to use Option 2c, which means the UE should also support non-contiguous operation in the DL (DC_48C_n48A) and the network can configure DL_(n)48CA with the middle LTE cell DL-only and the UL with a gap (non-contiguous). However, there are still other band combinations in Case 3 configuration which needs to be considered. Since the Case 3 configurations for b41+n41 has been removed in the previous meeting and Rel-18 specification is also published in 2023-01, we proposed to change “In Rel-16 and Rel-17” with “From Rel-16” and add DC_(n)48DA related information into Option 2c.

Proposal 1: For Case 3, we propose the following elaborated solution for Option 2c

### Issue 1-1-1: Which solution to be adopted for Case 3

- Option 2b: In Rel-16 and Rel-17, report an additional band combination DC_48A_n48A in both UL and DL to support the Case 3 configurations DL DC_(n)48CA with UL DC_48A_n48A and DL DC_(n)48DA with UL DC_48A_n48A, i.e. - UE indicate “contiguous” capability for DL DC_(n)48CA with UL DC_(n)48AA - UE additionally indicate “non-contiguous” capability for DC_48A_n48A in both UL and DL - Option 2c:From Rel-16, UE shall also support non-contiguous operation in the DL (DC_48C_n48A or DC_48D_48A), then the network can configure DL_(n)48CA or DC_(n)48DA with the middle LTE cell DL-only and the UL with a gap (non-contiguous)

Agreement:

- Drop Option 2b.

- Check with RAN2 whether the interpretation for “both” can be non-contiguous in UL and contiguous in DL.

Solution for Case 4

In Table 1, we summarize how to interpret intraBandENDC-Support for Case 4 Option 2b from last meeting agreement. If intraBandENDC-Support is indicated as “contiguous(not reported)”, the UE can support DL DC_48A-(n)48AA with UL DC_(n)48AA. If intraBandENDC-Support is indicated as “non-contiguous”, the UE can support DL DC_48A-48A_n48A with UL DC_48A_n48A. If intraBandENDC-Support is indicated as “both”, the UE can support support DL DC_48A-(n)48AA with UL DC_(n)48AA, support DL DC_48A-(n)48AA with DC_48A_n48A, and support DL DC_48A-48A_n48A with UL DC_48A_n48A.

| Case 4 Configuration for b48+n48 (LTE:CA_48A-48A, NR:n48A) |  |  |  |
| --- | --- | --- | --- |
| Solution | intraBandENDC-Support | DL EN-DC Configuration | UL EN-DC Configuration |
| Option 2b | contiguous | DC_48A-(n)48AA | DC_(n)48AA |
|  | non-contiguous | DC_48A-48A_n48A | DC_48A_n48A |
|  | both | DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |
|  |  | DC_48A-48A_n48A | DC_48A_n48A |

Table 1. intraBandENDC-Support for b48+n48 Case 4 configuration

In our view point, Option 2b for Case 4 has the condition that intra-band EN-DC configuration needs to be limited to two sub-blocks and one of which consists of a contiguous EN-DC configuration in table Table 5.3B.0-1 in 38.101-3. However, the first sub-bullet in Option 2b of Case 4 agreement includes DC_48A-48A_n48A with UL DC_48A_n48A where DC_48A-48A_n48A is a three sub-blocks case. Besides, the second sub-bullet in Option 2b also indicates that the Case 4 is limited to one NR sub-block with one or more E-UTRA sub-blocks, which is not aligned with total two sub-blocks for intra band EN-DC in the main bullet of Option 2b. Furthermore, the second sub-bullet also describes that the intraBandENDC-Support still indicting the relation between any one of the E-UTRA sub-blocks (band entries) and the single NR sub-block, which implies that DL DC_48A-48A_n48A with UL DC_48A_n48A can be supported by indicating intraBandENDC-Support=non-contiguous. Considering “both” means “contiguous” and “non-contiguous”, it is reasonable for the UE to indicate intraBandENDC-Support as “both” that the UE should also support DL DC_48A-48A_n48A with UL DC_48A_n48A.

In order to align the first main bullet with two sub bullets in Case 4 agreement, we propose to modify the Case 4 agreement as follows

- For the first bullet

  - Modify “Rel-16 and Rel-17” with “From Rel-16”

- For the first sub bullet

  - remove DC_48A-48A_n48A with UL DC_48A_n48A

- For the second bullet

  - modify “one NR sub-block (band entry) with one or more E-UTRA sub-blocks” with “one NR sub-block (band entry) with two E-UTRA sub-blocks”

- Add the new bullet to clarify three sub-blocks case

  - If “both” capability for Case 4 is indicated, the UE should also support DL DC_48A-48A_n48A with UL DC_48A_n48A

Proposal 2: For Case 4, we propose the following elaborated agreement

Agreement:

- Option 2b: From Rel-16, bandcombinations of contiguous and non-contiguous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous EN-DC configuration in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contiguous and non-contiguous EN-DC in the UL, i.e.

  - UE indicate “both” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A

  - The Case 4 is limited to one NR sub-block (band entry) with two E-UTRA sub-blocks, the intraBandENDC-Support still indicting the relation between any one of the E-UTRA sub-blocks (band entries) and the single NR sub-block

- If “both” capability for Case 4 is indicated, the UE should also support DL DC_48A-48A_n48A with UL DC_48A_n48A.

- Check with RAN2 about option 2b and new signalling is not precluded for case 4.

# Conclusion

The observations and proposals in this contribution are summarized in the following.

Proposal 1: For Case 3, we propose the following elaborated solution for Option 2c

### Issue 1-1-1: Which solution to be adopted for Case 3

- Option 2b: In Rel-16 and Rel-17, report an additional band combination DC_48A_n48A in both UL and DL to support the Case 3 configurations DL DC_(n)48CA with UL DC_48A_n48A and DL DC_(n)48DA with UL DC_48A_n48A, i.e. - UE indicate “contiguous” capability for DL DC_(n)48CA with UL DC_(n)48AA - UE additionally indicate “non-contiguous” capability for DC_48A_n48A in both UL and DL - Option 2c:From Rel-16, UE shall also support non-contiguous operation in the DL (DC_48C_n48A or DC_48D_48A), then the network can configure DL_(n)48CA or DC_(n)48DA with the middle LTE cell DL-only and the UL with a gap (non-contiguous)

Agreement:

- Drop Option 2b.

- Check with RAN2 whether the interpretation for “both” can be non-contiguous in UL and contiguous in DL.

Proposal 2: For Case 4, we propose the following elaborated agreement

Agreement:

- Option 2b: From Rel-16, bandcombinations of contiguous and non-contiguous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous EN-DC configuration in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contiguous and non-contiguous EN-DC in the UL, i.e.

  - UE indicate “both” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A

  - The Case 4 is limited to one NR sub-block (band entry) with two E-UTRA sub-blocks, the intraBandENDC-Support still indicting the relation between any one of the E-UTRA sub-blocks (band entries) and the single NR sub-block

- If “both” capability for Case 4 is indicated, the UE should also support DL DC_48A-48A_n48A with UL DC_48A_n48A.

- Check with RAN2 about option 2b and new signalling is not precluded for case 4.

# Reference

- R4-2220125, “Topic summary for [105][145] RAN_task_UERF_part2”, RAN4#105, Moderator (OPPO)

- R4-2220589, “WF on intra-band EN-DC support capability”, RAN4#105, OPPO

- R4-2220837, “LS on intra-band EN-DC combination”, RAN4#105, Huawei, HiSilicon

- R4-2218284, “Discussion on Intra-Band EN-DC band combinations”, RAN4#105, Google Inc., CableLabs, Federated Wireless