3GPP TSG-RAN WG4 Meeting # 106	R4-2302737

# Athens, Greece, February 27 – March 3, 2022

Agenda item:	9.18.2

Source:	Apple Inc.

Title:	On Requirements for Application Layer Throughput

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#105 requirements for physical layer throughput were discussed and way forward [1] was approved. It was encouraged to bring simulation results for alignment. In this paper simulation results for alignment.

# 2. Simulation Results

In RAN4#105 it was agreed that companies bring simulation results for alignment in RAN4#106. For the simulation parameters in the table below we provide we simulation results in the sub-sections below.

Table 1: Simulation Parameters for Physical layer throughput

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 4 | 1 4 | 1 4 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

FR1 FDD

Table 2: Simulation results for FDD 2x2

| SNR (dB) | HARQ Re-Tx Disable |  | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | TP (Mbps) | BLER |  |  |
| 0 | 3.1 | 0.08 | 3.2 | 0.07 | 3 | 1 |
| 2 | 4.2 | 0.10 | 4.4 | 0.08 | 3 | 1 |
| 4 | 5.9 | 0.09 | 6.2 | 0.07 | 4 | 1 |
| 6 | 7.9 | 0.10 | 8.2 | 0.07 | 5 | 1 |
| 8 | 10.1 | 0.06 | 10.3 | 0.05 | 6 | 1 |
| 10 | 12.3 | 0.06 | 12.5 | 0.05 | 6 | 1 |
| 12 | 14.8 | 0.07 | 15.2 | 0.06 | 7 | 1 |
| 14 | 17.5 | 0.09 | 18.1 | 0.07 | 7 | 1 |
| 16 | 20.7 | 0.08 | 21.2 | 0.06 | 8 | 2 |
| 18 | 23.4 | 0.10 | 24.2 | 0.08 | 8 | 2 |
| 20 | 26.3 | 0.12 | 27.6 | 0.09 | 9 | 2 |

Table 3: Simulation results for FDD 2x4

| SNR (dB) | HARQ Re-Tx Disable |  | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | TP (Mbps) | BLER |  |  |
| 0 | 3.8 | 0.34 | 5.0 | 0.23 | 4 | 1 |
| 2 | 6.6 | 0.19 | 7.2 | 0.14 | 4 | 1 |
| 4 | 7.7 | 0.19 | 8.8 | 0.13 | 4 | 2 |
| 6 | 10.8 | 0.18 | 11.9 | 0.13 | 4 | 2 |
| 8 | 14.7 | 0.15 | 15.8 | 0.10 | 5 | 2 |
| 10 | 19.2 | 0.09 | 20.0 | 0.07 | 6 | 2 |
| 12 | 23.0 | 0.08 | 23.6 | 0.06 | 7 | 2 |
| 14 | 26.8 | 0.09 | 27.8 | 0.07 | 8 | 2 |
| 16 | 31.2 | 0.11 | 32.8 | 0.07 | 9 | 2 |
| 18 | 36.6 | 0.09 | 37.8 | 0.07 | 10 | 2 |
| 20 | 41.4 | 0.08 | 42.5 | 0.07 | 11 | 2 |

FR1 TDD

Table 4: Simulation results for TDD 2x2

| SNR (dB) | HARQ Re-Tx Disable |  | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | TP (Mbps) | BLER |  |  |
| 0 | 8.8 | 0.06 | 9.2 | 0.04 | 3 | 1 |
| 2 | 12.1 | 0.08 | 12.7 | 0.06 | 3 | 1 |
| 4 | 17.5 | 0.07 | 18.1 | 0.05 | 4 | 1 |
| 6 | 23.6 | 0.08 | 24.4 | 0.06 | 5 | 1 |
| 8 | 30.6 | 0.04 | 30.9 | 0.04 | 5 | 1 |
| 10 | 37.7 | 0.04 | 38.2 | 0.03 | 6 | 1 |
| 12 | 45.7 | 0.04 | 46.3 | 0.04 | 7 | 1 |
| 14 | 53.7 | 0.07 | 54.7 | 0.05 | 7 | 2 |
| 16 | 63.3 | 0.07 | 64.8 | 0.05 | 8 | 2 |
| 18 | 72.6 | 0.08 | 75.2 | 0.05 | 8 | 2 |
| 20 | 82.0 | 0.10 | 84.9 | 0.08 | 8 | 2 |

Table 5: Simulation results for TDD 2x4

| SNR (dB) | HARQ Re-Tx Disable |  | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | TP (Mbps) | BLER |  |  |
| 0 | 13.1 | 0.22 | 15.5 | 0.16 | 3 | 1 |
| 2 | 21.2 | 0.14 | 22.7 | 0.10 | 4 | 1 |
| 4 | 26.7 | 0.07 | 27.9 | 0.05 | 4 | 2 |
| 6 | 34.3 | 0.13 | 36.5 | 0.10 | 4 | 2 |
| 8 | 48.1 | 0.09 | 49.7 | 0.07 | 5 | 2 |
| 10 | 61.5 | 0.05 | 62.5 | 0.04 | 6 | 2 |
| 12 | 75.5 | 0.02 | 75.6 | 0.02 | 7 | 2 |
| 14 | 87.8 | 0.05 | 89.3 | 0.04 | 8 | 2 |
| 16 | 104.9 | 0.02 | 105.4 | 0.02 | 9 | 2 |
| 18 | 116.0 | 0.06 | 118.4 | 0.05 | 10 | 2 |
| 20 | 125.5 | 0.10 | 130.7 | 0.07 | 11 | 2 |

FR2

Table 6: Simulation results for FR2 TDD 2x2

| SNR (dB) | HARQ Re-Tx Disable |  | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | TP (Mbps) | BLER |  |  |
| 0 | 14.7 | 0.28 | 17.1 | 0.21 | 5 | 1 |
| 2 | 20.7 | 0.23 | 22.9 | 0.18 | 6 | 1 |
| 4 | 28.6 | 0.18 | 30.9 | 0.14 | 7 | 1 |
| 6 | 37.2 | 0.17 | 40.1 | 0.14 | 8 | 1 |
| 8 | 43.8 | 0.22 | 47.9 | 0.17 | 9 | 1 |
| 10 | 51.6 | 0.23 | 57.1 | 0.17 | 9 | 1 |
| 12 | 65.6 | 0.20 | 71.2 | 0.15 | 9 | 2 |
| 14 | 80.4 | 0.17 | 86.4 | 0.13 | 9 | 2 |
| 16 | 93.1 | 0.17 | 100.0 | 0.13 | 9 | 2 |

# 4. Conclusion

In this paper, we provide simulation results for alignment for defining physical layer throughput requirements.

# Reference

- R4-2220276, “Way forward on physical layer throughput requirements”, Intel