3GPP TSG-RAN WG4 Meeting #106	R4-2303484

Athens, Greece, February 27 – March 3, 2023

Title: 	WF on NR-U enhancement

Agenda Item:	8.13.6

Source: 	Apple

Document for:	Approval

# PC3 MPR allowance

## 1Tx MPR

<Agreement>: PC3 MPR for 1Tx is agreed. Values are the result from averaging company contributions and rounding up and down to the closest 0.5dB.

PC3 MPR for 1Tx

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.0 | ≤ 1.5 |
|  | QPSK | ≤ 1.0 | ≤ 2.0 |
|  | 16 QAM | ≤ 1.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 2.0 | ≤ 3.0 |
|  | 256 QAM | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 2.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 2.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 4.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

## 2Tx MPR

<Agreement>: The values are agreed with brackets for further checking. Companies are asked to provide analysis and revision if issues are encountered.

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ [2.0] | ≤ [3.0] |
|  | QPSK | ≤ [2.0] | ≤ [3.0] |
|  | 16 QAM | ≤ [2.5] | ≤ [3.0] |
|  | 64 QAM | ≤ [2.5] | ≤ [3.5] |
|  | 256 QAM | ≤ [4.5] | ≤ [4.5] |
| CP-OFDM | QPSK | ≤ [3.5] | ≤ [4.0] |
|  | 16 QAM | ≤ [3.5] | ≤ [4.0] |
|  | 64 QAM | ≤ [4.5] | ≤ [4.5] |
|  | 256 QAM | ≤ [6.5] | ≤ [6.5] |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

# PC3 A-MPR allowance

## A-MPR for NS_30 (1Tx)

<Agreement>: The PC3 1Tx A-MPR for NS_30 is agreed.

- For 100MHz channel bandwidth, the A-MPR would be added in future. This aspect is captured by Note 5.

PC3 1Tx A-MPR for NS_30 power class 3

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-OFDM | PI/2 BPSK | ≤ 12.0 | ≤ 15.5 | ≤ 3.5 | ≤ 5.5 | See Table MPR table for PC3 |
|  | QPSK | ≤ 12.5 | ≤ 16.0 | ≤ 4.0 | ≤ 6.5 |  |
|  | 16 QAM | ≤ 12.5 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 13.0 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 13.0 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
| CP-OFDM | QPSK | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 16 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 256 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.5 | ≤ 7.5 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5230 and 5270 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 2 and NOTE 3. |  |  |  |  |  |  |

## A-MPR for NS_54 (1Tx)

<Agreement>: The PC3 1Tx A-MPR for NS_54 is agreed.

PC3 1Tx A-MPR for NS_54

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK | See Table PC3 MPR | ≤ 2.0 | ≤ 4.5 |
|  | QPSK |  | ≤ 3.0 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 5.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.0 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3 and NOTE 5. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |

## A-MPR for South Korea LPI

<Agreement>:

- NR-U PC3 A-MPR for 1Tx and 2Tx is defined separately in South Korea.

- Brackets for NR-U PC3 A-MPR with 2Tx (2x20dBm) for both antenna isolation of 10dB and 16dB in South Korea are removed.

- Values are harmonized values so that 2Tx A-MPR is not lower than 1Tx.

## Discussion on Full and Partial RB

From the tables,

Full RB allocation is,

- when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2 (MPR)

- when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to configuration [B] in Table 6.2F.2-2 (MPR)

- when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted (A-MPR)

Partial RB allocation is,

- when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2 (MPR)

- when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted (A-MPR)

- when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted (A-MPR)

Proposal made on RAN4#106:

Full RB allocation and Partial RB allocation need to be updated with consistency between MPR and A-MPR:

    - Full RB allocation:

      - when all RB’s in a 20 MHz channel, or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted, or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configurations in Table 6.2F.2-2

    - Partial RB allocation:

      - when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted or when sub-bands are transmitted according to configurations in Table 6.2F.2-2

<Way Forward>: Companies are requested to provide further input next meeting.

# Network Signalling Harmonization and Enhancement

Legacy device handling

To ensure correct functioning of the legacy UEs when an "extended" IE is signalled, RAN WG2 suggests using the highest value "7" as “RESERVED”. The rationale behind it as that if the network signals one of the NS values from the extended range in the corresponding extended IE, legacy UEs should be somehow informed about it because they cannot process and decode the extended IE. Thus, the highest value "7" is used as an implicit indicator that some NS value is present in the extended space. And the legacy UEs do not need to know which extended NS value it is – they will just receive value "7" and will act on it as unsupported NS value.

<Agreement>:

- RAN4 shall take into account the extended range of NS values introduced by RAN2.

- Label the highest NS value "7" as "RESERVED" for all existing NS values.

Legacy device handling

If the extended NS range is enabled starting from Rel-16, then there could be Rel-16 devices implementing NR-U, which by the time when they are released to the market do not fully support all NS flags defined in e.g. Rel-18 or even later releases. A Rel-16 NR-U device may support Rel-17 NS flags, but not Rel-18 ones. Since the Rel-18 specification work is not over, there can be a Rel-16 NR-U devices that can support Rel-16 and Rel-17 flags, but not Rel-18 ones. Otherwise, if the network does not know which NS flags are supported by the UE, then a UE implementing e.g. Rel-17 band n102, but not the Rel-18 flags, will reject the RRC message configuring the secondary cell on the shared access spectrum band.

The most straightforward approach to enable the corresponding signalling is to consider modifiedMPR-Behaviour field that can indicate which NS flags are supported. In fact, some bands already use this field to indicate which NS flags are supported by the UE.

As a baseline approach, a UE will support (at least) those NR-U NS flags that are defined in the corresponding release supported by the UE.

<Way Forward>: Companies are asked to consider the following options and bring considerations and proposals next meeting.

- Option 1: Consider defining the corresponding values in modifiedMPR-Behaviour field to indicate that an earlier release UE supports band n96/n102 NS flags defined in later releases.

- Option 2: Discuss further whether we need separate values in modifiedMPR-Behaviour field to indicate separately support of the NS flags introduced in Rel-17 and Rel-18.

Harmonization of Kenya, Qatar and Jordan

<Agreement>: Requirements of Kenya, Qatar and Jordan are harmonized as follows:

- Re-use 3GPP band n102 for the LPI/VLP operation in Kenya, Qatar and Jordan.

- For the LPI operation in Kenya, Qatar and Jordan NS_01 can be re-used.

- For the VLP operation in Qatar and Jordan, the corresponding "flat 6dB back-off" NS flag can be re-used.

- For the VLP operation in Kenya, same NS flag as for e.g. Australia and New Zealand can be re-used.

Harmonization of Dominican Republic

<Agreement>: Requirements of Dominican Republic are harmonized as follows:

- Re-use 3GPP band n96 for the LPI/VLP operation in Dominican Republic.

- For the LPI operation in Dominican Republic, same NS flag as for South Korea, NS_60, can be re-used.

- For the VLP operation in Dominican Republic, same NS flag as for e.g. Canada can be re-used.

Aspects for Japan Requirements

<Agreement>: Requirements for Japan need separate NS flags to support the LPI and VLP modes.

Summary of NS values for PC5

<Agreement>: The updated summary of NS values for PC5 is endorsed.

Updated NS values for PC5.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | [NS_y1] |
| UK | N/A | NS_01 | [NS_y2] |
| Morocco | N/A | NS_01 | [NS_y2] |
| UAE | N/A | NS_01 | N/A |
| Saudi Arabia | N/A | NS_01 | N/A |
| Kenya | N/A | NS_01 | [NS_y5] |
| Qatar | N/A | NS_01 | [NS_y2] |
| Jordan | N/A | NS_01 | [NS_y2] |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | N/A |
| Canada | NS_54 | NS_59 | [NS_y3] |
| Brazil | N/A | NS_53 | [NS_y4] |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | [NS_y2] |
| Colombia | N/A | NS_53 | N/A |
| Dominican Republic | N/A | [NS_60] | [NS_y3] |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |
| Hong Kong | N/A | NS_58 | [NS_y1] |
| Australia | N/A | NS_01 | [NS_y5] |
| New Zealand | N/A | NS_01 | [NS_y5] |
| Malaysia | N/A | NS_01 | [NS_y2] |
| Japan | N/A | [NS_x1] | [NS_y6] |

# Intra-band contiguous UL CA

In Release 17 the CA_n96B/C UL CA and configurations are specified. Even so, it was clarified in RAN that although NR-U ULCA requirements were completed, NR-U ULCA could not be deployed since the A-MPR for n96B/C UL was not specified, especially for the U.S. NS_53 and NS_54.

One of the reasons for this delay is the significant effort that is required due to the number of cases to be studied even compared to most NR A-MPR cases:

- CP and DFT waveforms

- All modulation orders

- Full and interlaced waveforms

- Wideband operation full and partial sub-bands

- And combinations of the above within the two CCs of the ULCA configurations

- The number of NS to cover for n46, n96 and n102.

<Agreement>:

- Further work on missing UL CA requirements is treated in Rel-18 basket not being part of block approval.

- Proponents request intra-band UL CA combinations and corresponding network signalling for PC5 to be included into the basket.

- Companies are encouraged to provide simulation and measurement results.