3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304073
Online, April 17 – April 26, 2023

Agenda item:	5.17.3

Source:	MediaTek Beijing Inc.

Title:	Research on FR1 MIMO OTA noise impact at low band

Document for:	Discussion

1	Introduction

It was agreed to study the noise impact in the chamber for bands below 1GHz, CAICT had shown their experiment results in [1]. To clarify the noise impact at low bands, similar experiments were done at MTK lab.

2	Discussion

We use the typical system setup as shown in Fig. 1. Power control units (PCUs), functioning as adjustable attenuators, are added after each port of the amplifier. Switching the attenuators with 10dB step, keep other variables unchanged. The measurement results at band n28 (780.5MHz) are shown in Fig. 2.

Fig. 1  The system setup for FR1 MIMO OTA testing (The PCUs function as adjustable attenuators.)

Fig. 2  Measured MIMO OTA performance with different attenuation values

From Fig.2, we can see the similar phenomenon in [1]. As the attenuation increases, the measurement results first get better and then become stable. From the results, 20dB attenuation should be added in our chamber to make the test results more stable.

Observation 1:  Suitable attenuation after amplifiers may exclude some uncertainties such as amplifier noise impact.

Proposal 1: Labs should do the noise impact experiments to eliminate the noise impact at low bands. Suitable attenuation may need to be added in the system setup.

3	Conclusion

In this paper, we show the experiment results to clarify the noise impact at low bands.

Observation 1:  Suitable attenuation after amplifiers may exclude some uncertainties such as amplifier noise impact.

Proposal 1: Labs should do the noise impact experiments to eliminate the noise impact at low bands. Suitable attenuation may need to be added in the system setup.

References

- R4-2301049, “On noise impact and FR1 lab alignment”, CAICT, 3GPP RAN4#106, Feb. 2023.