3GPP TSG-RAN WG4 Meeting # 106bis-e		R4-2304152

# Online, April 17 – April 26, 2023

Agenda item:	5.8.3.4

Source:	Apple

Title:	On scheduling/measurement restrictions for multiple Rx chains

Document for:	Discussion

# Introduction

In RAN4#106 meeting, the WF[1] for NR FR2 multi-Rx chain DL reception – Part 2 was agreed, and there were open issues as followings,

Open issues for Scheduling/Measurement restrictions are captured in section for Topic #3 in topic summary [R4-2302767].

In this contribution, we discuss the open issues for scheduling and measurement restriction for UE with FR2 multi-Rx chain DL reception capability.

# Discussion on measurement restriction

The existing measurement restriction is specified as followings (using RLM as one example):

#### 8.1.2.3 Measurement restrictions for SSB based RLM [unrelated part omitted] For FR2, when the SSB for RLM measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for RLM and CSI-RS. Longer measurement period for SSB based RLM is expected, and no requirements are defined. [unrelated part omitted]

#### 8.1.3.3 Measurement restrictions for CSI-RS based RLM [unrelated part omitted] For FR2, when the CSI-RS for RLM measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for RLM and SSB. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined. For FR2, when the CSI-RS for RLM measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, - In the following cases, UE is required to measure one of but not both CSI-RS for RLM and the other CSI-RS. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined. - The CSI-RS for RLM or the other CSI-RS in a resource set configured with repetition ON, or - The other CSI-RS is configured in q1 and beam failure is detected, or - The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE, - Otherwise, UE shall be able to measure the CSI-RS for RLM without any restriction. [unrelated part omitted]

Based on the existing measurement restriction requirement, three categories of RS shall be considered when defining measurement restriction requirement:

(1) The SSB for RLM, for BFD/CBD/L1-RSRP are all with beam sweeping

(2) The CSI-RS with beam sweeping: CSI-RS for CBD, CSI-RS for L1-RSRP with repetition on.

(3) The CSI-RS without beam sweeping: CSI-RS for RLM, CSI-RS for BFD, CSI-RS for L1-RSRP with repetition off.

The existing measurement restriction is applied as shown in following table 1. “Yes” means measurement restriction is applied, otherwise “No” means no measurement restriction is applied and parallel measurement shall be performed.

Table 1. Summary for existing measurement restriction applicability

| Scheduling Restriction Applicability | SSB RLM/BFD/CBD/L1-RSRP | CSI-RS RLM/BFD/L1-RSRP(repetition OFF) | CSI-RS CBD/L1-RSRP(repetition ON) |
| --- | --- | --- | --- |
| SSB RLM/BFD/CBD/L1-RSRP | NA | Yes | Yes |
| CSI-RS RLM/BFD/L1-RSRP(repetition OFF) | Yes | QCLed typeD: No Non-QCLed typeD: Yes | Yes |
| CSI-RS CBD/L1-RSRP(repetition ON) | Yes | Yes | Yes |

Thus, the current measurement restriction applies based on the following main principles:

- At least one of the RS needs beam sweeping (e.g., SSB, or CSI-RS for CBD, or CSI-RS for L1-RSRP with repetition ON), or

- These two RSs are not QCLed type D

Observation: the existing measurement restriction applies based on the following main principles:

- At least one of the RS needs beam sweeping (e.g., SSB for RLM/BFD/CBD/L1-RSRP, or CSI-RS for CBD, or CSI-RS for L1-RSRP with repetition ON), or

- These two RSs are not QCLed type D

For multiple-Rx chain capable UE whose multiple-Rx chain are enabled, if UE has reported the group based L1-RSRP or L1-SINR for these two RSs, that means UE is able to receive or measure these two RSs simultaneously. In RAN1 spec TS38.214 section 5.2.1.4.2, it is specified that,

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb- Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index', - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting. - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters.

In TS38.331, the CSI-ReportConfig for group reporting is defined as following,

csi-SSB-ResourceSetList, csi-SSB-ResourceSetListExt List of references to SSB resources used for CSI measurement and reporting in a CSI-RS resource set (see TS 38.214 [19], clause 5.2.1.2). The csi-SSB-ResourceSetListExt provides additional references and can only be configured if csi-SSB-ResourceSetList is configured and groupBasedBeamReporting-v1710 is configured in the CSI- ReportConfig that indicates this CSI-ResourceConfig as resourcesForChannelMeasurement. If groupBasedBeamReporting-v1710 is configured in the IE CSI-ReportConfig that indicates this CSI-ResourceConfig as resourceForChannelMeasurement, the network configures 2 resource sets, which may be two NZP CSI-RS resource sets, two CSI SSB resource sets or one NZP CSI-RS resource set and one CSI-SSB resource set (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B: - if the list has one CSI-SSB resource set, this resource set is indicated by a resource set indicator set to 1, while the resource set indicator of the NZP CSI-RS resource set is 0; - if the list has two CSI-SSB resource sets, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1.

Thus, no matter SSB or CSI-RS and no matter ‘with Rx beam sweeping’ or ‘without Rx beam sweeping’, as long as these two RSs for RLM/CBD/BFD/L1-RSRP has been reported in a pair of grouped CSI reporting, no measurement restriction shall be applied for multiple-Rx chain capable UE whose multiple-Rx chain are enabled.

Another possibility is, even though UE has not reported such paired RS for grouped CSI reporting, UE can still not need measurement restriction if the separation between the Rx beams for those RS is large enough. However, the separation is probably up to UE implementation and therefore how to specify it in requirement needs FFS and needs more conclusion from RF session discussion. One example is shown in the figure 1.

Figure 1. spatial separation between RS for L1 measurement

Proposal 1: For measurement restriction for L1 measurement (RLM/BFD/CBD/L1-RSRP), if UE supports multi-Rx chain and the multi-Rx chain is enabled:

- if the RS(SSB or CSI-RS)  for RLM/BFD/CBD/L1-RSRP and another RS(SSB or CSI-RS)  for RLM/BFD/CBD/L1-RSRP has been paired in a grouped CSI reporting(e.g., grouped L1-RSRP or L1-SINR), no measurement restriction shall be applied.

- if the spatial separation is large enough between the RS (SSB or CSI-RS) for RLM/BFD/CBD/L1-RSRP and another RS (SSB or CSI-RS) for RLM/BFD/CBD/L1-RSRP, no measurement restriction shall be applied. Whether and how to specify it is FFS and needs more conclusions from RF session.

# Discussion on scheduling restriction

3.1 Scheduling restriction for L1 measurement (RLM/BFD/CBD/L1-RSRP)

As discussed in section 2, for L1 measurement following three categories are considered:

(1) The SSB for RLM, for BFD/CBD/L1-RSRP are all with beam sweeping

(2) The CSI-RS with beam sweeping: CSI-RS for CBD, CSI-RS for L1-RSRP with repetition on.

(3) The CSI-RS without beam sweeping: CSI-RS for RLM, CSI-RS for BFD, CSI-RS for L1-RSRP with repetition off.

The existing scheduling restriction for L1 measurement is applied as shown in following table 2. “Yes” means scheduling restriction is applied, otherwise “No” means no scheduling restriction is applied.

Table 2. Summary for existing scheduling restriction applicability for L1

|  | SSB RLM/BFD/CBD/L1-RSRP | CSI-RS RLM/BFD/L1-RSRP(repetition OFF) | CSI-RS CBD/L1-RSRP(repetition ON) |
| --- | --- | --- | --- |
| Scheduling Restriction Applicability | Yes | QCLed-typeD with TCI state for PDCCH or PDSCH: No Non-QCLed-typeD with TCI state for PDCCH or PDSCH: Yes | Yes |

Similar as measurement restriction, no matter SSB or CSI-RS and no matter ‘with Rx beam sweeping’ or ‘without Rx beam sweeping’, as long as the RS for RLM/CBD/BFD/L1-RSRP and the CSI-RS for TCI of PDCCH or PDSCH has been reported in a pair of grouped CSI reporting, no scheduling restriction shall be applied for multiple-Rx chain capable UE whose multiple-Rx chain are enabled. In this case, network is aware of which RS pair is supported by this UE because of the grouped CSI reporting and therefore there no need to have new signaling to indicate such scheduling restriction applicability from UE to network.

Another possibility is also similar as measurement restriction, even though UE has not reported such paired RS for grouped CSI reporting, UE can still not need scheduling restriction if the spatial separation between the Rx beams for L1 RS and PDSCH/PDCCH is large enough. However, the spatial separation is probably up to UE implementation and therefore UE may need to indicate to network to on which measurement occasion UE has no scheduling restriction. Thus, in our view, RAN4 can FFS on whether and how to indicate network the scheduling restriction information due to spatial separation and needs more conclusion from RF session discussion.

Proposal 2: For scheduling restriction for L1 measurement (RLM/BFD/CBD/L1-RSRP), if UE supports multi-Rx chain and the multi-Rx chain is enabled:

- if the RS(SSB or CSI-RS) for RLM/BFD/CBD/L1-RSRP and the CSI-RS for TCI of PDCCH or PDSCH has been paired in a grouped CSI reporting(e.g., grouped L1-RSRP or L1-SINR), no scheduling restriction shall be applied on this RS based RLM/BFD/CBD/L1-RSRP measurement.

- if the spatial separation is large enough between the RS (SSB or CSI-RS) for RLM/BFD/CBD/L1-RSRP and the CSI-RS for TCI of PDCCH or PDSCH, no scheduling restriction shall be applied on this RS based RLM/BFD/CBD/L1-RSRP measurement. Whether and how to indicate network the scheduling restriction information due to spatial separation is FFS, and needs more conclusions from RF session.

3.2 Scheduling restriction for L3 measurement

For SSB based L3 measurement for mobility, the beam sweeping is always assumed on thew whole spherical area to detect the potential neighbor cells’ SSBs, and on those SSBs for L3 measurement, the scheduling restriction is applied. However, since the multi-Rx chain UE now has capability of simultaneous reception on different AoAs, it would be possible to avoid scheduling restriction on some of the L3 measurement occasions. As shown in the following figure 2 as an example, the rough Rx beam sweeping on right panel will not cause interruption to the data/control channel reception on the left panel.

Figure 2. Example for scheduling restriction on L3 measurement

However, the only difficult part is that network is not aware of which L3 measurement occasions can be used for scheduling. We do observe the benefit to enhance the throughput for the UE to mute the scheduling restriction on some of the L3 measurement occasions, e.g., in the figure 2 case, 50% of throughput loss can be saved compared with legacy requirement. Since L3 measurement is periodic and quite static (RRC configuration based), RAN4 can further discuss how to indicate the scheduling restriction information to network, and network can use the some of the L3 measurement occasions for scheduling. Details of indication and signaling can be FFS.

Proposal 3: For scheduling restriction for L3 measurement, if UE supports multi-Rx chain and the multi-Rx chain is enabled:

- if the spatial separation is large enough between the RS for L3 measurement and the CSI-RS for TCI of PDCCH or PDSCH, no scheduling restriction shall be applied on this L3 measurement occasions. UE may need to indicate scheduling restriction information to network. Details of indication and signaling can be FFS, and needs more conclusions from RF session.

# Grouped CSI reporting for SSB+CSI-RS

Regarding the measurement and scheduling restriction for L1 measurement (RLM/BFD/CBD/L1-RSRP) in proposal 1 and 2, we mentioned one configuration of grouped CSI reporting for SSB+CSI-RS. However, we also identified some potential issue in the current RAN1 and RAN2 spec. In the current TS38.331, the CSI resource set can be configured as followings,

And in TS38.214, it was defined that:

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb- Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index', - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting. - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters.

It could be possible that one resource set contains CSI-RS and the other resource set contains SSB for the CSI measurement. But when we looked into the reporting configuration, it shows that the reporting quantity only supports grouped CSI reporting based on one single type of RS, i.e., either CSI-RS or SSB. The definition in TS38.331 is:

With such definition in the RAN1/RAN2 spec, even though network configured paired resource to UE for group reporting (CSI-RS set + SSB set) and UE can measure SSB and CSI-RS from different TRPs simultaneously, UE is not able to report this group results to network. In our view, this reporting is very important for network to understand which pair of RS in the candidate RS sets can be measured/received by UE simultaneously and then network can use corresponding Tx beams to serve this UE (UE can use two Rx beams to match those two Tx beams), as shown in the following figure. We think it’s desirable to also support the grouped CSI reporting for mixed RSs(i.e., CSI-RS+SSB), and the corresponding signaling in reporting quantity shall be added. Since such enhancement needs RAN1/2 expertise, we recommend to check with RAN1/2 about the feasibility for the grouped CSI reporting of mixed RSs(i.e., CSI-RS+SSB).

Figure 3. mismatch between resource set configuration and grouped CSI reporting

Proposal 4: send LS to RAN1 and RAN2 to check the feasibility for the R18 grouped CSI reporting of mixed RSs(i.e., CSI-RS+SSB), i.e., whether reportQuantity in R18 can be enhanced to support grouped CSI reporting of CSI-RS+SSB based L1 measurement results.

# Conclusion

In this contribution, we discuss the measurement and scheduling restriction for multi-Rx chain capable UE.

Observation: the existing measurement restriction applies based on the following main principles:

- At least one of the RS needs beam sweeping (e.g., SSB for RLM/BFD/CBD/L1-RSRP, or CSI-RS for CBD, or CSI-RS for L1-RSRP with repetition ON), or

- These two RSs are not QCLed type D

Proposal 1: For measurement restriction for L1 measurement (RLM/BFD/CBD/L1-RSRP), if UE supports multi-Rx chain and the multi-Rx chain is enabled:

- if the RS(SSB or CSI-RS)  for RLM/BFD/CBD/L1-RSRP and another RS(SSB or CSI-RS)  for RLM/BFD/CBD/L1-RSRP has been paired in a grouped CSI reporting(e.g., grouped L1-RSRP or L1-SINR), no measurement restriction shall be applied.

- if the spatial separation is large enough between the RS (SSB or CSI-RS) for RLM/BFD/CBD/L1-RSRP and another RS (SSB or CSI-RS) for RLM/BFD/CBD/L1-RSRP, no measurement restriction shall be applied. Whether and how to specify it is FFS and needs more conclusions from RF session.

Proposal 2: For scheduling restriction for L1 measurement (RLM/BFD/CBD/L1-RSRP), if UE supports multi-Rx chain and the multi-Rx chain is enabled:

- if the RS(SSB or CSI-RS) for RLM/BFD/CBD/L1-RSRP and the CSI-RS for TCI of PDCCH or PDSCH has been paired in a grouped CSI reporting(e.g., grouped L1-RSRP or L1-SINR), no scheduling restriction shall be applied on this RS based RLM/BFD/CBD/L1-RSRP measurement.

- if the spatial separation is large enough between the RS (SSB or CSI-RS) for RLM/BFD/CBD/L1-RSRP and the CSI-RS for TCI of PDCCH or PDSCH, no scheduling restriction shall be applied on this RS based RLM/BFD/CBD/L1-RSRP measurement. Whether and how to indicate network the scheduling restriction information due to spatial separation is FFS, and needs more conclusions from RF session.

Proposal 3: For scheduling restriction for L3 measurement, if UE supports multi-Rx chain and the multi-Rx chain is enabled:

- if the spatial separation is large enough between the RS for L3 measurement and the CSI-RS for TCI of PDCCH or PDSCH, no scheduling restriction shall be applied on this L3 measurement occasions. UE may need to indicate scheduling restriction information to network. Details of indication and signaling can be FFS, and needs more conclusions from RF session.

Proposal 4: send LS to RAN1 and RAN2 to check the feasibility for the R18 grouped CSI reporting of mixed RSs(i.e., CSI-RS+SSB), i.e., whether reportQuantity in R18 can be enhanced to support grouped CSI reporting of CSI-RS+SSB based L1 measurement results.

# References

[1] R4-2303302, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, RAN4 #106.