3GPP TSG-RAN WG4 Meeting # 106bis-e				                     R4-2304789

Online, April 17 – April 26, 2023

Agenda Item:	5.8.3.2

Source: 	Xiaomi

Title: 	on the L1-RSRP measurement delay

Document for:	Approval

- Introduction

During the RAN4#106 meeting, the multi-RX chain topics has been and the groupbasedbeamreporting-r17 has been identified as the baseline mechanism for simultaneous reception. In this case, further L1-RSRP measurement delay requirement can be based on such IE and hence further discussion is provided in the following discussion paper.

- Discussion

The general aspect has been discussed and couple of agreements are quite important to define the L1-RSRP measurement delay requirement and hence the agreement in WF [1] are captured as below:

#### Issue 1-1-3: M-TRP scenarios for Rel-18 multi-Rx DL reception

- Focus on intra-cell multi-TRP operation scenario

- No requirements will be introduced for inter-cell multi-TRP operation in R18.

#### Issue 1-2-7: Necessity of group-based beam reporting for simultaneous reception

- Rel-17 group-based reporting is used as a prerequisite to define requirement for simultaneous reception

  - Note: Simultaneous reception term above includes Data/Data, RS/RS and Data/RS simultaneous reception cases.

#### Baseline L1 measurement requirements for potential multi-Rx enhancement

- For cell specific RLM: section 8.1

- For cell specific link recovery: section 8.5

- For TRP specific link recovery: section 8.18

- For L1-RSRP measurement: section 9.5.

< Way forward >

#### Issue 1-1-8: Scenario for RSs type combination for simultaneous L1 measurements/data reception

- Option 1:

  - Combination of RS/RS and RS/data for simultaneous reception are moved to discussions related to measurement restriction/scheduling restriction requirements enhancement.

- Option 2:

  - Other options are not precluded, if necessary.

From the WF agreement, it has been agreed that the Rel-17 group-based reporting is used as a prerequisite to define the requirement and consequently, as the Rel-17 group-based reporting is only supported for intra-cell m-TRP scenario, so requirements will be defined only in such scenario.

Observation 1: Rel-17 group-based reporting is used as a prerequisite to define the requirement and hence only intra-cell m-TRP is supported.

In this case, we would like to look further into the Rel-17 group-based reporting mechanism. For the groupbasedbeamreporting-r17, the TS 38.214 has indicated the resource setting configuration for the such scenario which indicates that 2 CSI Resource Sets in the resource setting will be used.

5.2.1.4.1	Resource Setting configuration

For aperiodic CSI, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig not configured with groupBasedBeamReporting-r17 is linked to periodic, or semi-persistent, or aperiodic resource setting(s):

-	When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement for L1-RSRP or for channel and interference measurement for L1-SINR computation.

-	When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on NZP CSI-RS.

-	When three Resource Settings are configured, the first Resource Setting (higher layer parameter resourcesForChannelMeasurement) is for channel measurement, the second one (given by higher layer parameter csi-IM-ResourcesForInterference) is for CSI-IM based interference measurement and the third one (given by higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for NZP CSI-RS based interference measurement.

For aperiodic CSI, and for periodic and semi-persistent CSI resource settings, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig configured with groupBasedBeamReporting-r17 is linked to periodic or semi-persistent, setting(s):

-	When one Resource Setting is configured, the Resource setting is given by resourcesForChannelMeasurement for L1-RSRP measurement. In such a case, the number of configured CSI Resource Sets in the Resource Setting is S=2

For aperiodic CSI, and for aperiodic CSI resource settings, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig configured with groupBasedBeamReporting-r17 is associated with resourcesForChannel and resourcesForChannel2, which correspond to first and second resource sets, respectively, for L1-RSRP measurement.

Furthermore, in TS 38.331, for the IE groupbasedbeamreporting-v1710, the CSI-SSB-ResourceSetList and nzp-CSI-RS-ResourceSetList will be configured in the CSI-ResourceConfig field. This is shown as below in detail:

From the above specification analysis, we see that for the groupbasedbeamreporting-r17, there are two CSI resource sets can be configured and for each CSI resource set, it can be CSI-SSB resource set or nzp-CSI-RS resource set.

Observation 2: For the groupbasedbeamreporting-r17, there are two CSI resource sets can be configured and for each CSI resource set, it can be CSI-SSB resource set or nzp-CSI-RS resource set.

Based on the groupbasedbeamreporting-r17 mechanism, the network will configure two CSI resource sets and one for each TRP. Also these two resource sets can be CSI-SSB or NZP-CSI-RS resource sets. The number of resources of each resources sets are configured and can also be different. UE should report the L1-RSRP based on the measurement results with at most 4 groups of data and the corresponding SSBRI or CRI will be also reported. An example of the L1-RSRP report can be seen as below:

Table 1 Illustration of the L1-RSRP report of groupbasedbeamreporting-r17

| CMR set ID with largest L1-RSRP | group ID | CMR resource ID in group#0 | CMR resource ID in group#1 |
| --- | --- | --- | --- |
| CMR set ID （1bit） | group ID#0 | SSBRI/CRI#， L1-RSRP（7bit） | SSBRI/CRI#， L1-RSRP（4bit） |
|  | group ID#1 | SSBRI/CRI#， L1-RSRP（4bit） | SSBRI/CRI#， L1-RSRP（4bit） |
|  | group ID#2 | SSBRI/CRI#， L1-RSRP（4bit） | SSBRI/CRI#， L1-RSRP（4bit） |
|  | group ID#3 | SSBRI/CRI#， L1-RSRP（4bit） | SSBRI/CRI#， L1-RSRP（4bit） |

This also indicates that for each L1-RSRP measurement of the groupbasedbeamreporting-r17, the UE is required to measure the SSB or CSI-RS based reference signal simultaneously and corresponding L1-RSRP measurement result for the simultaneous measurement will be reported. In this case, the requirement for each measurement should be defined with three different cases as:

1, both SSB-based L1-RSRP measurement delay

2, both CSI-RS based L1-RSRP measurement delay

3, SSB and CSI-RS based L1-RSRP measurement delay

Proposal 1: It is proposed to define the L1-RSRP measurement delay with three different cases as:

1, both SSB-based L1-RSRP measurement delay

2, both CSI-RS based L1-RSRP measurement delay

3, SSB and CSI-RS based L1-RSRP measurement delay

Currently the L1-RSRP measurement is using different measurement delay for SSB based L1-RSP and CSI-RS based L1-RSRP. From resource set configuration perspective, the L1-RSRP measurement for groupbasedbeamreporting-r17 can be both SSB based, CSI-RS based and one for SSB based and one for CSI-RS based. Currently for SSB based L1-RSRP measurement delay, N=8 for beam sweeping factor has been used. For CSI-RS based L1-RSRP measurement delay, N is related to the repetition on/off of such configured CSI-RS resource set and the number for resources per resource set. Hence the requirements might need to be considered together.

Observation 3: The SSB-based and CSI-RS based L1-RSRP measurement delay are related to different beam sweeping factors.

For the case of both SSB-based L1-RSRP measurement delay, In our previous paper, we have analysed that for the UE equipped with two panels, due to different UE implementation, there will be three scenarios as fully overlap, non-overlap as well as the partial overlap of the two panel RX beam coverage. Some companies has provided their views as no delay enhancement can be reached as option 6 shown above based on the non-overlap assumption for the UE two antenna panels. From our understanding, the non-overlap scenario needs the UE to perform 8 times L1-RSRSP measurement for 16 different beam directions with 2 directions at each time as the multi-RX capability. In this case, the measurement still needs 8 times of beam measurement and hence no L1 enhancement can be reached. However, for the extreme cases such as the fully overlap of two panels, there will be only 8 beams needs to be tested and 2 directions can be tested each time, hence the half of beam measurement as 4 will be needed. From this perspective, the beam sweeping factor depends largely on the coverage overlap of different panels.

Observation 4: For non-overlap of two panel assumption, there will be no beam sweeping factor reduction for SSB based L1-RSRP.

Observation 5: For fully overlap of two panel assumption, the beam sweeping factor can be enhanced from 8 to 4 for SSB based L1-RSRP.

Similarly, with the partially overlap scenario, if m of the beams are overlapped for the two panels, then the total directions needs to be tested is N= floor(m/2) + 8 – m. For example. M=0, N=8; M=8, N=4 which is matched to the above observation 1 and 2 for non-overlap and fully overlap cases. For the below example, the first panel is for beam #0 to #7 with red and the second panel is for beam #2 to #9 with yellow. In this case the overlapped beams as #2 to #7 with orange and there are 6 beams overlap as total. In this case, N=5 as the beam factor after the overlap scaling reduction.

Proposal 2: The beam sweeping factor can be reduced as N= floor(m/2) + 8 – m where m is the overlapped of the two panels.

From our understanding, this UE panel coverage information can be reported as a UE capability which represents the UE panel coverage overlap. The exact report number can be directly about the scaling factor and hence network can use this capability to evaluate the L1-RSRP measurement time.

Proposal 3: The scaling factor can be a UE capability.

For the case of both CSI-RS based L1-RSRP measurement, currently the beam sweeping factor is N depends on the CSI-RS resources configuration and can be concluded as below table 1:

Table 1 Beam sweeping factor N for different cases for CSI-RS:

|  | Periodic CSI-RS | Semi-persistent CSI-RS | Aperiodic CSI-RS |
| --- | --- | --- | --- |
| Repetition = on | N=ceil(maxNumberRxBeam / Nres_per_set) | N=ceil(maxNumberRxBeam / Nres_per_set) | N=ceil(maxNumberRxBeam / Nres_per_set) |
| Repetition = off | 1 | 1 | 1 |

From table 1 we can see for repetition on or off of the configured CSI-RS resources set the beam sweeping factor will be different from one or ceil(maxNumberRxBeam / Nres_per_set). The maxNumberRxBeam is a UE per band capability that the beam sweeping capability per CSI-RS resource set and Nres_per_set is number of resources in the resource set.

Observation 6: Beam sweeping factor N for CSI-RS is based on the repetition configuration, maxNumberRxBeam UE capability and Nres_per_set configuration.

With table 1, we can see if one of the CSI-RS resource set is set with repetition off, then there will be no beam sweeping factor enhancement. Furthermore, as the beam sweeping factor depends on the CSI-RS resource setting a lot, it is hard for a UE to reach beam sweeping factor enhancement in such scenario. In this case, we would like to propose that no UE L1-RSRP measurement delay enhancement for two CSI-RS based L1-RSRP case.

Proposal 4: No UE L1-RSRP measurement delay enhancement for two CSI-RS based L1-RSRP case.

Lastly, for the case of one CSI-RS resource and one SSB based L1-RSRP measurement, we believe such scenario, each SSB or CSI-RS based L1-RSRSP measurement delay requirement apply.

Proposal 5: Each SSB or CSI-RS based L1-RSRSP measurement delay requirement apply when the CSI resource sets are configured as one for SSB and one for CSI-RS.

3	Conclusions

In this paper, we give further discussion on L1-RSRP measurement issue, the observations and proposals are captured as below:

Observation 1: Rel-17 group-based reporting is used as a prerequisite to define the requirement and hence only intra-cell m-TRP is supported.

Observation 2: For the groupbasedbeamreporting-r17, there are two CSI resource sets can be configured and for each CSI resource set, it can be CSI-SSB resource set or nzp-CSI-RS resource set.

Observation 3: The SSB-based and CSI-RS based L1-RSRP measurement delay are related to different beam sweeping factors.

Observation 4: For non-overlap of two panel assumption, there will be no beam sweeping factor reduction for SSB based L1-RSRP.

Observation 5: For fully overlap of two panel assumption, the beam sweeping factor can be enhanced from 8 to 4 for SSB based L1-RSRP.

Observation 6: Beam sweeping factor N for CSI-RS is based on the repetition configuration, maxNumberRxBeam UE capability and Nres_per_set configuration.

Proposal 1: It is proposed to define the L1-RSRP measurement delay with three different cases as:

1, both SSB-based L1-RSRP measurement delay

2, both CSI-RS based L1-RSRP measurement delay

3, SSB and CSI-RS based L1-RSRP measurement delay

Proposal 2: The beam sweeping factor can be reduced as N= floor(m/2) + 8 – m where m is the overlapped of the two panels.

Proposal 3: The scaling factor can be a UE capability.

Proposal 4: No UE L1-RSRP measurement delay enhancement for two CSI-RS based L1-RSRP case.

Proposal 5: Each SSB or CSI-RS based L1-RSRSP measurement delay requirement apply when the CSI resource sets are configured as one for SSB and one for CSI-RS.

4 References

[1]R4-2303302		WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), VIVO