3GPP TSG-RAN WG4 Meeting # 106bis-e				                     R4-2304792

Online, April 17 – April 26, 2023

Agenda Item:	5.8.3.5

Source: 	Xiaomi

Title: 	on the TCI state switching delay with dual TCI

Document for:	Approval

- Introduction

During the RAN4#106 meeting the TCI state switching delay requirement has been discussed and the agreements are captured in the WF [1]. The scenarios have been fully discussed while the requirements have not been decided yet. In this paper, we give further discussion on the requirements.

- Discussion

The agreement of the WF [1] is captured as below:

- Agreements in online session

  - Group Based Reporting and s/mDCI

    - Group based reporting applies to both sDCI and mDCI

  - Investigate and if needed specify requirements for the following TCI state switching scenarios

    - single TCI to dual TCI

    - dual TCI to dual TCI

    - dual TCI to single TCI

  - TCI Switch triggering method:

    - MAC-CE based dual TCI state switch for PDCCH reception

    - DCI based dual TCI state switch for PDCSH reception

    - FFS RRC triggered TCI state configuration.

For the triggering method, the MAC-CE based TCI state switch for PDCCH reception and DCI based dual TCI state switch for PDSCH reception have been confirmed. For RRC triggered TCI state configuration, currently the RRC triggered TCI state is used for only one TCI state is configured in the RRC and the only applicable scenario will be the switch from dual TCI to single TCI (RRC triggered). This has been argued as corner case, while from our understanding, the requirement is quite straight forward that the legacy RRC based TCI state switching requirement can apply.

Observation 1: The RRC triggered TCI state switching scenario can be the switch from dual TCI to single TCI (RRC triggered).

Observation 2: The legacy RRC based TCI state switching requirement can apply for above case.

For the MAC-CE based TCI state switching delay requirement, we have discussed it fully with also the different TCI state configuration of original TCI state and target TCI state and the requirement is shown in table 1 is captured as below.

Table 1: MAC CE based TCI state switching delay requirement

| Original TCI state | Target TCI state | Requirement |
| --- | --- | --- |
| Single TCI or Dual TCI for PDCCH SFN | Dual TCI for PDCCH SFN both known | Legacy single TCI known requirement apply |
|  | Dual TCI for PDCCH SFN one or both un-known | Legacy single TCI un-known requirement apply |
| Single TCI or Dual TCI for PDCCH non-SFN | Dual TCI for PDCCH non-SFN | Legacy single TCI requirement apply for each RX chain |
| Dual TCI for PDCCH SFN or Dual TCI for PDCCH non-SFN | Single TCI | Legacy single TCI requirement apply |

This table 1 has clearly included all the single TCI, PDCCH SFN and PDCCH non-SFN of MAC-CE based TCI state switching and corresponding requirements are defined. We can see the legacy MAC-CE based TCI state switching delay requirements apply for each case.

Proposal 1: To agree table 1 for the MAC CE based TCI state switching delay requirement.

For the DCI based dual TCI state switch for PDCSH reception, for s-DCI, only one PDCCH will be received by the UE and correspondingly two TCI states will be received. Currently the DCI based TCI state switching is using the timeDurationForQCL to define such requirement. The timeDurationForQCL is a per feature set UE capability and hence for m-TRP scenario, it is believed that this value is not TRP specific.

Observation 3: The timeDurationForQCL is a per feature set UE capability and this value is not TRP specific.

From this perspective, as currently for s-DCI scenario, the same timeDurationForQCL should apply when the PDCCH is received by UE and hence the legacy DCI based TCI state switching requirement apply.

Proposal 2: For DCI-based TCI state switching delay, the legacy DCI based requirement apply for s-DCI.

For m-DCI, each TRP transmits its own PDCCH and PDSCH. In this case, the legacy requirement apply for each TCI state switching for each TRP.

Proposal 3: For DCI-based TCI state switching delay, the legacy requirement apply for each TCI state switching for each TRP for m-DCI.

3	Conclusions

In this paper, we give further discussion on TCI state switching delay, the observations and proposals are captured as below:

Observation 1: The RRC triggered TCI state switching scenario can be the switch from dual TCI to single TCI (RRC triggered).

Observation 2: The legacy RRC based TCI state switching requirement can apply for above case.

Proposal 1: To agree table 1 for the MAC CE based TCI state switching delay requirement.

Proposal 2: For DCI-based TCI state switching delay, the legacy DCI based requirement apply for s-DCI.

Proposal 3: For DCI-based TCI state switching delay, the legacy requirement apply for each TCI state switching for each TRP for m-DCI.

4 References

[1] R4-2303317		WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2), Qualcomm